ILISP Revision 2 documentation

APPS for CAD/CAM/NESTING on waterjet cutting machines

Version 2025.3.4357

Last updated October 14, 2025

By Bo Johansson

© IGEMS AB
Borås
Sweden

How to modify the postprocessor	3

Postprocessor functions	5

Postprocessor support functions	34

Apps	40

ILISP Functions	43

Geometrical example	71

Toolbar	73

Camera	74

Variables	75

Dynamic Dialogs	76

ILISP command to control the CAD system	94

Sheet value calculation script	101

Connection IGEMS/Organizer with external ERP	102

ILISP functions for the CAM module	110

ILISP functions for the Automatic Nesting module	116

Lead script	118

Old reports system (before 2018.4)	119

New report system (from 2018.4)	126

Application reports	136

Other postprocessor functions	137

Interfacing ILISP with a .NET language	138

OEM-Labeling of IGEMS	139

The structure of the postprocessing (for internal use)	140

Important variables for Z-axis and sensoring	151

CNC files with subroutines	153

Simulation functions	155

Variables	159

How to modify the postprocessor

The postprocessor file is a standard text file with extension IPF (Igems Postprocessor File) It can be edited in a optional text editor, but we recommend using the built-in LISP editor. The editor is started by pressing CTR + F5 when this window is open.

Edit actual postprocessor

This will open the LISP editor.

The postprocessor is developed in our internal ILISP language. All function in ILISP is described in this document.

By placing the cursor on a LISP function and then press F1 you will come to the actual page in this ILISP help file, in this case to the STRCAT function.

How can I make a new postprocessor?

This can be done in the postprocessor tab in the machine settings.

Postprocessor functions

List of postprocessor functions

The job is created by many sequential events. The every moves and handle all the equipment on the machine. For each event there is a function. In this function you can customize exactly the machine instructions for every step.

| arc | Moves the jet with an arc. |
| --- | --- |
| chbargram | Handle pressure and abrasive amount. |
| chmode | Function for change mode, for example between marking, drilling, cutting |
| chtool | Used to change distance between parts and handle tools to be used. |
| cutoff | Turns of water and abrasive at the end of cutting. |
| drilling | Generate information for drilling. |
| final | User function called after footer. |
| flyctrl | Turns of water and abrasive during cutting. Difficult for many CNC. May be deprecated |
| fmode | Change speed mode: speed interpolation, mm /min or time mode. |
| footer | Handle the end of the CNC file. |
| header | Generate information at the beginning of the CNC file. |
| itext | Function for inkjet printing on part. |
| line | Moves the jet linear to a position. |
| line3d | Move down to correct Z after piercing if increased Piercing Height is used. |
| markoff | Turn off special marking equipment. |
| markon | Turn on special marking equipment. |
| measure | Function that are used for measuring in advance. |
| measurepoint | Function that are used to localize the 5-X part (space X) before cutting |
| mrapid | Rapids at the end of the cutting, normally for handling parking position. |
| part | Handle information about the part that should be cut. |
| pathinfo | This event gives information about next coming cutting path. |
| piercing | Start the jet with or without abrasive. |
| pumponoff | Turn on and off the pump. |
| quality | This function is called when the geometry change quality. |
| rapid | Instructions for rapid transport between different cuts. |
| refpos | Function that are used to localize the surface in 5-x cutting before cutting |
| rewind | Handle rewind because of rotation limitations in the 5-axis kinematic. |
| rotax | Rotate A, B or C during cutting without any movement in X, Y or Z. |
| srapid | Srapids used for localization of the part before cutting. |
| stop | Generate a stop. |
| text | Handle text |
| tilt | Tilt the 5-axis kinematic and rotating axis rapidly to next cutting position. |
| tooldown | Move the tool down vertically to cutting position. |
| tooldown3d | Moves the tool to a cutting position. |
| toolup | Move the tool up vertically to rapid height after cutting. |
| toolup3d | Moves the tool to a position after cutting. |

HEADER

The header event is always called at the beginning of the postprocessing. The function can be used to view information, initialize values, and set default values for G and M-codes.

Some global variables are already set before the header function are called. (Before the postprocessor is loaded) You can read about those variables in the OPTIONS function. The HEADER function can also use following variables.

| $AngleIpo | Bool | T if Angle Interpolation is activated in the machine settings. Note! This feature can only be used by IGEMS CNC |
| --- | --- | --- |
| $6x-direction | Int | An integer that means following directions of the rotation unit. 0=1,0,0 (East) 1=0,1,0 (North) 2=-1,0,0 (West) 3=0,-1,0 (South) 4=1,0,1 (East 45 Up) 5=0,1,1 (North 45 Up) 6=0,0,1 (Up) |
| $6x-origo | List | This is the distance to the part zero in 3D-5X on 6x-toolpaths |
| $6x-reverce | Bool | T if the value of $C is reversed else nil. |
| $abrasive | String | This variable has the name of the Abrasive Quality. If it’s not a waterjet machine then this variable will be empty "". |
| $abrasiveflow | Int | Amount of abrasive in gram (note in the strategy setting) |
| $abs | Bool | T if the coordinate are in Absolute mode (G90) and nil if the coordinate are in incremental mode (G91) |
| $alloy | String | This is the Material Quality. |
| $bar | Int | The first used pressure (see also $psi) |
| $box | List | This is a list of 4 reals that describe a rectangle area that can include all parts. (X-lower left, Y-lower left, X-upper right, Y-upper right) |
| $build | Int | Build version number. This variable was introduced in version 2016.3.1060. For example: Version 2016.3.1060 will return 1060. |
| $cando5x | Bool | T if the machine has possibilities to cut 5X cutting else nil. See also the variable $uses5x. |
| $cando6x | Bool | T if the machine has possibilities to make 6X cutting else nil. See also the variable $uses6x. |
| $cncPressure | Int | Means following: 0=Pressure cannot be set from the CNC. 1=Pressure is handled by the CNC using M-codes for Low and High Pressure. 2=Pressure is handled by CNC by programming the Pressure. |
| $cpfeed | Real | Velocity for circular piercing in mm/min. |
| $cpradius | Real | Radius for circular piercing. The value describes the movement of the tool. |
| $cptime | Real | Time in seconds for circular piercing. |
| $cpturns | Real | Number of turns for circular piercing. |
| $contains | Int | A bit coded value that can be used to check in advance what different kinds of operations are included in the whole job. Summary of following 0= 1=Cutting (Not released) 2=Marking 4=Low pressure piercing 8=Drilling 16= Circular piercing 32=Hole command cutting option 64=Some toolpath will include speed ramping 128=Some toolpath will not include speed ramping 256=If Special marking is used. 512=If Inkjet is used Up to 15 bits value will be available. This variable is available also in the PART command but there the value is for the actual part. |
| $cutbar | Int | Pressure for cutting in AWJ cutting and Pure waterjet cutting. |
| $cutparname | String | This is the name of the actual cutting parameter setting. |
| $cutflow | Int | Amount of abrasive for cutting in gram/min. |
| $cuttingupz | real | The value for the lift height between cuttings (checking what's possible). |
| $date | String | The date in the format at used in current country. |
| $density | int | The density of the material in kilo for 1 mᶟ |
| $digfile | String | The name if the actual drawing. If the file is saved then drive and path are included in the name. |
| $drillbytype | Int | 0=Sheetwice drilling. 1=Toolwice drilling (This option is not longer available) 2=Partwice drilling. 3=No optimizing, the drilling will be made at the Cuton function (This option is not longer available) |
| $drillfeed | Real | Feed for drilling. |
| $drillrpm | Int | Rpm for drilling tool. |
| $drillstep | Real | Drilling step. |
| $drillupz | real | The value for the lift height between drilling positions (checking what's possible). |
| $f | Real | Fixed speed for AWJ and pure waterjet |
| $fine | Real | Cutting speed for fine quality (see also $xrough, $rough, $medium and $xfine). |
| $flyonoff | Bool | T if Flying on/off is activated else nil |
| $fnumber | Real | When using numbers as name of the NC-file then this variable contains that number. If you use the drawing name as default filename or typing a filename that can’t be converted to number then this value will be 1000. (Before R2022.2.2733 this variable was an integer) |
| $group | String | This is the material group. |
| $headinfinity | Bool | The variable is T if the major axis on the kinematic can rotate infinity. |
| $highbar | Int | The highest bar of $markbar, $piercbar and $cutbar in AWJ-cutting (for machines that only support two different pressures) (*) |
| $highflow | Int | The highest amount of abrasive used on the job. (For machines that only support two different levels of abrasive). |
| $inittools | Int | The number on the first tool that will be used. |
| $inittooldist | Real | The distance between the first tool that will be used. |
| $kerfcnc | Bool | T if the kerf is controlled in the CNC. (normally G41/G42) |
| $kerfsize | Real | This is the tool radius if the kerf should be handled completely in the controller. |
| $kinematic | Int | This variable shows how the tiltangles are calculated on 5-axis machines. 0=No 5-axis cutting head is configured. 1=Control by motor axis 2=Major axis in direction of cutting, second axis is the cutting angle. 3=Major axis in direction of jet, second axis is the cutting angle. |
| $lag | Bool | T if Lag angle compensation is activated. |
| $lowatend | Bool | This variable is T if the first cut should start with Low pressure. It's normally used to set the pressure to low at the end of the program. Good when running the same program over and over again. |
| $lowbar | Int | The lowest bar of $markbar, $piercbar and $cutbar in AWJ-cutting (for machine that only support two different pressures) (*) |
| $lowflow | Int | The lowest amount of abrasive used on this job. (For machines that only support two different levels of abrasive). |
| $machine | String | This is the name of the Machine Setup |
| $machinetype | Int | 0=AWJ, 1=Pure water, 2=Plasma, 3=Oxyfuel, 5=Laser. |
| $markbar | Int | Pressure for markings in AWJ-cutting(*) |
| $markbytype | Int | 0=Sheetwise marking. 1=Toolwise marking. 2=Partwise marking. 3=No optimizing, the marking will be done in the order of the programmed toolpath. |
| $markflow | Int | Amount of abrasive for marking in gram/min. |
| $markupz | real | The value for the lift height between markings (checking what's possible). |
| $maxbar | Int | Maximum possible pressure. |
| $maxslope | Real | Maximum cutting angle that will be used on entire job. Note! This variable has another meaning in the Piercing function. The value is not negative and in degree. |
| $maxspeed | Real | Maximum speed used in the program |
| $maxtilt | Real | Maximum tilting angle for the 5-axis cutting head. If No 5-axis head is configured then this value is 0. The value is in degree. |
| $maxtools | Int | Number of cutting tools |
| $medium | Real | Cutting speed for straight cut in medium quality (see also $xrough, $rough, $fine and $xfine). |
| $maxpartid | int | Number of unique parts on the cutorder. |
| $maxpathid | int | Number of unique toolpath on the cutorder. |
| $maxsortid | int | Number of parts on the cutorder. |
| $metric | Bool | T on Metrical units and nil if Imperial mode (inches) |
| $minbar | Int | Minimum possible pressure. |
| $minspeed | Real | Minimum speed used by the program |
| $name | String | The name of the CNC-file. |
| $nlines | Bool | The variable I T if line number should be used else nil. |
| $orifice | Real | Diameter of the Orifice |
| $partupz | real | The value for the lift heigt between parts (checking what's possible). |
| $piercbar | Int | Pressure for piercings in AWJ cutting(*) |
| $piercebytype | Int | 0=Sheetwise pre-piercing. 1=Toolwise pre-piercing. 2=Partwise pre-piercing. 3=No optimizing, the pre-piercing will be done together with the cutting of the geometry by the Cuton function. |
| $piercflow | Int | Amount of abrasive for piercing in gram/min. |
| $piercupz | real | The value for the lift height between piercings (checking what's possible). |
| $psi | Int | The first used pressure (see also $bar). |
| $pstat | Real | Piercing time for stationary piercing in seconds |
| $pumpstart | Bool | T if the pump should be started automatically else nil |
| $pumpstop | Bool | T if the pump should be stopped automatically else nil |
| $rough | Real | Cutting speed for straight cut in rough quality (see also $xrough, $medium, $fine and $xfine). |
| $sensordist | Real | The distance between each measurement. If no stamping sensor is activated, then this value is 0. |
| $sensorminlift | Real | The minimum lift height from material when using hight sensor. |
| $sensormode | Int | If the value is: Nil = No Sensor should be used at all. 0 = Sheetwise measure. The sensor is measure the height once in the beginning of the CNC-file. 1 = Partwice measure. The height sensor is measure the height once at every part. 2= Dragging sensor. The measurement is done all time. 3= Stamping sensor. The measurements are by a curtain distance. |
| $sensorsec | Real | The time in second between each measurement. The time is based on straight cut in medium quality. If no stamping sensor is activated then this value is set to 0. |
| $sheetname | String | The name of the sheet that are used. If no sheet has been used the variable is an empty string. |
| $sheetx | Real | Size of the sheet in X direction (CAD coordinates). If no sheets are in use, then the value is 0. |
| $sheety | Real | Size of the sheet in Y direction (CAD coordinates). If no sheets are in use, then the value is 0. |
| $singularity | Bool | If it’s a singularity kinematic when jet is straight |
| $speedIpo | Bool | T if $speedmode is 3 else nil. |
| $speedmarking | Real | Speed that will be used for marking |
| $speedmode | Int | 1=Fixed speed, 2=Interpolation by steps, 3=Interpolation in CNC |
| $speedrapid | Real | Speed that will be used for Rapid movements in X and Y |
| $standoff | Real | (This variable is no longer available) |
| $startmax | Int | New in IGEMS2.LSP: Number of jet on in the toolpath. Pre-piercing, Piercing and Markings. See also $startindex at the PIERCING function |
| $tac | Bool | T if Tac option is activated else nil. |
| $thickness | Real | The thickness of the material. |
| $time | String | The actual time |
| $toolchanges | Int | Number of tool setups. Minimum is 1. |
| $tube | Real | Diameter of mixing tube |
| $type | Int | Machine type: 0=AWJ, 1=Pure water, 2=Plasma, 3= Oxyfulel, 4=Laser |
| $useflyctrl | Bool | This variable is T if the Flying jet ON/OFF control checkbox is activated in the Delay tab in the machine setting. |
| $uses5x | Bool | T if 5X cutting will be used in the actual process. From version R2016.3.1036 this variable is T also when TAC is used. |
| $uses6x | Bool | T if 6X cutting will be used in the actual process else nil. |
| $usesdrill | Bool | T if a drill will be used in the actual process. |
| $voc | Bool | T if Variable tool radius is activated else nil |
| $xfine | Real | Cutting speed for straight cut in X-fine quality (see also $xrough, $rough, $medium and $fine). |
| $xrough | Real | Cutting speed for straight cut in X-rough quality (see also, $rough, $medium, $fine and $xfine). |
| $z | Real | This value is initialized to the lift height that is used between parts. |
| $zaxisoption | Int | 0=Fixed Z-axis 1=CNC controlled = No sensoring 2=Height sensoring activated. Zero is always on top. 3=Sensoring in advance. Zero is always at table. |
| $zbase | Int | 0=The Z-zero should be at the table of the machine. 1=Zero should be at the material thickness or the surface of the material. 2=Zero should be at the very top position. |
| $zmaxmove | Real | The maximum accepted Z-movement from machine table to upper position. |
| $zmaxup | Real | The larges Z-axis value that can be used. This is different depending on the $Zbase value. Example: $zbase=1 and $zmaxmove is 300. If you have a 12 mm material and an underlay of 5 then $zmaxup will be 200-thickness – underlay =283. |
| $zmaxzaxis | int | Number of movable Z-axis |
| $zmode | Int | The $zmode have following meaning. 0=Fixed Z-axis. 1=Automatic, Up/Down controlled by water On/Off. 2=Up/Down with separate M-codes. 3=Up/Down controlled by Z-coordinates. 4=Automatic Height sensor that are controlled by water On/Off. 5=Height sensor that are controlled by M-codes. 6=Height sensor that are controlled by Z-coordinates. 7=By measuring probe information. |
| $zmovedown | real | This value can be used to move down the zero point from the Machine coordinate system if it’s located on the highest position to the surface of the material. This value is useful when the $zbase is 1 but the value will be correct for $zbase 0 and 2 as well. |

(*) In pure waterjet cutting this variable is set to the same value as $cutbar

Example 1:

(DEFUN header()
  (WRITE "%")
  (WRITE (STRCAT (NTXT) "G40"))
)

Example 2:

(DEFUN header()
  (WRITE (STRCAT (NTXT) "(FILE: " $name ")"))
  (WRITE (STRCAT (NTXT) "(MATR: " $Group "/" $alloy "/" (RTS $thickness) ")"))
  (WRITE (STRCAT (NTXT) "(DRAWING: " $digfile ")"))
  (WRITE (STRCAT (NTXT) "(DATE: " $date ")"))
  (WRITE (STRCAT (NTXT) "(MACHINE: " $machine ")"))
  (WRITE (STRCAT (NTXT) (IF $metric "G71" "G70")))
  (WRITE (STRCAT (NTXT) (IF $abs "G90" "G91")))
  (WRITE (STRCAT (NTXT) "G40 G00"))
)

This could generate a CNC-file like follows:

(FILE: P277)

(MATR: INOX/Standard/20)

(DRAWING: C:/R10-IDE/SAMPLE.dig)

(DATE: 2011-01-18)

(MACHINE: IGEMS)

G71

G90

G40 G00

List of postprocessor functions

PART

As soon as the machining continues on another part then this function is called. Some machine need this information, others do not. Following variables can be used:

| $3d5xpart | Bool | It T if the part is a 3D-5X part else nil. |
| --- | --- | --- |
| $contains | Int | This variable is the same as you have in the HEADER. But this is local only in the PART function. |
| $chpart | Bool | This variable is T if it's a new part else nil. |
| $sortid | Int | This variable is an index starts on zero and increasing for all parts on the drawing. The order of the index is in that order they are selected in the Order command. All parts on the sheet layout a unique $sortid. |
| $partid | Int | This variable is an index that starts on zero and increasing for each unique part definition on the sheet layout. If you have one part and have copy that to 100 copies, they will all have the same $partid. |
| $pathid | Int | This variable is an index that starts on zero and increase by one for each unique toolpath on a part. Note! Two identical path where one is rotated will have the same $pathid. |
| $partcolor | Int | This is a RGB int describing the color of the part. The information is usable in IGEMS CNC since the part can have the same color. |
| $partname | String | The name of the part as defined in Create part |
| $customer | String | The name of the customer as defined in Create part |
| $partdate | String | The date as defined in Create part |
| $quantity | Int | Number if parts defined in Create part. |
| $connected | Bool | If the tool path is connected with the part |
| $sortid | Int | This variable is an index starts on zero and increasing for all parts on the drawing. The order of the index is in that order they are selected in the Order command. All parts on the sheet layout a unique $sortid. (Rev 2 only). |
| $box | list | A list of four reals that describe LoweLeft corner and Upper Right corner (X Y X Y) |
| $boxcenter | list | Center of the bounding $box |
| $boxrotated | list | A list of four reals that describe LoweLeft corner and Upper Right corner (X Y X Y) in actual rotation angle of the part. |
| $orderattrib | String | The attribute added on the part in the Order command. |

Example:

(DEFUN part()
  (WRITE (STRCAT (NTXT) "($name " " $customer " " $date ")"))
)

List of postprocessor functions

PUMPONOFF

This function is called if the postprocessor should write instructions for pump on and off.

The function is not called until the pump needs to be used. For example, if the job starts with inkjet marking and drilling then pumpon will not be called before the cutting starts.

Following variables can be used:

| $bar | Int | The pressure in bar that the pump should be initialized or finished in. |
| --- | --- | --- |
| $barunchanged | Bool | If the pressure should NOT be set or changed. |
| $delay | Real | This time is coming from the Delay tab in the Machine setting. It is probably not needed to put any delay after pump off instruction. |
| $metric | Bool | If CNC output should be using metric system (bar). |
| $off | Bool | If pump should be turned off the T else nil |
| $on | Bool | If pump should be turned on then T else nil |
| $pressurehandle | Int | 0 If the pump pressure cannot be controlled from instructions in the CNC file. 1 If the pump pressure is handled by two M-code (High and Low pressure) 2 If the pump pressure can be set a variable pressure from instructions in the CNC file. |
| $psi | Int | The pressure in psi that the pump should be initialized or finished in. |
| $usehighbar | Bool | If the pressure should be initialized or finished in high pressure then value is T else nil |

Example:

(DEFUN pumponoff ()

(COND

($barunchanged nil)

((= $pressurehandle 0) nil) ; Manual handling of pressure = do nothing

((= $pressurehandle 1) ; M-codes handle pressure

(IF $usehighbar

(WRITE (STRCAT (NTXT) "M77 ;HIGH PRESSURE"))

(WRITE (STRCAT (NTXT) "M76 ;LOW PRESSURE"))))

((= $pressurehandle 2) ; Variable pressure control

(IF $metric

(WRITE (STRCAT (NTXT) "M47 P" (ITOA $bar) " ;SET PRESSURE")))

(WRITE (STRCAT (NTXT) "M47 P" (ITOA $psi) " ;SET PRESSURE")))

)

(IF $on

(PROGN

(WRITE (STRCAT (NTXT) "G100 M50 ; PUMP ON"))

(DELAY $delay))

(WRITE (STRCAT (NTXT) "M51 ; PUMP OFF"))

)

)

List of postprocessor functions

CHBARGRAM

This function is called every time IGEMS needs to change pressure or amount of abrasive.

Following variables can be used:

| $abrasivehandle | Int | How the abrasive is handled: 0=Manual (a fixed value) 1=By M-codes Low and High 2=Variable controlled from CNC |
| --- | --- | --- |
| $bar | Int | The pressure in bar. |
| $barrel | Int | The difference between previous bar and actual bar |
| $barunchanged | Bool | The variable is T if the pressure is the same |
| $delay | Real | Delay time for the pump to reduce or increase the pressure. |
| $gram | Int | The amount of abrasive per minute. If no abrasive should be used than the value is 0. |
| $gramunchanged | Bool | The variable is T if the amount of abrasive is the same. |
| $init | Bool | If it’s the first initialization of the pressure or abrasive amount then the variable is T else it’s nil. |
| $lbs | Real | The amount of abrasive per minute. If no abrasive should be used then the value is 0. ($lbs = $gram /453.5934). |
| $pressurehandle | Int | How the pressure is controlled: 0=Manual 1=By two M-codes Low and High pressure 2=Variable pressure control. |
| $psi | Int | The pressure in PSI ($psi = $bar*14.50326). |
| $usehighbar | Bool | This variable is useful if the pump pressure is controlled by M-codes. The variable is T if the pump should change to high pressure and nil then the pump should be set to low pressure |
| $usehighflow | Bool | This variable is useful if the abrasive feeder is controlled by M-codes The variable is T if the feeder should use normal flow and nil if it should be changed to reduced flow. |

The example below is just handling pressure and abrasive amount by M-codes.

(DEFUN chbargram ()

(IF (NOT $barunchanged)

(IF $usehighbar

(PROGN

(WRITE (STRCAT (NTXT) "M77 ;HIGH PRESSURE "))

(DELAY $delay))

(PROGN

(WRITE (STRCAT (NTXT) "M76 ;LOW PRESSURE "))

(DELAY $delay)))

)

(IF (NOT $gramunchanged)

(IF $usehighflow

(WRITE (STRCAT (NTXT) "M46 ;NORMAL ABRASIVE FLOW"))

(WRITE (STRCAT (NTXT) "M47 ;REDUCED ABRASIVE FLOW")))

)

)

The example below describe all methods. Manual handling, handling by M-codes and variable settings of pressure and amount of abrasive.

(DEFUN chbargram ()

(COND

($barunchanged nil)

((= $pressurehandle 0) ; Manual handling

(IF $metric

(WRITE (STRCAT (NTXT) "M00 ;CHANGE PRESSURE TO " (ITOA $bar) " BAR"))

(WRITE (STRCAT (NTXT) "M00 ;CHANGE PRESSURE TO " (ITOA $psi) " PSI"))))

((= $pressurehandle 1) ; Handling by M-codes

(IF $usehighbar

(PROGN

(WRITE (STRCAT (NTXT) "M77 ;HIGH PRESSURE "))

(DELAY $delay))

(PROGN

(WRITE (STRCAT (NTXT) "M76 ;LOW PRESSURE "))

(DELAY $delay))))

((= $pressurehandle 2) ; Variable setting

(IF $metric

(PROGN

(WRITE (STRCAT (NTXT) "M47 P" (ITOA $bar) " ;SET PRESSURE"))

(WRITE (STRCAT (NTXT) "M46 Q" (ITOA (- $bar 200)) " S" (ITOA (+ $bar 200)))))

(PROGN

(WRITE (STRCAT (NTXT) "M47 P" (ITOA $psi) " ;SET PRESSURE"))

(WRITE (STRCAT (NTXT) "M46 Q" (ITOA (- $psi 3000)) " S" (ITOA (+ $psi 3000)))))))

)

(COND

($gramunchanged nil)

((= $abrasivehandle 0) ; Manual handling

(IF $metric

(WRITE (STRCAT (NTXT) "M00 ;CHANGE ABRASIVE TO " (ITOA $gram) " GRAM"))

(WRITE (STRCAT (NTXT) "M00 ;CHANGE ABRASIVE TO " (ITOA $lbs) " LBS"))))

((= $abrasivehandle 1) ; Handling by M-code

(IF $usehighflow

(WRITE (STRCAT (NTXT) "M46 ;NORMAL ABRASIVE FLOW"))

(WRITE (STRCAT (NTXT) "M47 ;REDUCED ABRASIVE FLOW"))))

((= $abrasivehandle 2) ; Variable setting

(IF $metric

(WRITE (STRCAT (NTXT) "M44 P" (ITOA $gram) " ;SET ABRASIVE FLOW"))

(WRITE (STRCAT (NTXT) "M44 P" (RTOS $lbs 2) " ;SET ABRASIVE FLOW"))))

)

)

List of postprocessor functions

CHTOOL

The CHTOOL function is used for changing active cutting tool or/and change the distance between the tools.

The function will never be called if the machine has only one tool.

Following variables that was existing in Revision 1 will be deprecated in Revison 2: $activectrl, $chactive, chdist, $distctrl, $offset, $reason and $skip. Those variables are no longer needed since Revision 2 use another syntax to do the same job. They will be deprecated in coming versions.

Following variable can be used in Revision 2:

| Variable | Type | Description |
| --- | --- | --- |
| $feed | real | Speed for the tool distance change on the cross beam in mm/min |
| $toolindex | Int | Increasing value starts on 1 and is increased by 1 for each tool change. |
| $maxdist | real | Maximum distance between the tools |
| $maxtools | int | Number of tools on this machine. |
| $mindist | real | Minimum distance between the tools |
| $msg | String | An automatic generated text with information of tool distances and active tools. |
| $msgactive | String | An automatic generated text message with information about active tools. |
| $msgdist | String | An automatic generated text message with information about distance between tools |
| $pack | Bol | If the outermost not used tools should be packed as closed as possible. This will increase the active cutting area on the machine. |
| $parking | Bol | If tools that are not in use should be parked in a non-moving position at the very end of the cross beam. nil=Do not park, T=Park. |
| $tooldist | real | Distance between each activated used cutting tool. |
| $tooldisthandle | Int | The variable is 0 if the distance is controlled manually. The variable is 1 if the distance can be changed from the CNC file. |
| $tooldistunchanged | Bool | The variable is T if no change of tool distance is required. |
| $toolhandle | Int | The variable is 0 if the on/off setting are handled manually. The variable is 1 if the on/off settings can be changed from the CNC file. |
| $toollist | List | A list of distances between each tool. |
| $tools | Int | A bit coded value that controls what tools should be active (ON). 1=T1 2=T2 4=T3 8=T4 16=T5 32=T6 64=T7 128=T8 The summary of the value controls active tools. Example: 11=(1+2+8)=T1+T2+T4 |
| $toolunchanged | Bool | The variable is T if no change of active tool is required. |
| $x | Real | First coordinate that will be used after the CHTOOL function. |
| $y | Real | First Y coordinate that will be used after the CHTOOL function. |

Example 1:

(DEFUN chtool (/ toolstr toolvar)

(COND

($tooldistunchanged nil) ; No change of tool distance needed

((= $tooldisthandle 0) ; Manual handling of tool distance

(WRITE (STRCAT (NTXT) "M00 (" $msgdist ")")))

((= $tooldisthandle 1) ; Automatic adjustment of tool distance

(SETQ toolstr "" toolvar 22) ; R22 just an example of cnc variable

(FOREACH s $toollist

(SETQ toolstr (STRCAT toolstr "R" (ITOA toolvar) "=" (RTS s) " "))

(SETQ toolvar (1+ toolvar))

)

(WRITE (STRCAT (NTXT) "M50 " toolstr " (USING " $msgdist ")")))

)

(COND

($toolunchanged nil); No change of active tool needed

((= $toolhandle 0) ; Tools are activated manually

(WRITE (STRCAT (NTXT) "M00 (" $msgactiv ")")))

((= $toolhandle 1) ; Tools are handled from the CNC code

(WRITE (STRCAT (NTXT) "T" (ITOA $tools) "(" $msgactive ")" )))

)

)

List of postprocessor functions

CHMODE

This event is called in the beginning of the CNC-file initialize a working mode. The event is called every time the machine change working mode and the event can be used to initialize different options in the machine.

Following variables can be used:

| $mold | Int | The variable handles the previous working mode nil=First time the event is called, this variable is always nil. 0=End of measure mode. 1=End of drilling mode. 2=End of marking mode. 3=End of pre-piercing mode. 4=End of cutting mode. 5=End of inkjet marking mode. 6=End of special marking mode. 7=End of probe sequence 9=End of drilling sensoring mode. |
| --- | --- | --- |
| $mode | Int | The variable means the following. 0=Start measure mode. 1=Start drilling mode. 2=Start marking mode. 3=Start pre-piercing mode. 4=Start of cutting mode. 5=Start of inkjet marking mode. 6=Start of special marking mode. 7=Start of probe sequence. 9=Start of drilling sensoring mode. |
| $modestr | str | This is a string describing actual mode "MEASURING" "DRILLING" "MARKING" "PRE-PIERCING" "CUTTING" "INKJET" "SPECIAL MARKING" "PROBE" "REFPOINTS" "SENSOR" |
| $rpm | Int | Rotation speed of drill if $mode = 1 |

Example:

(DEFUN chmode ()
  (COND
    ((NOT $mold) nil)
    ((= $mold 0) (WRITE (STRCAT (NTXT) "(END OF MEASURE SECTION)")))
    ((= $mold 1) (WRITE (STRCAT (NTXT) "(END OF DRILLING SECTION)")))
    ((= $mold 2) (WRITE (STRCAT (NTXT) "(END OF MARKING SECTION)")))
    ((= $mold 3) (WRITE (STRCAT (NTXT) "(END OF PRE-PIERCING SECTION)")))
    ((= $mold 4) (WRITE (STRCAT (NTXT) "(END OF CUTTING SECTION)")))
    ((= $mold 5) (WRITE (STRCAT (NTXT) "(END OF INKJET MARKING SECTION)")))
    ((= $mold 6) (WRITE (STRCAT (NTXT) "(END OF SPECIAL MARKING SECTION)")))
  )
  (COND
    ((= $mode 0) (WRITE (STRCAT (NTXT) "(START OF MEASURE SECTION)")))
    ((= $mode 1) (WRITE (STRCAT (NTXT) "(START OF DRILLING SECTION)")))
    ((= $mode 2) (WRITE (STRCAT (NTXT) "(START OF MARKING SECTION)")))
    ((= $mode 3) (WRITE (STRCAT (NTXT) "(START OF PRE-PIERCING SECTION)")))
    ((= $mode 4) (WRITE (STRCAT (NTXT) "(START OF CUTTING SECTION)")))
    ((= $mode 5) (WRITE (STRCAT (NTXT) "(START OF INKJET MARKING SECTION)")))
    ((= $mode 6) (WRITE (STRCAT (NTXT) "(START OF SPECIAL MARKING SECTION)")))
  )
)

The variables $mold and $mode remains unchanged. This means that the value can be used by other functions.

List of postprocessor functions

RAPID

The rapid function is always activated to move the jet between different positions in non-cutting mode.

To save production time you can do the rotation of the 5-X head and the rotation unit inside the RAPID function.

If you do that you must specify the variable *TiltOnRapid* to T else the simulation will not be correct.

(SETQ *TiltOnRapid* T)

| $ab | Bool | T if the movement have any A or B information else the variable is nil. |
| --- | --- | --- |
| $a | Real | The angle in radians of the major axis. If no 5-axis equipment are in use this variable is always 0 |
| $b | Real | The angle in radians of the minor axis. If no 5-axis equipment are in use this variable is always 0 |
| $c | Real | The angle in radians of the 6th axis. If no rotation unit are in use this variable is always 0.0. This value should only be used if the variable $uses6x is not nil. |
| $fr | Real | The velocity for the movement in mm/min |
| $reason | Int | -1=Rapid to a startpoint in X and Y as a very first movement on 5X cutting to avoid collision. This point should be a movement in X and Y only. 0=Normal rapid No Z-coordinate should be used. 1=Manual rapid inserted in a part (only between cuttings in 2D) 2=Manual rapid inserted in the order command between the parts (only between cuttings) 3=This is a movement to a sensor position. For example, to take the height before drilling or special marking. 4=Rapid inside a 3D-5X part (from R2016.3.1080) Note! It is important that the movements are interpolated. If G00 does not interpolate then use G01 at a high speed. 5=Rapid inside a 3D-5X part, this is the last movement of the rapid, before piercing. 6=Rapid to the Stop point generated in 3D-5X. Note! Only $reason 4, 5 or 6 should have Z-value. |
| $starttan | Real | Angle of the first cutting direction on the geometry. Note this value is in radians. This variable is also available in the PIERCING and MARKON functions. |
| $x | Real | The X-coordinate. |
| $y | Real | The Y-coordinate. |
| $z | Real | The z-value for the tool refer to table, thickness or highest position. This can be set in the machine setting. Note! Only $reason 4, 5 or 6 should have Z-value. |
| $uses6x | Bool | This variable is T if a rotation axis should be used. |

Example:

(SETQ *TiltOnRapid* T)

(DEFUN rapid (/ zstr bevstr rotstr)

(SETQ zstr (IF (MEMBER $reason (LIST 4 5 6)) (STRCAT " Z" (RTS $z)) ""))

(SETQ bevstr (IF $ab (STRCAT " A" (ATS $a) " B" (ATS $b)) ""))

(SETQ rotstr (IF $uses6x (STRCAT " C" (ANGTOS $c 3 nil)) ""))

(WRITE (STRCAT (NTXT) "G00 X" (RTS $x) " Y" (RTS $y) zstr bevstr rotstr))

(SETQ skiptilt T)

)

List of postprocessor functions

TILT

This function is called if you are using a 5-axis kinematic or a rotation unit. The function will rotate the 5-X kinematic or/and the rotation. It should always be done in rapid mode.

Note!

It is also possible to do this rotation in the RAPID function. In that case you don’t need any TILT function.

If you handle the TILT inside the RAPID function you must specify the variable *TiltOnRapid* to T

(SETQ *TiltOnRapid* T)

| $a | Real | The angle in radians of the major axis. If no 5-axis equipment are in use this variable is always 0. |
| --- | --- | --- |
| $ab | Bool | T if the movement have any A or B information else the variable is nil. |
| $b | Real | The angle in radians of the minor axis. If no 5-axis equipment are in use this variable is always 0 |
| $c | Real | The angle in radians of the 6th axis. If no rotation unit are in use this variable is always 0.0. This value should only be used if the variable $uses6x is not nil. |
| $reason | Int | The $reason means following: 1=Rotate axis straight if no tilt is needed (Added May 16, 2019 before the $reason was 4) 2=Rotate the axis for tilted piercing. 3=Rotate the A axis to correct angle before straight piercing. 4: Called from the footer |
| $starttan | Real | Angle of the first cutting direction on the geometry. Note this value is in radians. |
| $uses6x | Bool | This variable is T if a rotation axis should be used. |

(DEFUN tilt (/ bevstr rotstr)

(SETQ bevstr (IF $ab (STRCAT " A" (ATS $a) " B" (ATS $b)) ""))

(SETQ rotstr (IF $uses6x (STRCAT " C" (ANGTOS $c 3 nil)) ""))

(IF (NOT skiptilt) (WRITE (STRCAT (NTXT) "G0" bevstr rotstr)))

(SETQ skiptilt nil)

)

List of postprocessor functions

QUALITY

This function is always activated before the PIERCING function. It can also be used during the cutting. In most controllers we don’t need this event. The QUALITY function can use following variables:

| $initqua | Bool | If the quality command is called before the piercing (start quality). If this variable is T then $chqua is nil and opposite. |
| --- | --- | --- |
| $chqua | Bool | If the quality change is inside a geometry while the cutting process is active, then the variable is T else its nil. If this variable is T then $initua is nil and opposite. |
| $qua | Int | The quality of the cut. 1=X-Rough, 2=Rough, 3=Medium, 4=Fine and 5 is X-Fine |
| $f | Real | Cutting speed for straight cut in selected quality |

Example:

(DEFUN quality ()
  (IF $chqua
    (WRITE (STRCAT (NTXT) " (QUALITY CHANGED TO " (ITOA $qua) " DURING CUTTING)"))
    (WRITE (STRCAT (NTXT) " (QUALITY INITIALIZED TO " (ITOA $qua) " BEFORE CUTTING START"))
  )
)

List of postprocessor functions

TOOLDOWN

TOOLDOWN3D

TOOLDOWN is used if the jet is moved in a vertical direction (within 0.01 mm).

TOOLDOWN3D is used if the jet moves in a non-vertical direction.

In most cases the variable $sensor can handle all needed options.

| $fz | Real | The velocity for the rapid speed in mm/min |
| --- | --- | --- |
| $ipd | Real | This value is the value of Increased Piercing Distance. If this is not used, then the value is 0.0. (Cutting tab in the strategy). Note! The IPD is not used for Blind leads and Air starts. |
| $msg | String | An automatic generated text that describes the reason of Toolup |
| $reason | Int | The $reason means following: 1=Initialization in the beginning of the file. 2=Between drillings 3=Between markings 4=Between pre piercings 5=Between cuts in a part 6=Between two parts in cutting 7=Before cleaning or pressure reduce. 9=Down for sensoring. |
| $sensor | Int | 0 Use previous height. No need to measure the height again (save time). 1 or -1 Detect the height. 2 or -2 Detect the height and continue by stamping sensoring. 3 or -3 Detect the height and continue by dragging sensoring. The negative value is for IGEMS cnc. When it's negative then this is overriding the manual sensor mode that is set in the HMI (probably only for IGEMS cnc). |
| $sensordist | Real | The distance between each measurement when using Stamping sensor. The variable is used to calculate the time between each measurement and are only used in AWJ-cutting. |
| $sensormode | Int | This variable indicates the selected sensor option in the strategy. If no height sensor is used then this variable is set to nil. 0=Once. Normal implementation is that the sensor measures the height once at the start. In R1 this method was called Sheetwice. 1=Partwice. Normal implementation is that the sensor measures the height at every new part. 2=Every piercing 3=Stamping. The height sensor measures the height periodical. The time between the measurements is controlled by the variable $sensorsec. See also $sensordist. 4=Dragging. The height sensor should measure the height continuously all the time. This option should be used when cutting large parts with bevel. 5=Zone. This option will make a new sensoring if the distance from previous sensoring is longer than the Flat zone radius. 6=Auto. This option is automatically switching between Zone, Stamping and Dragging options. Depending on type, size of cut. Use the variable $sensor to write different options in the CNC file. |
| $sensorsec | Real | The time between each measurement when using Stamping sensor. The time is calculated from the variable $sensordist and the High cutting speed in Normal quality. This variable is only used in AWJ cutting. |
| $x | Real | The X position. |
| $y | Real | The Y position. |
| $z | Real | The Z position. |
| $zmode | Int | The $zmode has following meaning. 0=Fixed Z-axis. 1=Automatic, Up/Down controlled by water On/Off. 2=Up/Down with separate M-codes. 3=Up/Down controlled by Z-coordinates. 4=Automatic Height sensor that are controlled by water On/Off. 5=Height sensor that is controlled by M-codes. 6=Height sensor that is controlled by Z-coordinates. 7=By measuring probe information. If the machine has a pneumatic stroke, then this can be used together with all options above. |
| $zref | Bool/Int | This variable is an integer that indicate the closest reference point, measured by the measure function. If you haven’t used the measure function, this variable is nil. |

Example

(DEFUN tooldown ()
  (IF $sensor 
    (WRITE (STRCAT (NTXT) "M44 (SENSOR ON, TOOL DOWN)"))
    (WRITE (STRCAT (NTXT) "G0 Z" (RTS $z)))))
)

List of postprocessor functions

PATHINFO

This event gives information about next coming cutting path.

Following variables are available.

| $box | List | A list of four reals that describe Lowe Left corner and Upper Right corner (X Y X Y) |
| --- | --- | --- |
| $y | List | Center of the bounding $box |
| $connected | Int | T if the toolpath is connected with the part |

PIERCING

This event is used to turn ON the cutting or the marking process (see also MARKON).

To be able to use this event a function named PIERCING must be implemented in the post.

The variable $delay is deprecated. Use $JetValveOn, $JetAbrasiveOn and $JetPreload

Following variables are available:

| $attrib | String | This is the attribute given in the hole command or the attribute given in the marking command. New in R2017.3.388 |
| --- | --- | --- |
| $ab | Bool | T if the movement have any A or B information else the variable is nil. |
| $bar | Int | Actual pressure that will be used at the start of the piercing. |
| $circ | Bool | If circular piercing should be used or not. |
| $clen | Real | Cutting length without lead-in and lead-out. |
| $cpfeed | Real | The velocity used in circular piercing. |
| $cpradius | Real | This is the movement radius that should be used for circular piercing. |
| $cpturns | Real | Number of turns that should be used for circular piercing |
| $ctype | Int | 1: Tilted piercing 2: Tilted cutting (bevel mode is top, bottom or advanced) 4: Top bevel 8: Bottom bevel 16: Ruled bevel. |
| $external | Bool | T if the piercing belongs to an external geometry else nil (NOTE! See $edge below) |
| $edge | Int | -1 Means it’s an external contour 0 Means an open contour (slot or common cutline or marking) 1 Means it’s an inside contour |
| $gram | Int | Actual amount of abrasive that should be used at the start of the piercing. |
| $high | Bool | If the piercing is made with the same pressure as cutting then variable is T else nil. |
| $kerf | Int | The variable gives information about the kerf that will be used after the piercing is finish. 0=No kerf, 1=Left (G41), 2=Right (G42). Tis instruction shoul nomraly be written to the CNC-file. When using all kind of bevel cutting and when using IGEMS internal toll radius compensation this value is always 0. Note! See also $side that have a similar meaning. |
| $jetabrasiveon | Real | This is the time it takes for the abrasive to arrive the material after the CNC have send the signal Abrasive On. The value depends on the length of the hose and other equipment. Probably around 1 to 3 seconds. |
| $jetpreload | Real | If this value is a value, then the abrasive should be turned on before the water (opposite as normal piercing) In that case the variable is the time for the pre load. This is used on low pressure piercings, to avoid that the material is dammed. |
| $jetvalveon | Real | This value is the time in seconds that it takes for the waterjet valve to react after the CNC have send the signal Water ON. This is normally very short probably around 0.1 to 0.2 seconds. |
| $lastcutonpart | Bool | This variable I T if it is the last cut before the machining continuous on another part. This variable is set during the actual geometry LINE,ARC,CUTOFF. |
| $leadname | String | The name of the lead used in the Contour command. |
| $leadplan | List | New in IGEMS2: In normal 2D cutting this variable is (0 0 -1) and describe then normal into the material for the circular piercing and the lead movement. (See also $leadtilt below) |
| $leadtilt | Bool | If the lead-in and the circular piercing is not planar with XY-plan then the variable is T else nil. Use the functions CPIERC-LINE and CPIERC-CIRCLE to generate the circular movements. |
| $lowatoff | Bool | If this cut should end in low pressure, to have the value correct for next piercing. |
| $lowhigh | Bool | If it's a low pressure piercing without optimizing. Note! The variable $high is also T in this case, be sure to test $lowhigh first in a COND function. |
| $maxslope | Real | Maximum cutting angle that will be used on actual geometry. Note! This variable has another meaning in the Header function. The value is not negative and in degree. |
| $marking | Bool | The variable is T if the cuton belongs to a marking else this variable is nil |
| $nodename | String | The name of the actual toolpath. |
| $path5x | Bool | Actual path requires 5-axis kinematic |
| $piercingpos | List | A list of three reals describing the piercing position from the actual zero point. Note the third real is always from the bottom of the material. |
| $sensor | Int | This variable can be used on IGEMS CNC and instruct the controller what sensoring mode that should be used when using the Automatic sensoring function. 0=Use previous Z height probing for the Z-level. 1=Make a new Z-height probing. 2=Use Stamping sensoring (for straight cuts). 3=Use Dragging sensoring (for bevel cuts). |
| $pstat | Real | Piercing time on for stationary piercing (you can normally use $time that have the time for the actual piercing type). |
| $ptype | Int | The piercing type: -2=Point marking -1=Marking 0=Blind lead 1=Linear piercing 2=Stationary piercing 3=Circular piercing 4=Drilling with no optimizing (Se also the drilling function) 5=Air start 6=User (The piercing type 4 means drilling and it do not use the Piercing function, it use the drilling function.) |
| $side | Int | The variable gives information about the tool side used when making the toolpath. This value is 0, 1 or 2 also when IGEMS Internal tool radius compensation are used. This variable is similar as $kerf, but for special cases when it’s important to know the kerf side. 1 or 2 does not means that G41 or G42 should be used in the CNC-file. |
| $speedramp | Bool | Flag for the speed optimization command. |
| $startindex | Int | New in IGEMS2.LSP: A counter for each new piercing, pre-piercing or marking. The counter starts with 1 at the first piercing. See also $startmax at the HEADER |
| $starttan | Real | Angle of the first cutting direction on the geometry. Note this value is in radians. This variable is also available in the RAPID function. |
| $tacflag | Bool | This 2D geometry should be cut with TAC |
| $time | Real | The time for the piercing. |
| $useabr | Bool | Is T if abrasive should be used else nil |
| $usebevel | Bool | This value is T if the actual cut has Bevel cuts. |
| $usebeveltac | Bool | This value is T if TAC is activated on Bevel cuts |
| $usetac | Bool | This value is T if TAC will be used on the actual 2D-geometry. Note! This flag is nil on all Bevel cuts. |
| $vaccumAssist | Bool | This value is T is vacuum assist should be used on low pressure piercings. The function is only activated on Sheet wise, Tool wise and Part wise pre-piercing. |
| $vacuumOndelay | Real | Delay time between vacuum on and abrasive on |
| $wacode | Bool | Is T if different codes should be used for Abrasive and water on. Is nil if only one code should be used. |

Example

(DEFUN piercing()

(IF $jetpreload

(PROGN

(WRITE (STRCAT (NTXT) "M06" (MSG "ABRASIVE ON")))

(DELAY $jetpreload)

(WRITE (STRCAT (NTXT) "M04" (MSG "WATER ON")))

(DELAY $jetvalveon))

(PROGN

(WRITE (STRCAT (NTXT) "M04" (MSG "WATER ON")))

(DELAY $jetvalveon)

(IF $useabr

(PROGN

(WRITE (STRCAT (NTXT) "M06" (MSG "ABRASIVE ON")))

(DELAY $JetAbrasiveOn)))

)

)

(COND

((NOT $circ)     ; If it's a non circular piercing, wait $time seconds

(DELAY $time))

((NOT $leadtilt) ; Circular piercing on flat material

(SETQ *CustomCircularPiercing* T) ; Activate simulation

(WRITE (STRCAT (NTXT) "M12 S" (RTT $time)))); Call a fantasy "M12"

(nil  ; Create long file without variables and logical conditions

(SETQ *CustomCircularPiercing* nil)

(CPIERC-LINE T)

(REPEAT $cpturns (CPIERC-CIRCLE))

(CPIERC-LINE nil))

(T ; Create short file useing variables and logical conditions

(SETQ *CustomCircularPiercing* T)

(CPIERC-LINE T)

(WRITE (STRCAT (NTXT) "R20=" (ITOA $cpturns)))

(WRITE (STRCAT (NTXT) "$WHILE R20"))

(WRITE (STRCAT (NTXT) "R20=R20-1"))

(CPIERC-CIRCLE)

(WRITE (STRCAT (NTXT) "$ENDWHILE"))

(CPIERC-LINE nil))

)

)

List of postprocessor functions

LINE3D

The function is used to move the jet to correct distance after tooldown or tooldown3d if Increased Piercing Height is used.

| $vertical | Bool | This variable is T if the motion is only in Z axis. |
| --- | --- | --- |
| $x | Real | The X-coordinate of the endpoint of the movement. |
| $y | Real | The Y-coordinate of the endpoint of the movement. |
| $z | Real | The z-value for the tool refer to table, thickness or highest position. This can be set in the machine setting. |

Example:

(DEFUN line3d ()

(IF $vertical

(WRITE (STRCAT (NTXT) "G1 Z" (RTS $z) " F" (RTS $fe)))

(WRITE (STRCAT (NTXT) "G1 X" (RTS $x) " Y" (RTS $y) " Z" (RTS $z) " F" (RTS $fe)))

)

)

List of postprocessor functions

LINE

This is the basic movement event. In fact, all movements can be with lines.

The function can use following variables:

| $a | Real | The angle in radians of the major axis. If no 5-axis equipment is in use this variable is always 0. |
| --- | --- | --- |
| $amotor | Real | The angle in radians of the major axis. The value is always in motor degree. |
| $ab | Bool | T if the movement has any A or B information else the variable is nil. |
| $b | Real | The angle in radians of the minor axis. If no 5-axis equipment are in use this variable is always 0 |
| $bmotor | Real | The angle in radians of the minor axis. The value is always in motor degree. |
| $c | Real | The angle in radians of the 6th axis. If no rotation unit are in use this variable is always 0.0 |
| $chkerf | Bool | The value is T if the kerf should be activated or deactivated |
| $delta | Real | This value describes the tangential angle between current LINE and next movement. The value is 0 if next movement is in same direction. It’s negative is next movement start in minus relative direction. It’s positive if next object start in positive relative direction. The value is always between –PI to +PI |
| $fe | Real | The cutting speed at the end of the movement in mm/min. If using the RTF function, then the speed will automatically be calculated inch/min or mm/min. |
| $flag | Int | This is a bit value with following meaning: 1=Belongs to a lead-in 2=Belongs to a lead out 4=Belongs to marking 8=Belongs to circular piercing 16 32 64=Belongs to a custom line. |
| $fs | Real | The cutting speed at the start of the movement. This value can be different from $fe when using cutting speed interpolation. If not using cutting speed interpolation then this value is always the same as $fe. |
| $ft | Real | The time for the movement in minutes. |
| $kerf | Int | Actual kerf: 0=No kerf (G40), 1=Tool on right side (G41), 2=Tool on left side (G42) |
| $last | Bool | This variable I T if it's the last movement before jet turns of |
| $len | Real | The length of the line. |
| $slope | Real | The max cutting angle at this position. Vertical is 0.0 The angle is caclulated in the vector direction as radians. |
| $vector | list | A list of three values that describe the direction of the jet. Example: (0 0 -10). The X,Y Z is always in IGEMS coordinates no matter if the Axis direction settings. The distance between (0 0 0) to the vector list describes the cutting length at each point. |
| $x | Real | The X-coordinate of the endpoint of the movement. The value is transformed according to the axis direction in the machine setup. |
| $y | Real | The Y-coordinate of the endpoint of the movement. The value is transformed according to the axis direction in the machine setup. |
| $z | Real | The z-value for the tool refers to table, thickness or highest position. This can be set in the machine setting. |
| $zref | Int | This variable is nil if no pre-measuring tool are used. In other case it’s an integer that refers to that measure point that is measured by the MEASURE function. |
| $uses6x | Bool | This variable is T if a rotation axis should be used. |

Example:

(DEFUN line (/ bevstr rotstr feedstr)

(SETQ bevstr (IF $ab (STRCAT " A" (ATS $a) " B" (ATS $b)) ""))

(SETQ rotstr (IF $uses6x (STRCAT " C" (ANGTOS $c 3 nil)) ""))

(SETQ feedstr (IF (NOT (EQUAL $fe oldfeed 0.5)) (STRCAT " F" (RTF (SETQ oldfeed $fe))) ""))

(IF $chkerf (WRITE (STRCAT (NTXT) "G4" (ITOA $kerf))))

(WRITE (STRCAT (NTXT) "G1 X" (RTS $x) " Y" (RTS $y) " Z" (RTS $z) bevstr rotstr feedstr))

)

If the function should write angle information for major and minor axis it will set the variable else it be set to an empty string.

If the speed is changed more than 0.5 mm/min then the variable feedstr will be set to that value else to an empty string. If you are using a cutting speed interpolation, then you can use the variables $fs and $fe . By comparing these values, you can calculate if the cutting speed on the line is in retardation or in acceleration mode or if the cutting speed is the same for the whole object.

List of postprocessor functions

ARC

This event handles circular movements. If you do not include a function for this event then the movement will be done LINE vectors.

The function can use the following variables:

| $a | Real | The angle in radians of the major axis. If no 5-axis equipment are in use this variable is always 0. The value is transformed to the actual kinematic. |
| --- | --- | --- |
| $amotor | Real | The angle in radians of the major axis. The value is always in motor degree. |
| $ab | Bool | T if the movement have any A or B information else nil. |
| $b | Real | The angle in radians of the minor axis. If no 5-axis equipment are in use this variable is always 0 |
| $bmotor | Real | The angle in radians of the minor axis. The value is always in motor degree. |
| $c | Real | The angle in radians of the 6th axis. If no rotation unit are in use this variable is always 0.0 |
| $ccw | Bool | The variable I T if the movement is counterclockwise (G03). Else the movement is clockwise (G02). |
| $fe | Real | The cutting speed at the end of the movement in mm/min. If using the RTF function, then the speed will automatically be calculated inch/min or mm/min. |
| $flag | Int | This is a bit value with following meaning: 1=Belongs to a lead-in 2=Belongs to a lead out 4=Belongs to marking 8=Belongs to circular piercing 16 32 |
| $fs | Real | The cutting speed at the start of the movement. This value can be different from $fe when using cutting speed interpolation. If not using cutting speed interpolation, then this value is always the same as $fe. |
| $ft | Real | The time for the movement in minutes. |
| $i | Real | Circle center in X (the value can be in absolute or incremental mode depending on the geometry settings in the machine setup). |
| $Iabs | Real | Circle center in X |
| $Irel | Real | Circle center in X relative from arc start point |
| $j | Real | Circle center in Y (the value can be in absolute or incremental mode depending on the geometry settings in the machine setup. |
| $Jabs | Real | Circle center in Y |
| $Jrel | Real | Circle center in Y relative from arc start point |
| $last | Bool | This variable is T if this is the last movement on the contour. |  |  |
| $len | Real | The length of the arc |
| $r | Real | The radius of the movement. On many controllers this value can be used for circular movements with an included angle less than 180 degree |
| $segment | Real | The segment angle of the arc. The value is positive for CCW and negative for CW. |
| $short | Bool | The variable is T if the move segment angle is less than 3.0 radians (171.89 degree) |
| $slope | Real | The max cutting angle at this position. Vertical is 0.0 The angle is caclulated in the vector direction as radians. |
| $vector | list | A list of three values that describe the direction of the jet. Example: (0 0 -1). The X,Y Z is always in IGEMS coordinates no matter if the Axis direction settings. |
| $x | Real | The endpoint in X (the value is transformed according to the Axis direction setting in the Machine Setup). |
| $xm | Real | The midpoint of the arc in X. This is the standard method for robot programming of arcs |
| $y | Real | The endpoint in Y (the value is transformed according to the Axis direction setting in the Machine Setup). |
| $ym | Real | The midpoint of the arc in Y. This is the standard method for robot programming of arcs |
| $z | Real | The z-value for the tool refers to table, thickness or highest position. This can be set in the machine setting. |
| $zref | Int | This variable is nil if no pre-measuring tool is used. In other case it’s an integer that refers to that measure point that are measured by the MEASURE function. |

Example:

This example use the R (radius) if the included angle is less than 180 degree

(DEFUN arc (/ bevstr rotstr feedstr)

(SETQ bevstr (IF $ab (STRCAT " A" (ATS $a) " B" (ATS $b)) ""))

(SETQ rotstr (IF $uses6x (STRCAT " C" (ANGTOS $c 3 nil)) ""))

(SETQ feedstr (IF (NOT (EQUAL $fe oldfeed 0.5)) (STRCAT " F" (RTF (SETQ oldfeed $fe))) ""))

(IF $ccw
    (WRITE (STRCAT (NTXT) "G03 X" (RTS $x) " Y" (RTS $y) " I" (RTS $i) " J" (RTS $j) bevstr rotstr feedstr))

(WRITE (STRCAT (NTXT) "G02 X" (RTS $x) " Y" (RTS $y) " I" (RTS $i) " J" (RTS $j) bevstr rotstr feedstr))

)

)

List of postprocessor functions

ROTAX

This function is called when there is no motion in X, Y or Z. The only motion is on the rotation axis A, B or C.

When this happens, it is not possible to use speed in mm/minute or inch/minute. The speed must be entered by time or by degree/minute.

If this function is not declared in the postprocessor, then the postprocessor will give a warning and use the LINE function. The speed will probably be incorrect in that case.

The following variables are available:

| $arel | Real | The relative changing of the primary axis in radians. |
| --- | --- | --- |
| $brel | Real | The relative changing of the secondary axis in radians. |
| $crel | Real | The relative change of the rotation axis. If no rotation axis is available, then this value is 0 |
| $ft | Real | The time it should take for the movement in minutes. (Note in some older version the time was in seconds for 2D) |
| $a | Real | The major axis rotation in radians |
| $b | Real | The secondary axis rotation in radians |
| $c | Real | The rotation axis (tube cutting) in radians |
| $uses6x | Bool | This variable I T if a rotation axis is available. |
| $ton | Bool | The variable is T if time mode should be activated. Note! This is not used when using a rotation axis. |
| $toff | Bool | The variable is T if time mode should be deactivated. Note! This is not used when using a rotation axis. |

Some controllers switch automatically to degree/minutes if no X or Y movement is mentioned.

Following example can be used if the summary of the rotations of primary and secondary axis should be calculated:

(DEFUN rotax (/ rot)
  (SETQ drot (DISTANCE (LIST 0.0 0.0 0.0)

(LIST (RADTODEG $arel) (RADTODEG $brel) (RADTODEG $crel))))

(SETQ feedstr (STRCAT " F" (RTF (/ drot $ft))))

(WRITE (STRCAT (NTXT) "G1 C"(ATS $a) " A" (ATS $b) feedstr))
)

Use this example if the speed should be calculated in that rotation axis that have the most rotation:

(DEFUN rotax (/ rot)
  (SETQ rot (MAX (ABS (* (/ 180 PI) $arel)) (ABS (* (/ 180 PI) $brel))))
  (SETQ feedstr (STRCAT " F" (RTF (/ rot $ft))))
  (WRITE (STRCAT (NTXT) "G1 C"(ATS $a) " A" (ATS $b) feedstr))
)

Use this example if you want to use speed defined by Invers time (normally G93)

(DEFUN rotax ()
  (SETQ feedstr (STRCAT "F" (RTOS (/ 1 $ft) 2)))
  (IF $ton (WRITE (STRCAT (NTXT) "G93")))
  (WRITE (STRCAT (NTXT) "G1 C"(ATS $a) " A" (ATS $b) feedstr))
  (IF $toff (WRITE (STRCAT (NTXT) "G94"))) ;change back to mm/min
)

On some machines use time in seconds instead of inverse time. In this case. How many seconds the motion will take

(DEFUN rotax (/ rotstr)
  (SETQ feedstr (STRCAT " F" (RTOS (* $ft 60.0) 3))); convert minutes to seconds
  (IF $ton (WRITE (STRCAT (NTXT) "G93"))) ; change to time mode
  (SETQ rotstr (IF $uses6x (STRCAT " C" (ATS $c)) ""))

(WRITE (STRCAT (NTXT) "G1 A"(ATS $a) " B" (ATS $b) rotstr feedstr))
  (IF $toff (WRITE (STRCAT (NTXT) "G94"))) ;change back to mm/min
)

List of postprocessor functions

REWIND

On kinematic solutions where the major axis is vertical or almost vertical then the construction will do a lot of rotations. Many constructions have a mechanical limit of how many degrees the major axis can rotate in negative or positive direction. This function is called before this is happen, it will rotate

Sometimes the rotation can come to a mechanical limit, then this function will close the jet then rotate the jet back or forward to avoid the mechanical limit. How much the major axis will rotate is depend of the limits and the geometry.

| $a | Real | The major axis rotation in radians |
| --- | --- | --- |
| $arel | Real | The relative changing of the primary axis in radians. |
| $b | Real | The secondary axis rotation in radians |
| $brel | Real | The relative changing of the secondary axis in radians. |

Example:
(DEFUN rewind ()
  (WRITE (STRCAT (NTXT) "M5 (JET OFF)"))
  (WRITE (STRCAT (NTXT) "G00 A" (ATS $a) " B" (ATS $b)))
  (WRITE (STRCAT (NTXT) "M3 (JET ON)"))

)

List of postprocessor functions

FLYCTRL

NOTE! This function will probably be deprecated.

This function will only be called if the checkbox "Flying jet ON/OFF control" is activated in the machine settings. This functionality can be used on CNC machines that can turn ON/OFF water and abrasive without making a stop in the movement. In the HEADER function it's a variable $useflyctrl that are T of the FLYCTRL function are activated in the machine settings.

| $reason | Int | 1=Turn on water 2=Turn on abrasive 3=Turn off abrasive 4=Turn off water |
| --- | --- | --- |

Note!

On some controllers it's also important where in the line the M-code is written.

"G1 X100 Y100 M3" or "M3 G1 X100 Y100"

may give different result.

Example for IGEMS cnc:

(DEFUN flyctrl ()

(COND

((= $reason 1) (WRITE (STRCAT (NTXT) "M3" (MSG "FLYING WATER ON"))))

((= $reason 2) (WRITE (STRCAT (NTXT) "M7" (MSG "FLYING ABRASIVE ON"))))

((= $reason 3) (WRITE (STRCAT (NTXT) "M9" (MSG "FLYING ABRASIVE OFF"))))

((= $reason 4) (WRITE (STRCAT (NTXT) "M5" (MSG "FLYING WATER OFF"))))

)

)

List of postprocessor functions

CUTOFF

This function turns off cutting and waterjet marking.

Note!
Variable $release and $delay are now used if the pump has no bleed out valve for releasing the pressure and the variable $time is deprecated.

The function can use following variables:

| $chkerf | Bool | This variable is T if the kerf should be changed else this variable is nil |
| --- | --- | --- |
| $delay | Real | If the pressure should be reduced during before the water is turned off then this is the time it takes to reduce the pressure. See also example below and the $relaese variable. |
| $jetabrasiveoff | Real/Bool | This is the time it takes to empty the hose after the Abrasive has been turned off. The value is also depending on the length of the abrasive hose. Normal value is around 1 to 3 seconds. If the abrasive already is turned off (because off flying on/off control) or if you cut in pure water then this value is nil. |
| $jetvalveoff | Real/Bool | This value is the time in seconds that it takes for the waterjet valve to react after the CNC have send the signal Water ON. This is normally very short probably around 0.1 to 0.2 seconds. |
| $kerf | Int | If $chkerf is T then the variable handle what the kerf should be changed to. If $chkerf is nil then the variable shows the actual kerf. 0=No kerf (G40), 1=Right side (G41), 2=Left side (G42) |
| $lowatoff | Bool | T if next piercing should be in low pressure else nil. |
| $marking | Bool | The variable is T if the cutoff belongs to a marking else this variable is nil |
| $release | Bool | If the pressure should be reduced by having the water on while the pump is reducing the pressure. See example below and $delay variable above. |
| $useabr | Bool | This variable is T if abrasive is used else this value is nil. |
| $wacode | Bool | This variable is T the controller is using different codes for abrasive and water off. |

Example

(DEFUN cutoff ()

(IF $Jetabrasiveoff

(PROGN

(WRITE (STRCAT (NTXT) "M7" (MSG "ABRASIVE OFF")))

(DELAY $JetabrasiveOff)))

(IF $JetValveOff

(PROGN

(IF $release

(PROGN

(WRITE (STRCAT (NTXT) "M76 ;LOW PRESSURE"))

(DELAY $delay)))

(WRITE (STRCAT (NTXT) "M5" (MSG "WATER OFF")))

(DELAY $jetvalveoff)))

(IF $chkerf (WRITE (STRCAT (NTXT) "G4" (ITOA $kerf))))

)

List of postprocessor functions

TOOLUP

TOOLUP3D

TOOLUP is used if the jet is moved up in a vertical direction (within 0.01 mm).

TOOLUP3D is used if the jet moves in a non-vertical direction.

The function is only called if the machine has a movable Z-axis that can be controlled from the CNC-file.

It’s called every time the tool should be moved up from cutting position to a specified lift height.

The following variable is available.

| $lastcutonpart | Bool | This value is T if it’s the last cut on the part else nil. Note! This variable can only be used for cutting and pre-piercing. It is not set on Drilling, Marking, Special marking or Inkjet marking. |
| --- | --- | --- |
| $msg | String | An automatic generated text that describes the reason of Toolup |
| $fz | Real | The velocity for the rapid speed in mm/min |
| $reason | Int | The reason means following (0=Called if you have a fixed Z) 1=Initialization in the beginning of the file. 2=Between drillings 3=Between markings 4=Between pre-piercings 5=Between cuts in a part 6=Between two parts in cutting or at last part 7=After cleaning or pressure educe. 9=Move up after sensoring. |
| $x | Real | It’s important to use this X-movement in the TOOLUP3D function. |
| $y | Real | It’s important to use this Y movement in the TOOLUP3D function. |
| $z | Real | The lift height in mm. The value is checked against over travel and can't exceed Max Z-axis movement minus the material thickness and the Standoff. |
| $zmode | Int | The $zmode have following meaning. 0=Fixed Z-axis. 1=Automatic, Up/Down controlled by water On/Off. 2=Up/Down with separate M-codes. 3=Up/Down controlled by Z-coordinates. 4=Automatic Height sensor that are controlled by water On/Off. 5=Height sensor that are controlled by M-codes. 6=Height sensor that are controlled by Z-coordinates. 7=By measuring probe information. If the machine has a pneumatic stroke, then this can be used together with all options above. |

Example:

(DEFUN toolup ()
  (IF sensorisdown (WRITE (STRCAT (NTXT) "M45 (SENSOR OFF)")))

(IF (> $reason 1)

(WRITE (STRCAT (NTXT) "G0 Z" (RTS $z) " (TOOL UP)")))

)

The sample above will turn off the sensor "M45" if the variable sensorisdown is True. The variable should be set in tooldown or tooldown3d function. Toolup will move up Z-axis if the $reason is higher than 1.

Example 2:

(DEFUN toolup3d ()

(IF sensorisdown (WRITE (STRCAT (NTXT) "M45 (SENSOR OFF)")))

(WRITE (STRCAT (NTXT) "G1 X" (RTS $x) " Y" (RTS $y) " Z" (RTS $z) " F" (RTF $fz) (TOOL UP3D))) )

In this function it is important to move the jet with X Y and Z-coordinates using G1 with a feedrate.

List of postprocessor functions

SRAPID

Srapids is used for localization of the part before cutting, and it's implemented just in a few postprocessors.

The SRAPID function handles the Start rapid points that can be entered from the part order command. The function is called in the beginning of postprocessing.

The SRAPID function can use following variables.

| $attrib | String | This is a text string with the text attribute (used to handle different equipment) |
| --- | --- | --- |
| $idx | Int | This number is counted down with one for each SRAPID. If there is 3 SRAPID the number will be 3, 2, 1 |
| $x | Real | The X-coordinate of the endpoint of the movement. |
| $y | Real | The Y-coordinate of the endpoint of the movement. |

The SRAPID will never be called to handle normal rapid events.

List of postprocessor functions

STOP

The stop event is used to generate a stop in the cutting process.

Following variables are available.

| $msg | String | A text message that is added in the Edit toolpath or Order command or an automatic generated text that comes from the stop options in the Method window. |
| --- | --- | --- |
| $cond | Bool | The variable is T if the checkbox Conditional is activated in the Stop dialog else nil. |
| $reason | Int | The reason value has following meaning: 0=Generated in case of TOOLSET and other automatic generated stops because of not complete postprocessor. 1=A stop generated in the Edit part command. 2=A stop generated in the Order command. 3=If the option "Stop after first cut" is activated. 4=If the option "Stop after first part" is activated. 5=If the option "Always pre-piercing is activated". This is mainly developed for 5-axis cutting, the STOP event will be used just before the cutting start. 6=If no cleaning or pressure release point is specified. 7=Stop generated by the Stop command in 3D-5X |

Example 1:
This example will add M00 and automatic generated message

(DEFUN stop ()
  (WRITE (STRCAT (NTXT) "M00 (" $msg ")"))
)

Example 2:
This example shows how the STOP event can be customized. Depending on the $reason you can add different information to the CNC-file.

(DEFUN stop ()
  (COND
    ((= $REASON 1)
      (WRITE (STRCAT (NTXT) "M00 (STOP IN EDIT PART MESSAGE: " $msg ")")))
    ((= $REASON 2)
      (WRITE (STRCAT (NTXT) "M00 (STOP IN ORDER MESSAGE: " $msg ")")))
    ((= $REASON 3)
      (WRITE (STRCAT (NTXT) "M00 (STOP AFTER FIRST CUT)")))
    ((= $reason 4)
      (WRITE (STRCAT (NTXT) "M00 (STOP AFTER FIRST PART)")))
    ((= $reason 5)
      (WRITE (STRCAT (NTXT) "M00 (STOP BEFORE 5X-CUTTING)")))
  )
)

List of postprocessor functions

DRILLING

This event activate the drilling cycle. The drilling tool is already in position for drilling.

| $attrib | String | The variable shows the value entered in the HOLE drilling function. If no attribute are entered or if the drilling are used as piercing then the value is an empty string. |
| --- | --- | --- |
| $depth | Real | The incremental depth from the start points x, y, z |
| $drillbytype | int | By this variable you can read what kind of optimizing the drilling sequences is set to: 0=Sheetwice, 1=Toolwice, 2=Partwice, 3=No optimation. When using number 3 (No optimizing), then the PIERCING will set the variable $ptype to 4. |
| $drillfeed | Real | Drilling feed in mm/min (this variable is available also in the header). |
| $drillmax | Int | Number of drillings made together in this sequence. |
| $drillnum | Int | First drilling will have number 1 the second number 2 and so on. |
| $drillrpm | Int | Revolutions per minute (this variable is available also in the header). |
| $drillstep | Real | Drilling step (this variable is available also in the header). |
| $f | Real | Drilling feed in mm/min |
| $height | Real |  |
| $holediameter | Real | If the drilling is made with the HOLE command and the source is a circle then ths variable is the diameter of the circle. When the drilling is done for piercing or if the source is not a circle then the value is 0. |
| $x | Real | The X-coordinate for the drilling position. |
| $xraw | Real | The X-coordinate for height sensoring position. |
| $y | Real | The Y-coordinate for the drilling position. |
| $yraw |  | The Y-coordinate for height sensoring position. |
| $z | Real | The z-coordinate for the drilling position at the surface. |
| $zclearence | Real | The height that the drill should move up to after the drilling is finish. This value is a relative distance from the Z value. |

List of postprocessor functions

FMODE

This function handles different speed modes.

The function has following variables:

| $flin | Int | The variable can have following settings: 0=Flin main switch. Flin should be deactivated 1=Flin main switch. Flin should be activated, next movements has different start and end speed. 2=Next movement have another start speed than last movements end speed. |
| --- | --- | --- |
| $fe | Real | The cutting speed at the end of next object. |
| $fs | Real | The cutting speed at the beginning of next object. |

The variable $flin can be used in other function. It’s nil if FLIN in not activated. In all other cases it is 0, 1 or 2.

List of postprocessor functions

TEXT

If you want the text should be generated by X, Y movements using the waterjet or the special marking tool then you can skip this function. It is handled automatically by a pre-defined inbuild text function.

If you have some special text marking device that you want to support, then you can define your own text function.

The function can use following variables:

| $angle | Real | The angle of the text |
| --- | --- | --- |
| $attrib | String | The attrib as is was given in the Marking command. |
| $height | Real | The text height |
| $length | Real | The length of the text |
| $font | String | The name of the text font |
| $txt | String | The text in Unicode. |
| $fonttype | Int | The type of font. 0=trueType |
| $spmark | Bool | The variable is T if the text marked by special marking else nil |
| $x | Real | The X-coordinate of the center of a box containing the text. |
| $y | Real | The Y-coordinate of the center of a box containing the text |
| $z | Real | The Z-coordinate of |

Example:

(DEFUN text ()

(WRITE (STRCAT (NTXT) "G0 X" (RTS $x) " Y" (RTS $y) " Z" (RTS $z)))

(WRITE (STRCAT (NTXT) "TEXTMARK(" (CHR 34) $txt (CHR 34) ")"))

)

List of postprocessor functions

MARKON

This function is used for activate marking for special marking tool. (Not for waterjet marking).

| $starttan | Real | Angle of the first cutting direction on the geometry. Note this value is in radians. This variable is also available in the RAPID function. |
| --- | --- | --- |
| $attrib | String | This is a text string with the text attribute (used to handle different equipment) |

Example:

(DEFUN markon ()

(WRITE (STRCAT (NTXT) "M17 (SPECIAL MARKING ON)"))

)

List of postprocessor functions

MARKOFF

This function is used for deactivate the special marking tool (Not for waterjet marking).

Example:

(DEFUN markoff ()

(WRITE (STRCAT (NTXT) "M19 (SPECIAL MARKING OFF)"))

)

List of postprocessor functions

ITEXT

This function is called if you are using inkjet equipment.

| $angle | Real | The angle in radians of the text. |
| --- | --- | --- |
| $length | Real | The length of the text. |
| $height | Real | The height of the text. |
| $font | String | The font used. |
| $txt | String | The text that should be printed. The $txt is always the first object in the $txtlist and this is used for compatibility reason. |
| $txtlist | List | A list with several strings. This to support inkjet printers with multiline capability. |
| $x | Real | The center of the text in X |
| $y | Real | The center of the text in Y |
| $z | Real | The center of the text in Z |
| $fe | Real | The cutting speed at the end of the movement. |

Example:

(DEFUN itext ()
  (WRITE (STRCAT (NTXT) "G00 X" (RTS $x) " Y" (RTS $y) " Z" (RTS $z))) 
  (WRITE (STRCAT (NTXT) "A:" (ATS $angle) " (ANGLE OF TEXT)"))
  (WRITE (STRCAT (NTXT) "L:" (RTS $length) " (LENGTH OF TEXT)"))
  (WRITE (STRCAT (NTXT) "H:" (RTS $height) " (HEIGHT OF TEXT)"))
  (WRITE (STRCAT (NTXT) "F:" $font " (TEXT FONT)"))
  (SETQ $txt "")

(FOREACH s $txtlist (SETQ $txt (STRCAT $txt s "/")))

(SETQ $txt (STRSUB $txt 1 -2))

(WRITE (STRCAT (NTXT) "T:" $txt)) ; text to print
  (WRITE (STRCAT (NTXT) "M100 (PRINT)"))
  (WRITE "")
)

In the example above the forward slash "/" is used to separate lines.

List of postprocessor functions

MEASURE

This function is called if the option "Measuring in advance" is activated. 
The idea is that the machine will measure the height on the given positions, and the height is stored in the controller memory for later use during the cutting. The method is often used on 5-axis cutting machines that can’t use a height sensor during the cutting.
The function can use following variables.

| $x | Real | The X-coordinate of the endpoint of the movement. |
| --- | --- | --- |
| $y | Real | The Y-coordinate of the endpoint of the movement. |
| $zref | Int | This variable is index that counts up by one for each point. The index starts with 0 or 1 for each new measuring sequence. |

Example:

(DEFUN measure ()
  (WRITE (STRCAT (NTXT) "G00 X" (RTS $x) " Y" (RTS $y)))
  (WRITE (STRCAT (NTXT) "MEASURE(" (ITOA $zref) ")"))

(SIM-DELAY 2.5)

)

The function could generate following CNC-file.

N12 G00 X200.513 Y201.678

N13 MEASURE(1)

N14 G00 X200.000 Y101.780

N15 MEASURE(2)

N16 G00 X200.000 Y2.000

N17 MEASURE(3)

N18 G00 X102.000 Y0.000

N19 MEASURE(4)

List of postprocessor functions

MEASUREPOINT

This instruction comes from the command "Measure points" in 3D-5X. This command is for automatic pre-location of parts on the CNC machine before the machine starts to work. Therefor all measurepoint will come as the very first operations. This is a very special function that are in use just by a few customers.

| $x | real | X position near the model. |
| --- | --- | --- |
| $y | real | Y position near the model. |
| $z | real | X position near the model. |
| $probetype | int | This is the probetype selected in 3D-5X |
| $index | int | a |
| $vector | list | This is a direction vector that starts at the x y z. The measuring will start at the end of the vector. |

List of postprocessor functions

REFPOS

This function will be called if the Ref pos option is used in the toolpath command in 3D-5X. The function will be called just before the RAPID to the piercing point before cuton.

| $a | Real | The major axis rotation in radians |
| --- | --- | --- |
| $b | Real | The secondary axis rotation in radians |
| $x | Real | The position in X |
| $y | Real | The position in Y |
| $z | Real | The position in Z |
| $vector | List | A list of three values that describe the direction of the jet. Example: (0 0 -1). The X,Y Z is always in IGEMS coordinates no matter if the Axis direction settings. |

(DEFUN refpos ()
  (WRITE (STRCAT (NTXT) "(REFPOS X" (RTS $x) " Y" (RTS $y) " Z" (RTS $z) 
        " C"(ATS $a) " A" (ATS $b) " VECTOR:" (TOSTR $vector) ")"))
)

List of postprocessor functions

MRAPID

This event should move the tools to the X, Y position using the machine coordinate system. The picture below is from the Various tab in the Strategy command.

| $x | Real | The X-coordinate for the parking position. This coordinate are not transformed. It’s exactly the value that are set in picture above. |
| --- | --- | --- |
| $y | Real | The Y-coordinate for the parking position. This coordinate are not transformed. It’s exactly the value that are set in picture above. |
| $reason | Int | This value have the following meaning: 1=If the MRAPID is called on making a Parking position Other $reasons may be added in the future. |

Example:

(DEFUN mrapid ()

(WRITE (STRCAT (NTXT) "G53 G0 X" (RTS $x) " Y" (RTS $y) " (PARKING POSITION)"))

)

List of postprocessor functions

FOOTER

This function are the last function that will be called while the CNC-file is open. Following variable can be used.

| $abrasiveamount | Real | Total abrasive consumption for actual job. |
| --- | --- | --- |
| $c | Real | If using a rotation axis ($uses6x = T) this is the rotation of the axis (part). |
| $cutlen | Real | Total cutting length for one jet |
| $datefirst | String | The variable is a string returned by the DATETOSTR function Example: "20201015" yyyymmdd. The value is the earliest (first) date of any part in the cut order. The value is used in IGEMS cnc. |
| $runsec | Int | Theoretically estimated time for the machine to do the job. |
| $runtime | String | The values above converted to a string |
| $igems | String | This is a string with the version of IGEMS. |
| $post | String | This is a string with the name of the postprocessor |
| $pumpstop | Bool | T if the pump should be stopped automatically else nil |

Example:

(DEFUN footer()
  (WRITE (STRCAT (NTXT) "(SECONDS " (ITOA $runsec) ")"))
  (WRITE (STRCAT (NTXT) "(USED TIME " $runtime ")"))
  (WRITE (STRCAT (NTXT) "(IGEMS:" $igems " POST:" $post " Date:" datum ")")))
  (WRITE (STRCAT (NTXT) "M30"))
  (WRITE "%")
)

List of postprocessor functions

FINAL

This function is called after that all postprocessing has been executed and the CNC-file is closed.

The example below open the file again, add a new line in the beginning.

(DEFUN final (/ txtlist txt fr lines fw)

(SETQ txtlist nil)

(SETQ fr (OPEN $filename "r"))

(IF (= $processmode 3)

(PROGN

(WHILE (SETQ txt (READ-LINE fr))

(SETQ txtlist (CONS txt txtlist))

)

(SETQ txtlist (REVERSE txtlist))

(CLOSE fr)

(SETQ lines (1+ (LENGTH txtlist)))

(SETQ fw (OPEN $filename "w"))

(WRITE-LINE (STRCAT "LINES: " (ITOA lines)) fw)

(FOREACH txt txtlist

(WRITE-LINE txt fw)

)

(CLOSE fw)

)

)

)

List of postprocessor functions

Postprocessor support functions

Following predefined function is very helpful when you create the postprocessor.

| OPTIONS | Add extra inputs to the Option dialog that shows when the postprocessor starts. |
| --- | --- |
| WRITE | Write a new line in the CNC file. |
| NTXT | Function for adding N<numbers> in the beginning of each line in the CNC file. |
| RTS | RealToString: Convert a real value to a coordinate value. |
| RTF | RealToFeed: Conver a real value to speed. |
| RTT | RealToTime: Convert a real value to time. |
| FTT | FeedToTime: Convert a real value to time when using time programming. |
| RTP | RealToPrice: Convert a real value to price (two decimals) |
| ATS | AngleToString: Convert a radian value to an angle. |
| MSG | Add a message string. |
| CPIERC-LINE | Functions to generate circular piercing movements. |
| CPIERC-CIRCLE | Functions to generate circular piercing movements. |
| RTCP | This function can be used if the controller cannot use RTCP. |
| CUSTOM-FILENAME | Define this function in the postprocessor if you want to customize own rules for suggesting filenames for the CNC file. |

The support functions exists only during the postprocessing, they cannot be used in applications. Below is an example how to use the function calls.

Example:

(WRITE (STRCAT (NTXT) "G1 X" (RTS $x) " Y" (RTS $y) " F" (RTF $fe) " " (MSG " (LINE)")))

(OPTIONS <caption> <variable> <default>)

This function will add extra inputs to the options dialog that shows every time postprocessing starts.

| caption | String | This is the text that will be shown in the option dialog |
| --- | --- | --- |
| variable | String | This is the variable name you want to use. Note! This must be a string. |
| default | Bool Real | If this argument is a real value, then it will be a new edit box that accepts values from minus 10000 to plus 10000. If the value is T or nil it will be a checkbox. |

The function should be written outside other function in the beginning of the postprocessor as the example below.

The example below adds two additional lines in the Option dialog

(SETQ *postinit* 1302)

(SETQ datum "2025.March.17")

(PRINC (STRCAT "Loading Test.ipf date:" datum " .."))

(OPTIONS "Standoff distance" "$standoff" 2.18)
(OPTIONS "Use Special equipment" "$UseSpecEqupment" nil)

To be able to add relevant questions, it is possible to read following variables already in this stage.

(Example: No idea to add question about drilling if drilling not will be used).

| $machine | str | This is then name of the selected machine. |
| --- | --- | --- |
| $digfile | str | Complete path and name of current drawing file. |
| $machinetype | Int | 0=AWJ 1=Pure water 2=Plasma 3=Oxyfuel 4=Laser |
| $cando5x | Bool | This variable is T if the machine has a 5-axis kinematic head else it is nil. |
| $cando6x | Bool | This variable is T if the machine has a rotation axis (in most cases to rotate a tube). |
| $candodrill | Bool | The variable is T if the machine has drilling equipment that can be used instead of piercing. |
| $candomarking | Bool | The variable is T if the machine has special marking tools else nil. |
| $candoinkjet | Bool | The variable is T if the machine has inkjet equipment. |
| $build | int | This is the build number of the IGEMS program. It’s the last portion of the version. 2025.1.4255 this means 4255. |
| $processmode | int | This number indicates which button is pressed on the process console. 0=Cost estimate. 1=Create report. 2=Simulation in 2D. 3=Create CNC file. 4=Simulation in 3D. 6=From application. |
| $headname | str | This is the name of the folder for the kinematic construction. Defined in the 5-axis kinematic tab in the machine settings. In this example the variable $headname is set to "IGEMSAC" |

Note! It's possible for us to add more variables. If you are missing some variable, then contact IGEMS.

(WRITE <text>)

Use this function to write information to the CNC-file. Depending on which process mode that are activated the function will do different things (Create CNC file, Cost estimate, Report, Simulation)

(NTXT)

The function returns "N" and an increasing number each time it is called. (N10, N11, N12, N13, N14)

The behavior is controlled from the Postprocessor tab in the Machine setting.

Format functions

(RTS <value>)

Real To String
The function will format the value to a string adapted for movements. The format is controlled from the Postprocessor tab in the Machine setting.

It is converted to an imperial value if the Output units is set to Imperial mode.

(RTF <value>)

Real To Feed

The function will format the value to a string adapted for speed. The format is controlled from the Postprocessor tab in the Machine setting and it is also converted to inches / minutes if output unit is set to Imperial mode in the same way as RTS.

(RTT <value>)

Real To Time

The function will convert and format the value to a decimal second. Number of decimals is controlled from the Postprocessor tab in the machine setting. The value is used for delays.

(FTT <value>

FeedToTime

When using rotation axis or for the ROTAX function the speed cannot be in mm/minute or inch/minute. It must be how long time the movement takes for each line in the CNC file. The number is controlled from the Postprocessor tab in the machine setting.

(RTP <value>)

Currencies are hardcoded to two digits. For example, 125.22€

(ATS <value>)

Angle To String

The function will convert and format the value (radians) to a degree angle. Number of decimals is controlled from the Postprocessor tab in the machine setting.

The angle can exceed 0-360 degrees and also be minus. If you need angles that always are between 0-360 degrees, then you can use the function ANGTOS.

(MSG <msg string>)

By adding the message text in this function, you can turn the message on or off from the output tab in the strategy settings.

Example:

(WRITE (STRCAT (NTXT) "G1 X" (RTS $x) " Y" (RTS $y) " F" (RTF $fe) " " (MSG " (LINE)")))

Circular piercing

Many CNC controllers are using macro for handling circular piercing. That will only work on 2D flat material. By using CPIERC-LINE and CPIERC-CIRCLE it is possible to use circular piercing also on advanced 3D parts when the circular movements are not parallel with XY plan (as the picture below).

(CPIERCE-LINE <start>)

This function should be used inside the PIERCING function. If the argument <start> is T then it will generate a Linear movement from the piercing position to the quadrant of the circular piercing movement. If the argument <start> is nil, then it will generate a linear movement back to center of the piercing position.

(CPIERCE-CIRCLE)

This function should be used inside the PIERCING function. It will generate movement for one full circle.

This will generate G3 movements in the CNC file if the piercing is parallel to XY plan. It will generate an octagon if the plan is not parallel with XY plan.

If the CNC controller don't have any programming possibilities, then the following example can be used.

(IF $circ

(PROGN

($CLINE $cpradius 0 $cpfeed)

(REPEAT $pturns

(CPIERC-CIRCLE)

)

(CPIERC-LINE nil))

(DELAY $time)

)

If you have a CNC controller where you can use some programming possibilities, then you can make a shorter CNC file by using following example.

(SETQ *CustomCircularPiercing* T)

(IF $circ

(PROGN

(CPIERC-LINE T)

(WRITE (STRCAT (NTXT) "R20=" (ITOA $cpturns)))

(WRITE (STRCAT (NTXT) "$WHILE R20"))

(WRITE (STRCAT (NTXT) "R20=R20-1"))

(CPIERC-CIRCLE)

(WRITE (STRCAT (NTXT) "$ENDWHILE"))

(CPIERC-LINE nil))

(DELAY $time)

)

Note: If you are using the example above you must insert following variable

(SETQ *CustomCircularPiercing* T)

(RTCP <$a> <$b>)

This function can be used if the CNC controller cannot use RTCP. The absolute best solution is if the RTCP calculation is done in the controller, and the coordinates in the CNC file describe the position of the nozzle. This is a workaround should only be used if the controller lacks that important functionality.

The function returns a list with three displacements (X Y and Z)

Example:

(IF $ab

(PROGN

(SETQ xyz (RTCP $a $b))

(SETQ $x (- $x (CAR xyz)) $y (- $y (CADR xyz)) $z (- (CADDR xyz)))

(WRITE (STRCAT (NTXT) "G1 X" (RTS $x) " Y" (RTS $y) " Z" (RTS $z))))

)

The function uses the parameters specified in the 5X-kinematic tab in the machine settings.

Theoretically this function can be used on all kinematic constructions based on two rotation axes.

(NORMALIZE $vector)

On events PIERCING, LINE, ARC the is a variable $vector that describe the theoretical direction and length of the jet at every point. This function will convert $vector to a new vector with the length 1mm.

Example:

(SETQ $vector (LIST 10 10 -20))

(DISTANCE (LIST 0 0 0) $vector)		returns 24.4940

(SETQ v2 (NORMALIZE $vector))

(DISTANCE (LIST 0 0 0) v2)		returns 1.0000

The function is useful on machines that uses vectors to handle cutting angles.

CUSTOM-FILENAME

Define a function with this name if you need IGEMS to suggest filenames that are built up by other rules than IGEMS standard rules. (The standard rules can be set up in the Postprocessor tab in the machine setting.) The function should return the suggested filename as a string valid for a filename.

The example below suggest the filename if previous name was not numbers.

If the previous filename starts with a number, then the number will be increased by 1 follow by the name of the drawing. If the previous filename starts with a number follow by a "-" and a second number (Example 1000-2) then the second number will be increased by 1 follow by the name of the drawing.

(DEFUN custom-filename (/ default serial decimal)

(SETQ serial (UC-GET $machine "CUSTOM-FILENAME-SERIAL" 1000))

(SETQ decimal (UC-GET $machine "CUSTOM-FILENAME-DECIMAL" 0))

(COND

((AND (= serial 0) (= decimal 0))

(STRCAT (FILE-NAMEONLY $digfile T)))

((= decimal 0)

(SETQ serial (1+ serial))

(STRCAT (ITOA serial) " " (FILE-NAMEONLY $digfile T)))

(T

(SETQ decimal (1+ decimal))

(STRCAT (ITOA serial) "-" (ITOA decimal) " " (FILE-NAMEONLY $digfile T)))

)

)

Following function must also be included in the MAIN to save the last used name.

(DEFUN update-filename (default / serial decimal)

(SETQ default (FILE-NAMEONLY default T))

(SETQ serial (ATOI default))

(UC-SET $machine "CUSTOM-FILENAME-SERIAL" serial)

(IF (> serial 0)

(SETQ decimal (ATOI (STRSUB default (+ (STRLEN (ITOA serial)) 2))))

(SETQ decimal 0)

)

(UC-SET $machine "CUSTOM-FILENAME-DECIMAL" decimal)

)

Note! You must call the function UPDATE-FILENAME in the HEADER function.

(DEFUN header ()

(IF $filename (UPDATE-FILENAME $filename))

(WRITE "%")

)

Rounder calibration

The IGEMS cnc has inbuild support for IGEMS rounder calibration. The Rounder equipment generate a compensation file that makes it possible for IGEMS cnc to modifies the XYZ positions to correct for deviations in the 5-axis kinematic. In IGEMS cnc the toolpath is corrected 500/second.

If you are using another CNC-controller you can use the functions below. This functions that can modify XYZ at each line in the CNC file. This is not optimal but the only way for other controllers that support the rounder feature.

Note! You can probably not use this functions on ARC movements without vectorizing.

(CMESH-INIT <filename>)

Should be called once early before the first call to (CMESH-GET). Loads the compensation data in the file given by <filename>If filename contains a search path, it is used as is, otherwise if only a file name, the compensation file is assumed to reside in the same directory as the postprocessor. The file should be in IGEMS ".ICF" format, otherwise an error will be thrown. Note that only the 2-axis input .icf files is currently supported.

Returns: an interpolation data object that is of no further use in ILISP at the moment.

Example:

(CMESH-INIT "c:/cmesh/cmesh.icf")

(CMESH-GET <a> <b>)

Gets the approximated compensation for a and b as a list of floating-point numbers.

The A and B argument should be in same unit format as the ".ICF" file. This is normally done in degree.

The function returns a list of three floats.

Example:

(CMESH-GET (REM (RADTODEG $a) 360.0))	returns '(0.12 -0.14 0.05)

On a kinematic where the major axis is vertical it's normal that this can rotate more than from 0 to 360 degree. The "ICF" file will only contains degree from 0 to 360 degree. Its therefor necessary to reduce the input to only be between 0 and 360 degrees.

Cutting speed on tube cutting

The cutting speed on tube cutting must be handled in a different way compared with normal shape cutting in X and Y. The reason is that the tube rotation is usually programmed as a rotation axis in degree per minute and the other movement is programmed in distance/minute.

When you cut the A geometry there is no rotation of the tube, in this case is possible to use distance/minute programming. When cutting B the only movement is the tube rotation and in this case it’s possible to use speed in degree/ minute. The C geometry is a mix of A and B and in this case it’s not possible to use either speed by distance/minute or degree/minute.

Invers time feed rate definition

The feed rate should be programmed in invers time and it’s solves the problem when the numerical control must mix different methods. The speed is entered as inverse time for the CNC-controller to finish the block. Since the value can differ al lot between each block it’s important to enter the value on all block that are programmed with invers feed rate definition.

FDM=Feed in mm or inches per minute.

D=Path length in mm or inches per minute.

F=FDM/D

Example:

N200 G93 G1 X100 F2

The time to traverse along the programmed path is 0.5 min. Wanted feed rate is 200mm/min.

200/100=2

The variable $ft can be used in all TUBE functions. The value stored in the variable is the time it should take for the controller to finish the block. To change this value to invers time just divide 1/time

Example

(DEFUN tube-4X (/ ivt)
  (SETQ ivt (/ 1 $ft))
  (WRITE (STRCAT "G1 X" (RTS $x) " Y" (RTS $y) " Z" (RTS $z) " C" (ATS $c) " F" (RTOS ivt 4)))
)

The invers time feed rate definition method may not be supported on all CNC-controller and it may have different G-codes. In many controllers this function can be activated by the G-code G93.

Specify the tube axis as a linear axis

If your CNC-controller does not support invers time feed rate definition than you can set up the tube axis as a linear axis. In this case one lap on the tube axis is the same as moving the linear axis 360 mm. If you have another ratio this must be specified in the postprocessor. This can be done by including following line in the beginning of the postprocessor.

Example:

(SETQ *TubeLap* 100.0)

(DEFUN tube-rotate ()
  (WRITE (STRCAT "G1 C" (RTS 100.0) " F" (RTF $fe)))
)

If you do like this the IGEMS will calculate all cutting speeds and it’s possible to use the normal method (distance/minute) of handling the cutting speed.

Apps

This information describe how to develop special applications and commands for IGEMS by using the programming language ILISP as developer tools. ILISP is normal text files that can be added to the system by the function Load app

Load app

Start this command and the following dialog box is shown:

Press the "Add" button and select the LSP file to add. A small sample is in the APP folder in IGEMS program directory.

Select ….\APP\ANIMALDEMO\ANIMALDEMO.LSP

Give the application a name.

The applications can be grouped together on the menu.

Press the "Image" button and select an image in the same folder as the LSP file. Click "OK to load the first application.

This is just an example on how to load apps.

Script editor

ILISP has an inbuilt editor to makes it possible to create and edit ILISP files.

The first area is for writing the code, the second area is a console where you can type and check ILISP functions.

The console area can be minimized by clicking on the button above.

By setting the marker on a reserved ILISP word and press F1 or the Help button you can view a description of all ILISP commands.

ILISP Functions

Data types in ILISP

ILISP supports following data types:

* lists

* symbols

* strings

* real numbers

* integers

* file descriptors

* subrs (built-in functions)

ILISP provides numerous predefined functions. Each function is called by giving its name (upper or lower case) as the first element of a list, with the arguments to that function (if any).

(- <number> <number> ...)

This function subtracts the second <number> from the first and returns the difference. If more than two <number> are given, the sum of the second through last is subtracted from the first, and the final result is returned. If only one <number> is given, the result of subtracting it from zero is returned. This function may be used with reals or integers.

(- 50 40)	returns 10
(- 40 40.0 2)	returns 8.0
(- 50 40.0 2.5)	returns 7.5
(- 8)	returns -8
(- -8)	returns 8

(* <number> <number> ...)

This function returns the product of all <number>. It may be used with reals or integers.

(* 2 3)	returns 6
(* 2 3 4.0)	returns 24

(/ <number> <number> ...)

This function divides the first <number> by the second and returns the quotient. If more than two numbers are given, the first number is divided by the product of the second through last, and the final quotient is returned. This function may be used with reals or integers. If you divide an integer with an integer the result will be an integer. To have a correct result one of the argument should be a real value.

(/ 3 2)	returns 1

(/ 3 2.0)	returns 1.5

(/ 100 2)	returns 50
(/ 120 2.0)	returns 60.0
(/ 200 20 2.0)	returns 5.0

(/= <atom1> <atom2>)

This is the "not equal to" relational function. It returns T if <atom1> is not numerically equal to <atom2> and nil if the two atoms are numerically equal. The function is valid for numbers and strings.

(/= 12 12)	returns nil
(/= 22 4.0)	returns T
(/= "ABC" "ABC")	returns nil
(/= "yes" "no")	returns T
(/= "text" 12)	returns T

(+ <number> <number> ...)

This function returns the sum of all <number>. It may be used with reals or integers. If all the <number> are integers, the result will be an integer; if any of the <number> are real’s, the integers will be promoted to real’s and the result will be a real.

(+ 1 2)	return 3
(+ 1 2 3 4.5)	return 10.5
(+ 1 2 3 4.0)	return 10

(< <atom> <atom> ...)

This is the "less than" relational function. It returns T if the first <atom> is numerically less than the second and nil otherwise. If more than two atoms are given, T is returned if each atom is less than the <atom> to its right. The function is valid for numbers and strings.

(< 22 21)	returns nil
(< 10 20)	returns T
(< 1 2 3)	returns T
(< 3 1 2)	returns nil
(< "a" "b")	returns T

(<= <atom> <atom> ...)

This is the "less than or equal to" relational function. It returns T if the first <atom> is numerically less than or equal to the second, and nil otherwise. If more than two atoms are given, T is returned if each atom is less than or equal to the <atom> to its right. The function is valid for numbers and strings.

(<= 10 11)	returns T
(<= 99 50.0)	returns nil
(<= 10 10.0 11)	returns T
(<= 99 1 2 3)	returns nil
(<= "a" "A")	returns T
(<= "AB" "AC")	returns T
(<= "a" "b" "a")	returns nil

(= <atom> <atom> ...)

This is the "equal to" relational function. It returns T if all the specified atoms are numerically equal, and nil otherwise. This function is valid for numbers and strings.

(= 18.0 18)	returns T
(= 42 19)	returns nil
(= "ABC" "abc")	returns nil
(= "lisp" 14.78)	returns nil

(> <atom> <atom> ...)

This is the "greater than" relational function. It returns T if the first <atom> is numerically greater than the second and nil otherwise. If more than two atoms are given, T is returned if each atom is greater than the <atom> to its right. The function is valid for numbers and strings.

(> 19 17)	returns T
(> 1 20.0)	returns nil
(> 5 3 1)	returns T
(> "A" "B")	returns nil
(> "a" "A")	returns T

(>= <atom> <atom> ...)

This is the "greater than or equal to" relational function. It returns T if the first <atom> is numerically greater than or equal to the second and nil otherwise. If more than two atoms are given, T is returned if each atom is greater than or equal to the <atom> to its right. The function is valid for numbers and strings.

(>= 80.0 14)	returns T
(>= 10 110.0)	returns nil
(>= 11.1 10.2 4)	returns nil
(>= "B" "A")	returns T

(1- <number>)

This function returns <number> reduced by 1,<number> may be a real or an integer.

(1- -5) 	returns -6
(1- 17.5) 	returns 16.5

(1+ <number>)

This function returns <number> increased by 1 <number> may be a real or an integer.

(1+ 12)	returns 13
(1+ 99.0)	returns 100.0

(ABS <number>)

This function returns the absolute value of <number>, <number> may be a real or an integer.

(ABS 44)	returns 44
(ABS -72.2)	returns 72.2

(ACCCALC <distance> <start speed> <end speed>)

The function returns the acceleration in mm/min2

(ACCENDSPEED <distance> <start speed> <acceleration>)

The function returns the actual speed

(ACCTIME <distance> <start speed> <end speed>)

The function returns the speed in minutes it takes for making a linear acceleration between the two speeds. The speeds should be entered as mm/minute.

(ACOS <number>)

This function returns the result of the inverse cosine function as radians.

(ACOS (/ 100.0 107.7))	returns 0.38499

For more information see the Geometrical example.

(ALERT <msg>)

This function shows a message box with the text <msg>

(ALERT "Press Enter")

(ALERTHELP <msg> <bookmark>)

This function shows a message box with the text <msg>. Clicking on the msg opens the IGEMS.DOC file at the actial bookmark.

(AND <expr> ...)

This function returns nil if any of the expressions evaluate to nil, otherwise it returns T.

(AND 20.0 -1 T "A")	returns T
(AND 1 T nil)	returns nil

The function is often used together with the CASE or IF function.

(ANGLE <ptl> <pt2>)

This function returns the angle between the two points. The result is returned in radians, with angles increasing in the counterclockwise direction. Zero degree is in IGEMS X positive direction.

(SETQ pt1 (LIST 0.0 0.0))
(SETQ pt2 (LIST 100.0 50.0))
(ANGLE pt1 pt2)	returns 0.463648
(ANGLE pt2 pt1)	returns 3.60524

For more information see the Geometrical example.

(ANGDIFF <angle1> <angle2>)

The function takes two angles in radians and returns the shortest difference between the angles. The result is always a real value between –PI to +PI.

(SETQ a1 PI)
(SETQ a2 (- PI))
(ANGDIFF a1 a2)	returns 0

(SETQ a3 (* PI 7))
(ANGDIFF a3 a1)	returns 0

(ANGDIFF 0.1 -0.1)	returns -0.2
(ANGDIFF -0.1 0.1)	returns 0.2
(ANGDIFF 3.14 2)	returns -1.14

For more information see the Geometrical example.

(ANGTOS <angle> <precision> [raw])

The function takes <angle> (a real, in radians) and returns it edited into a string. The argument <precision> control the number of digits. If the optional argument [raw] is T then the function can returns also negative values. If the argument [raw] is omitted or nil then the function returns only positive angles between 0 to 360.

(ANGTOS PI 3)	returns "180.000"
(ANGTOS (- PI) 0)	returns "180"
(ANGTOS (- PI) 0 T)	returns "-180"
(ANGTOS (* PI 4) 1 nil)	returns "0.0"
(ANGTOS (* PI 4) 1 T)	returns "720.0"
(SETQ pt1 (LIST 0.0 0.0))
(SETQ pt2 (LIST 0.0 100.0))
(SETQ a (ANGLE pt1 pt2))
(ANGTOS a 3)	returns "90.000"
(ANGTOS a 1)	returns "90.0"

(APPEND <expr> ...)

This function takes any number of lists <expr> and runs them together as one list.

(APPEND (LIST a b) (LIST 1 d))	returns (A B 1 D)
(APPEND '((a) (b)) '((c d)))	returns ((A) (B) (C) (D))

(APPLY <function> <list>)

Executes the function specified by <function> with the arguments given by <list>.

(APPLY '+ '( 9 5 3)) 			returns 17
(APPLY 'STRCAT (LIST "A" "b" "c" "-123"))	returns "AbC-123"

APPLY works with both built-in functions (subrs) and user-defined functions (those created with either DEFUN or LAMBDA).

(ARRAY <list>)

This function will create an array object. The array function is evaluating much faster than when searching by NTH I normal lists.

(SETQ samplelist (LIST "A" "B" "C" 15 "YES" "NO" '(10 10 12)))
(SETQ fast (ARRAY samplelist))		returns the array object
(NTH 0 fast)			returns "A"
(NTH 3 fast)			returns 15
(NTH 6 fast)			returns (10 10 12)
(SETQ result (NTH 4 fast))			sets result and returns "YES"

(ASCII <string>)

This function returns the conversion of the first character of <string> into its Unicode character code (an integer).

(ASCII "A")	returns 65
(ASCII "-")	returns 45
(ASCII "6")	returns 54
(ASCII "ABCDE")	returns 65

(ASIN <number>)

This function returns the result of the inverse sinus function as radians.

(ASIN (/ 40.0 107.7))	returns 0.38499

For more information see the Geometrical example.

(ASSOC <item> <associationlist>)

This function searches the association list for <item> as the key element and returns the entry. If <item> is not found as a key in associationlist then the function returns nil.

(SETQ alist (LIST (LIST "JAN" 31) (LIST "FEB" 28) (LIST "MAR" 31) (LIST "APR" 30)))
(ASSOC "JAN" alist)	returns ("JAN" 31)
(ASSOC "APR" alist)	returns ("APR" 30)
(ASSOC "JUN" alist)	returns nil

(ATAN <num1> [<num2>])

If <num2> is not supplied, ATAN returns the arctangent of <num1> in radians. <Num1> can be negative. The range of angles returned is from -PI to +PI radians.

(ATAN 0.5)	returns 0.463648
(ATAN -1.0)	returns -0.785398
(ATAN 1.0)	returns 0.785398

If both <num1> and <num2> are supplied, the arctangent of <num1/<num2> is returned, in radians. If <num2> is zero, an angle of plus or minus 1.570796 radians (+90° or -90°) is returned, depending on the sign of <num1>

(ATAN 2.0 3.0)	returns 0.588003
(ATAN 2.0 -3.0) 	returns 2.55359
(ANGTOS (ATAN 2.0 -3.0) 4) returns "146.3099"
(ATAN 1.0 0.0) 	returns 1.5708

For more information see the Geometrical example.

(ATOF <string>)

This function converts a string to a real value.

(ATOF "-44")	returns -44.0
(ATOF "0")	returns 0.0
(ATOF "TEXT")	returns 0.0
(ATOF "123.5TEXT")	returns 123.5

(ATOI <string>)

This function converts a string to an integer.

(ATOI "9.5")	returns 9
(ATOI "-44")	returns -44
(ATOI "TEXT")	returns 0

(ATOM <item>)

This function returns nil if <item> is a list and T otherwise. Anything that's not a list is considered an atom.

(ATOM nil)	returns T
(ATOM T)	returns T
(ATOM "TEXT")	returns T
(ATOM (LIST 1 2))	return nil
(ATOM '+)	return T

(ATOMS-FAMILY <format> [<search-list>])

This function returns a list with all symbol names. If the <format> value is 0 then the list is a list of symbol names. If the <format> value is 1 then the symbol name are converted to strings. The <search-list> should be a list of strings that specify the symbol names you want atoms-family to search for.

(ATOMS-FAMILY 0)			returns (+ - * / = APPEND ASCII ...)
(ATOMS-FAMILY 1)			returns ("+" "-" "*" "/" ...)
(ATOMS-FAMILY 1 (LIST "+" "NOTEXIST"))) 	returns ("+" "nil")

(BEEP [condition [sound]])

This function plays one of windows standard sound. The condition must be a non nil value if the sound should be used.

The function returns the condition value. If no condition value is used then the function return T.

If no sound information is given then the function use sound number 2.

The sound value is an integer with following meaning.

| Sound |  |
| --- | --- |
| 0 | Exclamation |
| 1 | Asterisk |
| 2 | Beep (default) |
| 3 | Hand |
| 4 | Question |

Note! The sound may differ from the information above since the sound can be changed in the control panel.

(BEEP)	returns T and sound 2
(BEEP nil)	returns nil
(BEEP (IF (> 2 1) "Yes" nil))	returns "Yes" and sound 2
(BEEP 1 3)	returns 1 and sound 3
(BEEP T 99)	returns T and sound 2

The function can for example be used to indicate input error in dialogs.

(BOUNDP <atom>)

The function returns T if <atom> has a value bound to it. If no value is bound to <atom> it returns nil.

(SETQ a 1 b nil)
(BOUNDP 1)	returns nil
(BOUNDP 'a)	returns T
(BOUNDP 'b)	returns nil

(CADDR <list>)

The function returns the third object in the list

(CADDR '(101 102 103))			returns 103
(CADDR '((100 101 102) (a b c)) (d)))	returns (d)

For more information see the list section.

(CADR <list>)

The function returns the second object in the list

(CADR '(101 102 103 104))		returns 102
(CADR '((100 101 102) (a b c)))	returns (a b c)

For more information see the list section.

(CAR <list>)

The function returns the first object in the list.

(CAR '(100 101 102))	returns 100
(CAR '((1 2) 100 200))	returns (1 2)
(CAR '())	returns nil

For more information see the list section.

(CDR <list>)

This function returns the list without the first object in the list.

(CDR '(100 101 102))	returns (101 102)
(CDR '((100 101) a b))	returns (a b)
(CDR '())	returns nil

When the list argument is a dotted pair, CDR returns the second element without the list.

(CDR '(1 . 100))	returns 100
(CDR '(2 . "INFO"))	returns "INFO"

For more information see the list section.

(CEIL <number>)

The function rounds up the value to closest value. See also Fix

(CEIL 1.1)	returns 2.0
(CEIL -18.9)	returns -18.0

(CHR <number>)

This function returns a character that represents that Unicode value. The <number> can be any integer value from 0 to 65535.

(CHR 65)	returns "A"
(CHR 66)	returns "B"
(CHR 333)	returns "ō"

(CIRCLEINTERSECT <center> <radius> <center> <radius>)

The function takes the center and the radius on two circles and returns a list with two intersection points.

If no intersections are found the function returns nil.

(CIRCLEINTERSECT (LIST 100 100) 100 (LIST 200 100) 100)
	returns ((150 13.39)(150.18 186.6))

(CIRCLE3D3P <p1> <p2> <p3>)

This function takes 3 3D points in space and returns a list of two lists. The first list is the 3D-center (cp) point and the second is a 3D-direction vector describing the normal from the circle plan (vec).

(CIRCLE3D3P '(0 0 0) '(10 20 30) '(2 34 32))	returns
((-6.97219 22.49272 10.66225) (-0.69148 -0.47312 0.54591))

Since there are two direction vectors that are correct you may to negate values if it should be use to describing the axis vectors in 5-axis kinematic.

(CIRCLEPOINTS <vector> <radius> <number>)

This function needs a <vector> a <radius> and a <number> that describe number of points that will be located on the theoretical circle.

Example:
(SETQ vector (LIST 0.7 0 0.3)
(SETQ radius 10.2)
(SETQ pts 4)
(CIRCLEPOINTS vector radius pts)	returns ((-4.01798 0.0 9.37528) (0.0 -10.2 0.0) (4.01798 0.0 -9.37528) (0.0 10.2 0.0))

(CLIP-GET)

This function returns a string with the contents of the clip board. With this function you can for example import information from excel.

(CLIP-SET <text>)

The function copy the <text> to the windows clipboard.

(CLOSE <file>)

This function closes a file and returns nil. <file> is a file descriptor obtained from the OPEN function. After a CLOSE, the file descriptor is unchanged, but is no longer valid.

(SETQ wfile (OPEN "c:/test/test.txt" "w")
(WRITE-LINE "First line" wfile)
(WRITE-LINE "Second line" wfile)
(CLOSE wfile)	returns nil

(CLS)

Clear the postprocessor console from all text.

(COMMAND <command-name> [visible])

This function will start any built-in command in IGEMS. If the optional argument [visible] is not nil the command name will not be written on the console.

(COMMAND "CAD.Circle" nil)

(COND (<test1> <execute1>) (<test2> <execute2) ...)

This function accepts any number of arguments. It evaluate the first item in each list until one of then returns a value other than non nil. It then evaluate those expressions which follow the tests that succeeded, and returns the value of the last expression.

(SETQ nr 1)
(COND
  ((= nr 0) (PRINC "Number is zero"))
  ((= nr 1) (PRINC "Number is one"))
  (T        (PRINC "Number is not zero or one"))
)

(CONS <new> <list>)

This function adds a new element in the beginning of the list and returns the complete new list.

(CONS 10 '(20 30 40))	returns (10 20 30 40)
(CONS a nil)	returns (a)
(CONS '(a b) '(10 20 30))	returns ((a b) 10 20 30)

CONS also accept an atom in place of the <list> argument, constructing a structure known as a dotted pair. When displaying a dotted pair, ILISP prints a period, or dot, between its first and second elements. The CDR function can be used to return the second atom of a dotted pair.

(CONS 'B 22)	returns (B . 22.0)
(CAR (CONS 'B 22) 	returns B
(CDR (CONS 'B 22)	returns 22

A dotted pair is a special kind of list, and is not accepted as an argument by some functions that handle ordinary lists.

(COS <angle>)

This function returns the cosine of <angle>, where <angle> is expressed in radians.

(COS 0.0)	returns 1.0
(COS PI)	returns -1.0
(COS (* PI 0.5))	returns 0.0

For more information see the Geometrical example.

(DATE)

The function returns a list with year month day.

(DATE)	returns (2013 11 16)

See also the time function

(DATETOSTR <"date">)

Returns a string with "YEARMONTHDAY"

Example:

(DATETOSTR "2020-10-16 12:59:15")	returns "20201016"

(DEFUN <name> <argument list> <expressions> ...)

DEFUN defines a function with the name <name> (note that the function name is automatically Quoted and must not be explicitly Quoted). Following the function name is a list of arguments (possibly void), optionally followed by a slash and the names of one or more local symbols for the function. The slash must be separated from the first local symbol and from the last argument, if any, by at least one space. If no arguments or local symbols are to be declared, an empty set of parentheses must be supplied after the function name.

(DEFUN sample (a b)...) 		this function takes two arguments 
(DEFUN sample () 		this function have no argument 
(DEFUN sample (/ local1 local2)	this function have two local variables.
(DEFUN sample (a / local) 		this have one argument and one local variable

Following the list of arguments and local symbols are one or more expressions to be evaluated when the function is executed.

The DEFUN function itself returns the name of the function being defined. The local symbols may be used within the function without changing their bindings at outer levels. The function will return the result of the last expression evaluated. All previous expressions in the function have only side effects. The DEFUN function itself returns the name of the function defined.

Never use the name of a built-in function or symbol as <name> since this will make the built-in functions inaccessible.

(DEGTORAD <degree>)

This function converts the <degree> angle to the same value as radians. See also the function RADTODEG

(DISTANCE <pt1> <pt2> [force2D])

This function returns the distance between the points <pt1> and <pt2>.  The two points can have any number of dimensions. If the optional argument [force2D] is T then the function returns a 2D distance even if the point arguments are in 3D.

(SETQ p1 (LIST 0.0 0.0))
(SETQ p2 (LIST 100.0 100.0))
(DISTANCE p1 p2)	returns 141.421
(DISTANCE p1 p1)	returns 0.0
(DISTANCE (LIST 100.0 0) p1) 	returns 100.0
(SETQ p3 (LIST 0 1 2 3 4))
(SETQ p4 (LIST 5 6 7 8 9))
(DISTANCE p3 p4)	returns 11.18034

For more information see the Geometrical example.

(DISTOF <expr> [min [max]])

This function takes a string or a numerical value as expr.

If the expr is a string then the function works as follows:

If the string can be converted to a numerical value and the value is between min and max then the function returns that numerical value else it returns nil

If the expr is a numerical value and the value is between min and max then the function returns that value else it returns nil.

If argument min is nil than the function does not check for a min value.

If the argument max is nil than the function does not check or a max value.

(DISTOF nil)	returns nil
(DISTOF T)	returns nil
(DISTOF "120.2")	returns 120.2
(DISTOF "120.2" 100 200)	returns 120.2
(DISTOF "120.2" 121 200)	returns nil
(DISTOF "120.2" 110 120)	returns nil
(DISTOF 12)	returns 12
(DISTOF 12 -20 20)	returns 12
(DISTOF -22 -20 20)	returns nil
(DISTOF (LIST a b))	returns nil
(DISTOF "120.2" nil nil)	returns 120.2
(DISTOF "120.2" 130 nil)	returns nil
(DISTOF "120.2" nil 120)	returns 120.2
(DISTOF "120.2" 120 121) 	returns 120.2

The distof function is often used when programming dialog boxes.

(DOSTRING <str> <on error>)

The function is similar to the load function but takes a string as input instead of a file. If something goes wrong then the function <on error> is called.

(ENVGET <key>)

This function can be used to read operation system environment variables.

Example:

(ENVGET "WINDIR")	returns "C:/WINDOWS"

(ENVSET <key> <value>)

This function can create or modify operating system variables

Example:

(ENVSET "PATH" (STRCAT (ENVGET "PATH") ";C:\\MYTEMP")

(EQ <expr1> <expr2>)

This functions determines whether <expr1> and <expr2> are identical; that is, they are actually bound to the same object (by SETQ, for example). EQ returns T if the two expressions are identical, and nil otherwise. It is typically used to determine whether two lists are actually the same.

(SETQ l1 '(a b c))
(SETQ l2 '(a b c))
(SETQ l3 l2)
(EQ l1 l3)	returns nil (l1 and l3 are not the same list)
(EQ l2 l3)	returns T (l2 and l3 are exactly the same list)

See also the EQUAL function

(EQUAL <expr1> <expr2> [<fuzz>])

This function determines whether <expr1> and <expr2> are equal, that they should evaluate to the same thing.

(SETQ l1 '(a b c))
(SETQ l2 '(a b c))
(SETQ l3 l2)
(EQUAL l1 l3)	returns T (l1 and l3 are evaluated to the same thing)
(EQUAL l2 l3)	returns T (l2 and l3 are exactly the same list)

Whereas two lists that are EQUAL may not be EQ, atoms that are EQUAL are always EQ as well. In addition, any two lists or atoms that are EQ are always EQUAL.

When comparing two real numbers (or two lists of real numbers, as in points), it is important to realize that two "identical" numbers may differ slightly if different methods were used to calculate them. Therefore, an optional numeric argument, <fuzz>, lets you specify the maximum amount by which <expr1> and <expr2> can differ and still be considered EQUAL.

(SETQ a 1.123456)
(SETQ b 1.123459)
(EQUAL a b)	returns nil
(EQUAL a b 0.01)	returns T
(SETQ p1 (LIST 0.1 100.1))
(SETQ p2 (LIST 0.2 99.9))
(EQUAL p1 p2)	returns nil
(EQUAL p1 p2 0.5)	returns T
(EQUAL nil 91 0.1)	returns nil

Functions to read or write operating system environment variables.
(ENVGET key)
(ENVSET key value)

Example:
(envset "PATH" (strcat (envget "PATH") ";c:\\temp\\"))

(EVAL <expr>)

Returns the result of an evaluation.

(SETQ a -2.0)
(EVAL 99.9)	returns 99.9
(EVAL a)	returns -2.0
(EVAL (ABS a))	returns 2.0

(SETQ var7 199.98)
(SETQ index 7)
(PRINC (EVAL (READ (STRCAT "VAR" (ITOA index)))	returns 199.98

See also the SET function

(EXP <number>)

This function returns e raised to the <number> power (natural antilog). It returns a real.

(EXP 1. 0) 	returns 2.71828
(EXP 2.2) 	returns 9.02501
(EXP -0.4) 	returns 0.67032

(EXPT <base> <power>)

This function returns <base> raised to the specified <power>. If both arguments are integers, the result is an integer. Otherwise, the result is a real.

(EXPT 10 2)	returns 100
(EXPT 10 3)	returns 1000 and is the same as (* 10 10 10)

For more information see the Geometrical example.

(FILE-COPY <source> <target> <overwrite>)

The function copy a file from source to target.

(FILE-DELETE <file name>)

The function deletes the file if it exists. You can't delete more than one file at a time. It's not possible to use "*".

(FILE-DELETE "c:/ncdata/100.cnc")	returns T if the file exists and it was deleted, else the function returns nil

(FILE-DIRDIALOG [initial folder])

The function will show windows select folder dialog and return a string with selected folder. If use press Cancel then the function returns nil.

(FILE-DIRONLY <file name>)

The function returns the path part from the filename. The last character is always a "/" or "\\"

Example:
(FILE-DIRONLY "C:/NCDATA/MACHINE2/CNC/1008.CNC")	returns "C:/NCDATA/MACHINE2/CNC/"

(FILE-EXISTS <file name>)

(DIR-EXISTS <path> [timeout = 3000])

timeout is an optional argument to set the maximum time, default value is 3000ms.

(DIR-MOVE <source path > <target path>)

Moves the directory with path 'source' to the path 'target'.
Also works to move and rename files.

Returns T on success, otherwise nil.
Examples:
(DIR-MOVE "c:/MyDirectory" "c:/MyRenamedDirectory")
(DIR-MOVE "c:/temp/1.dxf" "c:/temp/1-renamed.dxf")
(DIR-MOVE "c:/ABC" "c:/temp/ABC-MOVED")

(FILE-FINDALL <path> <mask> [<subfolders>])

Create a list of all files specified. If the optional argument <subfolder> is T then all files in the subfolders is also collected to the list.

(FILE-FINDALLDIR <path> <mask>)

The function returns a list with all folders located in the specified path.

(FILE-MAKEDIR <path>)

Create a directory or multiple directories. The function returns the <path> if success or nil if the function did not success.

(FILE-NAMEONLY <file path> [<remove ext>])

This function returns the file name part from a complete long filename including drive path and extension. In ILISP you can use forward slash instead of backwards slash. The optional argument <remove ext> can be included as a non nil value if you want to remove the extension.

Example:
(FILE-NAMEONLY "C:/NCDATA/MACHINE2/CNC/1008.CNC")	returns "1008.CNC"
(FILE-NAMEONLY "C:/NCDATA/MACHINE2/CNC/1008.CNC" T)	returns "1008"
(FILE-NAMEONLY "C:/NCDATA/MACHINE2/CNC/1008.CNC" nil)	returns "1008.CNC"
(SETQ a 100)
(FILE-NAMEONLY "C:\\NCDATA\\MACHINE2\\CNC\\1008.CNC" (= a 100))	returns "1008"

(FILE-OPENDIALOG <initdir> <filter> [multiselect])

This function shows a window open dialog with preview area. The function GETFILED is a similar function that may be removed in the future. 
<initdir> is the initial directory for the open dialog. If you want windows to decide then leave this argument as an empty string "".

<filter> Must be a string. The first part is what should be shown as information, the second part is the filter.

[multiselect] this optional argument is default nil. If it T then the function returns a list with the selected files.

Example:

(FILE-OPENDIALOG "" "All important files|*.NC;*.CNC")

The example above will ask for a file with extension ".NC" or ".CNC"

(FILE-READLINES <file name>)

Will open the file and read all strings to a list. The list will be returned.

If the files does not exists then the function returns nil

(FILE-SAVEDIALOG <initdir> <ext> <no warning>)

The function shows a window save as dialog. The function GETFILED is a similar function that may be removed in the future. 
<initdir> is the initial directory for the dialog.

If you want windows to decide then leave this argument as an empty string "".

If you include a filename, then this filename will be default.

If you include

<ext> is the default extension of the file to save.

If the optional argument

(FILE-SAVEDIALOG "c:/ncdata/" "txt")	returns nil or filename
(FILE-SAVEDIALOG "test.txt" "txt")	returns nil or the filename
(FILE-SAVEDIALOG "c:/ncdata/test.txt" "txt")	returns nil or filename
(FILE-SAVEDIALOG "c:\\ncdata\\test.txt" "txt")	returns nil or filename

(FILE-SIZE <file name>)

The function returns the files size as an integer. If file not exists or if the function cannot return the size then it will return nil.

(FILE-TIME <file name> [mode])

The function returns the time for selected file. A file can have several times. Use the [mode] argument to select wanted time.

| Mode | Meaning |
| --- | --- |
| Omitted or 0 or nil | Time for last change of file |
| 1 or T | Time when the file was created |
| 2 | Latest access time |
| 3 | If it is a picture. Then the time when the picture is taken. If this time not exist on the file then the function returns nil. |

If the file does not exist then the function returns nil.

(FILE-TIME "c:/ncdata/104.pgm" 1))	"2017-03-08 09:50"

(FILE-VALIDFILENAME <name>)

The function takes a string as argument and check if that is a valid filename. The name should be without drive, path and extension. Just the name itself. The function returns the name if OK and nil if the file name is bot valid.

(FIX <number>)

This function returns the conversion of <number> into an integer. <number> may be either an integer or a real. If real, it is truncated to the nearest integer by discarding the fractional portion.

(FIX 22)	returns 22
(FIX 11.9)	returns 11
(FIX -11.9)	returns -11

(FLOAT <number>)

This function returns the conversion of <number> into a real, <number> may be either an integer or a real.

(FLOAT 12)	returns 12.0
(FLOAT 19.8)	returns 19.8

(FLOOR <number>)

The function rounds down the value to closest value. See also Fix

(FLOOR 1.1)	returns 1.0
(FLOOR -18.9)	returns -19.0

(FOREACH <var> <list> <expr>...)

This function steps through <list> assigning each element to <var>, and evaluates each <expr> for every element in the list (or string see below). Any number of <expr> can be specified. FOREACH returns the result of the last <expr> evaluated.

(FOREACH s (LIST 10 " "20 " " 30) (PRINC s))

The result of this FOREACH should be 10 20 30. The functions return 30

ILISP also support a string instead of a list.

(FOREACH s "ILISP" (PRINC s))

The result of this FOREACH should be "I" "L" "I" "S" "P". This variant is powerful to use when parsing textstrings.

(FUNTOSTR <function>)

The function takes a defun as argument and returns the function as text.

Example:

(DEFUN myfunc ()
	(SETQ a 10 b 20)
)
(FUNTOSTR myfunc)	returns "(DEFUN myfunc () (SETQ a 10 b 20))"

(GC)

This command makes global garbage collect in the whole IGEMS.

(GCD <num1> <num2>)

This function returns the greatest common denominator of <num1> and <num2>. Both arguments must be non-zero integers. The result is the largest positive integer that divides the numbers without a remainder.

(GCD 12 4)	returns 4

(GCD 4 8)	returns 4

(GCD 8 3)	returns 1

(GCD 25 5)	returns 5

(GETDIR <default> [<flag>])

This function open a dialog where the user can select a directory.

| Flag | Meaning |
| --- | --- |
| 1 | If the flag is 1 then the user can create a new directory |

The function return nil if the user close the dialog or the selected directory.

(GETDIR "")	returns the selected directory for example "C:/NCDATA/"

(GETFILED <Prompt> <default> <extension> <flag>)

Note! This function may be removed in the future. 
Please use the function FILE-OPENDIALOG or FILE-SAVEDIALOG instead of GETFILED

The function opens a file dialog and asks for a file name. 
The <prompt> argument must be a string and it will be displayed as the label of the file dialog.

The <default> argument can be a complete path file name and extension. If the argument is a filename only then current windows path is used.

The <extension> argument should be without dot, and if it's an empty string then it's means the same as (*).

The <flag> argument is a bit coded integer value with following meaning:

| Flag | Meaning |
| --- | --- |
| 1 | Create a file new file. If this bit is not set then the function ask for an existing file. |
| 2 |  |
| 4 | Let the user enter an arbitrary file name extension, or no extension at all * |
| 8 | If this bit is set and bit 0 is not set, the function performs a library search for the file name entered. If it finds the file and its directory in the library search path, it strips the path and returns only the file name. (It does not strip the path name if it finds that a file of the same name is in a different directory.) * |
| 16 | If this bit is not set, the function returns the entire file name, including the path name. If it’s set it will only return the file name and extension. * |
| 32 | If this bit is set and bit 0 is set (indicating that a new file is being specified), users will not be warned if they are about to overwrite an existing file. The alert box to warn users that a file of the same name already exists will not be displayed; the old file will just be replaced. * |
| 64 | Do not transfer the remote file if the user specifies a URL. * |
| 128 | Do not allow URLs at all. * |

*Not implemented

(GETG <variable>)

Every time you start a new ILIPS application all variables and function are reset to nil.

This function pick up a variable earlier set by the SETG function from the global memory area.

(SETQ myvar 100)

Open a new drawing

(SETQ myvar(GETG myvar)) The variable myvar is now set in the active ILISP application

Note! See also the SETG function

Functions in this box may be replaced by other functions names like

CAD-GETANGLE, CAD-GETDIST, CAD-GETPOINT, CAD-GETCORNER and so on...

(GETANGLE <prompt> [basepoint])

(GETDIST <prompt> [basepoint])

(GETINT <prompt> <default> [min] [max])

(GETPOINT [base point] <prompt> <object>)

If the base point argument is used, then you can use the object argument. This will drag the object on the screen when you pick the point.

(GETREAL <Prompt> <default> <min><max>)

This function shows a dialog that ask for a numerical value.

(GETSTRING <Prompt> <default>)

(GETVAR <name>)

This function returns the system variable from IGEMS environment. The <name> must be a valid variable string. Following variables are available:

| Valid strings | Meaning |
| --- | --- |
| "DIGFILE" | Returns the current drawing file name. If the name refers to an existing file, then the complete path and extension are returned. |
| "FILEIMPORT" | This function will return the filename of the latest imported file using Copy/Paste, Drag/Drop or the Import command. (Only filename not path) |
| "IGEMSCNC" | Returns the complete path and program specified in the IGENS CNC field in the General tab on the Preferences dialog |
| "LANGUAGE" | returns the ISO639-1 two letter code for the language. Example: "se" for English or "se" for Swedish. |
| "OSMODE" | Returns an Integer for the actual settings of the different osnap modes. None = 0, Point = 1, EndPoint = 2, MidPoint = 4, Center = 8, Intersection = 16, Nearest = 32, Perpendicular = 64, Quadrant = 128, Tangent = 256,Grip = 512, Centroid = 1024. |
| "OSNAP" | Returns T if osnap mode is activated else nil. |
| "ORTHO" | Returns T is Ortho is activated else nil |
| "ORTHOANGLE" | Returns the angle step for the Ortho. The value can be between 1 and 359 degrees. |
| "PATHDATA" | Returns the IGEMS data folder. |
| "PATHIMPORT" | Returns the path to the latest imported file using Copy/Paste, Drag/Drop or the Import command. |
| "PATHSHARED" | Returns the shared folder location. |
| "PATHBIN" | Returns IGEMS bin folder. |
| "PATHROOT" | Returns IGEMS installation folder. |
| "PATHSCRIPT" | Returns the location of the latest loaded LISP file |
| "PATHTEMP" | Returns windows temp directory |
| "VIEWAREA" | Returns a list of the currently visible rectangle in the current drawing (x1 y1 x2 y2) |
| "UNITS" | returns actual dimension units 0 for metric and 1 for imperial |
| "USERNAME" | Returns the current windows user. |

Example:
(GETVAR "PATHDATA")	returns "C:/ProgramData/IGEMS Software/IGEMS/10.0.0.0/"

Note! See also the (SETVAR <string> <value>) command.

(GUID)

The function returns a unique windows string. The string can be used as a random ID.

(GUID)		can returns "eb21bb57-9a51-40f4-b0e3-a99d004eaae5"

(GUID)		can returns "a41cd4d2-2423-473d-bf7b-edd49dba2ad9"

(IF <testexpr> <thenexpr> [<elseexpr>])

This function conditionally evaluates expressions. If <testexpr> is not nil, then it evaluates <thenexpr> otherwise it evaluates <elseexpr>. The last expression <elseexpr> is optional. IF returns the value of the selected expression. If <elseexpr> is missing and <thenexpr> is nil then IF returns nil.

(IF (= T nil) "Same" "Not same")	returns "Not same"
(IF (= (+ 10 20) 30) "Same" "Not same")	returns "Same"
(IF (< 22 12) "OK")			returns nil

(INIFILE-READ <filename> <section> <key>)

This function will return the value as a string from the key in the section in the INI file. The function will return nil if the function fails to find the information.

All arguments <filename> <section> and <key> must be strings and the two last arguments are case sensitive.

This function can be used to read information from the kinematic INI file. The variable *HeadDefIniFile* has name and location for this file.

Example:

(SETQ value (ATOF (INIFILE-READ *HeadDefIniFile* "global" "ProbePark")))

The function will open the file look for the section and return the key value in selected file

[global]

ProbePark=396.15;

See also UC-GET and UC-SET functions.

(INTERS <line1start> <line1end> <line2start> <line2end> [infinity])

The function returns the intersection point between line1 and line2. If the optional argument [infinity] is nil or omitted then the intersection point must be on both of the lines else the function will returns nil. If the [infinity] argument is T then the function will always return the intersection point. (Not if the lines are parallel).

(INTERS p1 p2 p3 p4 T)	returns a point (z y)

(ITOA <number>)

The function returns the conversion of an integer or a real value to a string. If the value is a real it will be rounded by standard rules. Integers in ILISP is 32 bits and can handle values from -2146483648 till +2146483647. The ITOA function will convert the number to a 64 bits integer and then convert the value to a string without decimal.

(ITOA 99)	returns "99"
(ITOA 2146483650.0)	returns "2146483650"
(ITOA 0)	returns "0"
(ITOA -6622)	returns "-6622"
(ITOA 6.5)	returns "7"
(ITOA 6.4)	returns "6"

(KDTREE-CREATE <list of points>)

The function create a kdtree object from a list of points. The points has to have Y and Y first but can contain any data after this. The function return the kdtree object.

(SETQ p1 (LIST 10 10 "A") p2 (LIST 100 100 "B") p3 (LIST 110 110 "C"))
(SETQ kd (KDTREE-CREATE (LIST p1 p2 p3)))	;returns the object

(KDTREE-FINDCLOSEST <kd> <point> <size> [number of points])

Scans the kd tree for the closest point in the rectangle centered at the <point> with half-width size. Returns the list as specified in KDTREE-CREATE, or nil if no points are in the specified rectangle.

(SETQ point (LIST 100 100))
(SETQ ptlist (KDTREE-FINDCLOSEST kd point 20 2))	;returns ((100 100 "B") (110 110 "C"))

(KDTREE-INSERT <kd> <point>)

Insert a new point in an existing KDTREE. The <kd> is the object of an existing KDTREE. The function returns the <kd> object.

(SETQ new (LIST 123.456 -118 "The new point"))
(SETQ kd (KDTREE-INSERT kd new))		returns the object

(LAMBDA <arguments> <expr> ...)

LAMBDA defines an "anonymous" function. This is typically used when the overhead of defining a new function is not justified. It also makes the programmer's intention more apparent by laying out the function at the spot where it is to be used. LAMBDA returns the value of its last <expr>, and is often used in conjunction with APPLY and/or MAPCAR to perform a function on a list.

(APPLY '(LAMBDA (x y z) (* x (- y z))) (LIST 5 20 14))	returns 30
(SETQ q 0)
(MAPCAR '(LAMBDA (x) (SETQ q (1+ q)) (* x 5)) (LIST 2 4 -6 10.2))	returns 
(10 20 -30 51.0)

(LAST <list>)

This function returns the last element in the <list>.

(LAST '(a b c d e f))	returns f
(LAST '(a b c (d e))	returns (d e)
(LAST nil)	returns nil

(LENGTH <list>)

This function returns an integer indicating the number of elements in <list>.

(LENGTH '(a b 12 17))	returns 4
(LENGTH (LIST a b))	returns 2
(LENGTH nil)	returns 0

(LIST <expr> ...)

This function takes any number of expressions <expr> and put them together, returning a list.

(SETQ a 100 b 200)
(LIST 'a 'b 12 9)	returns (A B 12 9)
(LIST a b 12 9)	returns (100 200 12 9)

(LISTP <item>)

This function returns T if <items> is a list, and nil otherwise.

(LISTP '(a b c))	returns T
(LISTP 'a)	returns nil
(LISTP nil)	returns T (nil is an empty list)

(LOAD <filename> [<onfailure>])

This function loads a file of LISP expressions and evaluates those expressions. <filename> is a string that represents the filename without extension (an extension of ".lsp" is assumed). <filename> may include a directory prefix, as in "/function/test".)

If the operation is successful, LOAD returns the value of the last expression in the file. If the LOAD operation fails, it normally causes an LISP error. However, if the <onfailure> argument is supplied, LOAD returns the value of this argument upon failure instead of giving an error. This allows an LISP application calling LOAD to take alternative action upon failure.

Assume that the file "TEST.LSP" does not exist, then

(LOAD "C:/IGEMS_R10/LSP/TEST.LSP") 		causes a LISP error
(LOAD "C:/IGEMS_R10/LSP/TEST.LSP" "No such file")	returns "No such file"

If the same file exist.

(LOAD "C:/IGEMS_R10/LSP/TEST.LSP")		returns last evaluation

The LOAD function can be used from within another LISP function, or even recursively (in the file being loaded).

Note:  The function GETVAR is useful to get the address of different IGEMS folders.

(LOG <number>)

This function returns the natural log of <number> as a real.

(LOG 4.5)	returns 1.50408
(LOG 1.22)	returns 0.198851

(LOGAND <number> <number> ...)

This function returns the result of a logical bitwise AND of a list of <number>. The <numbers> must be integers and the result is also an integer.

(LOGAND 4 6)	returns 4
(LOGAND 4 1)	returns 0
(LOGAND 2 3 15)	returns 2
(LOGAND 8 3 4)	returns 0

(LOGAND 7 15 3)	returns 3

| 1 (bit1) | 2 (bit 2) | 4 (bit 3) | 8 (bit 4) | 16 (bit 5) | 32 (bit 6) |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 4 |  |  |  | = 7 |
| 1 | 2 | 4 | 8 |  |  | = 15 |
| 1 | 2 |  |  |  |  | = 3 |

Value 3 (bit 1 and 2) is common for all numbers in the list.

(LOGIOR <number> <number> ...)

This function returns the result of a logical bitwise inclusive OR of a list of <numbers>. The <numbers> must be integers, and the result is also an integer.

(LOGIOR 1 2 4)	returns 3
(LOGIOR 9 3)	returns 11

(LSH <num> <numbits>)

This function returns the logical bitwise shift of <num> by <numbits>. <num> and <numbits> must be integers, and the result is also an integer. If <numbits> is positive, <num> is shifted to the left, if negative, to the right. In either case, "zero" bits are shifted in, and the bits shifted out are discarded. If a "one" bit is shifted into or out of the top bit of an integer, its sign changes.

(LSH 2 1)	returns 4
(LSH 2 -1)	returns 1
(LSH 9 1)	returns 18
(LSH 9 -1)	returns 4

(MAPCAR <function> <list1> ... <listn>)

MAPCAR returns the result of executing <function> with the individual elements of <list1> through <listn> supplied as arguments to <function>. The number of <list>s must match the number of arguments required by <function>. The MAPCAR always return a list of the result.

(SETQ a 10 b 20 c 30)
(MAPCAR '1+ (LIST a b c))	returns (11 21 31)

(MAX <number> <number> ...)

This function returns the largest of the <number>s given. Each <number> may be a real or an integer.

(MAX 20 -12 77.1)	returns 77.18
(MAX -12 -10 -3)	returns -3

(MEMBER <expr> <list>)

This function searches <list> for an occurrence of <expr> and returns the remainder of <list> starting with the first occurrence of <expr>. If there is no occurrence of <exp> in <list>, MEMBER returns nil.

(MEMBER 'c '(a b c d e f))		returns (C D E F)
(MEMBER 10 (LIST 9 10 11 12))	returns (10 11 12)
(MEMBER 10 (LIST 20 30 40))	returns nil

(MIN <number> <number> ...)

This function returns the smallest of the <number>s given. Each <number> may be a real or an integer.

(MIN 100 200 300)	returns 100
(MIN -100 100)	returns -100

(MINUSP <item>)

This function returns T if <item> is a real or integer and evaluates to a negative value, otherwise it returns nil

(MINUSP -1)	returns T
(MINUSP 0.0)	returns nil
(MINUSP (- 10 12))	returns T

(NOT <item>)

This function returns T if the expression is nil, and nil otherwise.

(NOT T)	returns nil
(NOT nil)	returns T
(NOT 1)	returns nil

(NTH <number> <list>)

This function returns the <number> element of <list>, where <number> is the number of the element to return (zero is the first element). If <number> is greater than <list>s highest element number, nil is returned.
The NTH function works also on ARRAY objects

(NTH 0 '(a b c d))	returns A
(NTH 3 '(a b c d))	returns D
(NTH 3 '(10 20))	returns nil

(NULL <item>)

This function returns T if <item> is bound to nil, and nil otherwise. For example, given the following assignments:

(SETQ a "Text")
(SETQ b nil)
(SETQ c 100)
(NULL a)	returns nil
(NULL b)	returns T
(NULL c)

(NUMBERP <item>)

This function returns T if <item> is a real or an integer.

(NUMBERP "12")	returns nil
(NUMBERP 10)	returns T
(NUMBERP -2)	returns T
(NUMBERP nil)	returns nil

(OPEN <filename> <mode> [encoding])

This function opens a file for access by LISP's I/O functions. It returns a file descriptor to be used by the other I/O functions
<filename> is a string specifying the name and may include a directory prefix and a drive letter. You can use the backslash instead of the forward slash (but you must use double "\\" to obtain one backslash in a string.)

(SETQ fw (OPEN "c:/test/test.txt" "w"))

<mode> is the read/write flag. It must be a string containing a single lower case letter. The valid mode letters are described in the following table.
"r"	Open for reading. If <filename> does not exist, nil is returned.
"w"	Open for writing. If <filename> does not exist, a new file is created and opened. If <filename> already exists, its existing data will be overwritten.
"a"	Open for appending. If <filename> does not exist, a new file is created and opened. If <filename> already exists, it is opened and positioned at the end of the existing data, so any new data you write in the file will be appended to the existing data.
Assuming that the files named in the following examples do not exist

(SETQ fr (OPEN "new.txt" "r")) returns nil
(SETQ fw (OPEN "new.txt" "w"))	returns FILE
(SETQ fa (OPEN "new.txt" "a")) returns FILE

<encoding>
If encoding is left out or nil, the file is written in the current windows ANSI codepage. 
If encoding is T, the file is written in utf-8 encoding, which should be used if an ILISP code file is written. 
If encoding is a string, it’s the webname of the encoding used, for example "utf-8" "utf-16" "utf-32" and many more. When creating CNC-files you can left out the encoding argument.

(OR <expr> ...)

This function returns the logical OR of a list of expressions. OR evaluates the expressions from left to right, looking for a non-nil expression. If one is found, OR ceases further evaluation and returns T. If all of the expressions are nil, OR returns nil.

(OR nil nil)	returns nil
(OR T nil)	returns T
(OR "Text" 12)	returns T
(OR (= 1 1) (< 1 0))	returns T

Or is often use together with IF or COND functions.

PI

This is not a function, but rather the constant. It evaluates to approximately 3.1415926.

(PING <ip-string> <timeout>)

Checks if a remote connection is available.

<ip string> should be the address of remote computer, for example "192.168.5.110".

The <timeout> argument should be an integer for the timeout in milliseconds before failure (nil) is returned. The function returns T if the remote is responding otherwise nil.

(POINTSORT <pointlist> <method>)

This function sorts a list of points. The two first arguments in each point-list is the X and Y coordinates. The function can sort in following methods:

-1=The shortest of all methods 0-8.

0=Left to tight.

1=Right to Left.

2=Bottom to top.

3=Top to bottom.

4=Closest from first point.

5=Zig zag start Lower left.

6=ZigZag Start Lower Right.

7=ZigZag Start Upper Left.

8=ZigZag Start Upper right.

(SETQ pointlist ‘((10.0 2.0 "text1" 100) (5.0 5.9 "text2" 90))
(SETQ sorted (POINTSORT pointlist 1))	 returns ((5.0 5.9 "text2" 90) (10.0 2.0 "text1" 100))

(POLAR <point> <angle> <distance>)

This function returns a point in the <angle> and <distance> from the <point>. The <angle> is expressed in radians from the X axis, with angles increasing in the counterclockwise direction.
Note! The function always returns a 2D point even if a 3D point is given as argument.

(SETQ from (LIST 0 0))
(SETQ ang (* PI 0.5))
(SETQ dist 100)
(POLAR from ang dist)		returns (0 100)
(POLAR from (* ang 0.5) dist)	returns (70.7107 70.7107)

For more information see the Geometrical example.

(PRIN <text>)

This function print the text on the command line area.

(PRIN "Reading the file")

The function will return the text and write the text in the area you see below.

(PRINC <expr> [<file descriptor>]

This function prints the <expr> on the console or on the file. <expr> can be a string containing control characters.

\e	for escape

\n 	for newline

\r 	for return

\t 	for tab

\nnn for the character whose octal code is nnn.

(PRINC "Part1\tPart2")	will print "Part1   Part2" and return "Part1\tPart2"

(PRINT <expr> [<file descriptor>])

This function prints the <expr> on the console or on the file. <expr> can be a string containing control characters. The control characters will be printed out with a leading "\".

(PRINT "Part1\tPart2") will print "Part1\tPart2" and return "Part1\tPart2"

(PROGN <expr> ...)

This function evaluates each <expr> sequentially, and returns the value of the last expression. You can use PROGN to evaluate several expressions where only one expression is expected.

(IF (= a b)
  (PROGN
    (SETQ a (+ a 1.5))
    (SETQ b (- b 1.5))
  )
)

The IF function normally evaluates one expression. In this example, we have used PROGN to evaluate multiple expressions.

(QUIT <msg>)

This function write the <msg> on the consol and interrupt the lisp execution.

(QUIT "*Cancel*")	returns "*Cancel*"

(QUOTE <expr>)

Returns <expr> unevaluated. This can also be written 'expr.

(SETQ a 100)
(QUOTE a)	returns A

(RADTODEG <radian>)

This function convert the angle <radian> to degree. See also the function DEGTORAD

(RAND)

The function returns a random value between 0 to 1.

(RAND)	returns 0.15806
(RAND)	returns 0.7271

(READ <string>)

This function can convert a text to a lisp execution.
The function returns the first list or atom obtained from <string>. <string> must not contain blanks.

(SETQ txt "(SETQ a 100"))
(READ txt)	returns (SETQ a 100)
(EVAL (READ txt))	returns 100

(READ-CHAR [<file-desc>])

This function reads a single character from the keyboard input buffer or from the open file described by <file-desc>. It returns the (integer) ASCII code representing the character read.

If no <file-desc> is specified and there are no characters in the keyboard input buffer, READ-CHAR waits for you to type something at the keyboard (followed by RETURN). For instance, assuming that the keyboard input buffer is empty:

(SETQ tk (READ-CHAR))

will wait for something to be entered. If you type "ABC" followed by RETURN. READ-CHAR will return 65 (the unicode code for the letter "A"). The next three calls to READ-CHAR will return 66, 67. and I 0 (newline). respectively. If another READ-CHAR call is then made, it will again wait for input.

(READ-LINE [<file-desc>])

This function reads a string from the keyboard or from the open file described by <file-desc>. If the end of the file is encountered, READ-LINE returns nil, otherwise it returns the string that was read.

(SETQ fr (OPEN "c:/test/test.txt" "r")
(WHILE
  (SETQ txt (READ-LINE fr))
  (PRINC txt)
)
(CLOSE fr)

The example above will open a file, read and print out all lines, when it come to the end of the file it will close the file.

(REM <num1> <num2> ...)

This function divides <num1> by <num2> and returns the remainder <num1> mod <num2>. REM may be used with reals or integers.

(REM 19 6)	returns 1
(REM 3 2)	returns 1
(REM 2 2)	returns 0
(REM 100 2.2)	returns 1.0

(REPEAT <number> <expr> ...)

The function evaluates each <expr> <number> times and returns the value of the last expression. The <number> must be an integer.

(SETQ b 1)
(REPEAT 8
  (SETQ b (* b 2))
)
(EVAL b)	returns 256

(REVERSE <list>)

The function return the list with its elements reversed.

From version 2016.3.1060 the function can be used also on strings

(REVERSE (LIST 1 2 3 4))	returns (4 3 2 1)
(REVERSE '((a b) c d)))	returns (D C (A B))
(REVERSE nil)	returns nil (nil is an empty list)
(REVERSE "Text")	return "txeT")

(ROUND <number>)

The function round the value up or down to the closest value.

See also the functions CEIL and FLOOR.

(ROUND 0.1)	returns 0.0
(ROUND 1.49999999999)	returns 1.0
(ROUND 1.5)	returns 2.0
(ROUND -100.9)	returns -101.0

See also the functions CEIL and FLOR

(RTOS <number> <precision> [<all>])

This function converts a number to a string, the <precision> controls the number of digits. If the <all> argument is nil or not included then ending zeroes will be removed.

(RTOS 10 3)	returns "10"
(RTOS 10 3 T)	returns "10.000"
(RTOS 12.5 0)	returns "13"
(RTOS 12.4 0)	returns "12"
(RTOS 0 8 T)	returns "0.00000000"

(SET <'a> <value>)

The set function is similar to SETQ except that set evaluates both of its arguments.

(SETQ index 0)
(REPEAT 10
  (SET (READ (STRCAT "VAR" (ITOA index))) (* index 10))
)
(PRINC var5)		returns 50
(PRINC var7)		returns 70

See also the EVAL function

(SETG <variable> <value>)

Every time you start a new ILIPS application all variables and function are reset to nil.

This function saving the variable in global memory area. It is saved as long as IGEMS is not turned off. It can also be used between different drawings. The function can be used to save all kind of variables lists or functions.

(SETG myvar 100)	returns 100

Note! See also the (GETG <variable>) function

(SETQ <sym1> <expr1> [<sym2> <expr2>] ...)

This function sets the value of <sym1> to <expr1>, <sym2> to <expr2>, and so on. This is the basic assignment function in LISP. It returns the last <expr>.

(SETQ a 5.0)	returns 5.0
(SETQ b 100 c 32 d "Text")

Note! You should never use <symbol names> that are the same as the LISP functions.

(SETVAR <String> <value>)

The function can set following settings in IGEMS.

| Valid string | Value |
| --- | --- |
| "OSMODE" | Can set the different default Osnap modes. Example: (SETVAR "osmode" 7) activate Endpoint, Midpoint and Center. Following values are valid- Summary of following: None = 0, Point = 1, EndPoint = 2, MidPoint = 4, Center = 8, Intersection = 16, Nearest = 32, Perpendicular = 64, Quadrant = 128, Tangent = 256,Grip = 512, Centroid = 1024. |
| "OSNAP" | T if osnap mode should be active else nil. |
| "ORTHO" | T if Otho mode should be active else nil. |
| "ORTHOANGLE" | Sets the step angle for ortho command. Normally it's 90. |

Example:

(SETVAR "OSMODE" 1)
(SETVAR "OSNAP" T)

This will activate osnap mode endpoint.

(SIN <angle>)

This function returns the sin of <angle> as a real, where <angle> is expressed in radians.

(SIN 1.0)	returns 0.841471
(SIN 0.5)	returns 0.479426
(SIN 0)	returns 0.0

For more information see the Geometrical example.

(SLEEP <milliseconds>)

The function makes a sleep in that time given by the argument. This function can sometimes be risky to use. If you use a longer time it needs to be accompanied by the WINBREATH function.

(SORT <list> <function>)

With this function you can sort all kind of lists. The argument function should be a function that takes two arguments. The function should return 0 if the two compared values are equal.

If you want the list to be sorted in increasing order then the function return a negative value if the first value is smaller and a positive value if the first value is larger.

If you want the list to be sorted in opposite order then just swap < and >.

(SETQ unsort (LIST 2 1 4 3 6 5 8 7))

(DEFUN compare (a b)

(COND

((= a b) 0)

((< a b) -1)

((> a b) 1)

)

)

(SETQ sorted (SORT unsort COMPARE))	returns (1 2 3 4 5 6 7 8)

By modify the compare function you can sort different kind of information, sort in distance from a point, sort by dates or other criteria. Below is an example how to sort strings by the length of the text.

(SETQ unsort (LIST "AB" "A" "ABAB" "AB" "ABABAB" "ABABA"))

(DEFUN compare (a b)

(COND

((= (STRLEN a) (STRLEN b)) 0)

((< (STRLEN a) (STRLEN b)) -1)

((> (STRLEN a) (STRLEN b)) 1)

)

)

Instead of making a compare function you can use the LAMBDA function.

(SETQ unsort (LIST 2 1 4 3 6 5 8 7))

(SETQ sorted (SORT unsort (LAMBDA (a b) (- a b))))

(SQRT <number>)

This function returns the square root of <number> as a real. The <number> must be positive.

(SQRT 4)	returns 2.0
(SQRT 16)	returns 4.0
(SQRT 2)	returns 1.41421

For more information see the Geometrical example.

(STARTAPP <appcmd> [<file> <show>])

The command start another windows application. If <appcmd> does not include a full path name, startapp searches the directories in the PATH environment variable for the application. An optional string <file> specifies the file name to be opened.

If the show argument evaluates to T then the batfile will be shown in a window else not.

(STARTAPP "notepad" "c:/ncdata/test.txt")

(STRCASE <string> <lower>)

STRCASE takes the string specified by the <string> argument and returns a copy with all alphabetic characters converted to upper or lower case, depending on the second argument. <lower>. If <lower> is omitted or evaluates to nil, all alphabetic characters in <string> will be converted to upper case. If <lower> is supplied and is not nil, all alphabetic characters in <string> will be converted to lower case.

(STRCASE "SAmpLE")		returns "SAMPLE"
(STRCASE "SAmpLE" nil)		returns "SAMPLE"
(STRCASE "SAmpLE" T)		returns "sample"
(STRCASE "SAmpLE" (NUMBERP 1))	returns "sample"

(STRCAT <string1> <strinq2>...)

This function returns a string that is the summary of all <strings>.

(STRCAT "A" "B" "C" "D")	returns "ABCD"
(STRCAT "IGEMS_R" (ITOA 10))	returns "IGEMS_R10"
(STRCAT "A" "" "B")	returns "AB"

(STRFIND <find> <in string>)

The function search for the string <find> in the string <in string> and returns the number of characters before the <find> string as an integer. If the string <find> not exists, then the function returns nil.

(STRFIND "" "") 	returns 0
(STRFIND "" "any string") 	returns nil
(STRFIND " " "lisp is good")	returns 4
(STRFIND "cde" "abcdef") 	returns 2
(STRFIND "cde" "igems") 	returns nil

(STRLEN <string>)

This function returns the length, in characters, of string <string> as an integer.

(STRLEN "CAD/CAM")	returns 7
(STRLEN "")	returns 0
(STRLEN "AB")	returns 2

(STRREP <text> <repeat>)

This function returns the repeated <text> <repeat> times.

(STRREP "IGEMS" 3)		returns "IGEMSIGEMSIGEMS

(STRREPLACE <text> <replace> <with>)

This function replaces all text specified by <replace> with the text specified by the <with> argument in <text>. All argument must be strings.

(STRREPLACE "ABABIGEMSABAB" "AB" "")	returns "IGEMS"
(STRREPLACE "12,5;14,4" "," ".")	returns "12.5;14.4"

(STRS <text> [<text> …])

This function is similar to STRCAT but returns the value with double quotes.

Example:

(SETQ year 2021)
(SETQ day "Friday")
(STRS "Year:" (ITOA year) " Day:" day)	returns ""Year 2021 Day Friday""

The function is to simplify CLISP code in IGEMS CNC

(STRSUB <text> <start> [ <end>])

Note! The normal standard LISP has a function called SUBSTR this is different from this function.

The function returns a substring of the text, starting at the start position of the string and end at the end position of the text. If the end argument is not specified then the substring continues to the end of text. If the end value is smaller than the start value then the function returns an empty substring. If the start or end value are negative, the counter starts from the end of the text instead of the start of the text.

Example:
(STRSUB "ABCDEF" 3)		returns "CDEF"
(STRSUB "ABCDEF" 2 2)		returns "B"
(STRSUB "ABCDEF" 3 5)		returns "CDE"
(STRSUB "ABCDEF" -5)		returns "BCDEF"
(STRSUB "ABCDEF" -6 5)		returns "ABCDE"
(STRSUB "ABCDEF" 2 -2)		returns "BCDE"
(STRSUB "ABCDEF" -5 -1)		returns "BCDEF"
(STRSUB "ABCDEF" 10)		returns ""
(STRSUB "ABCDEF" -10)		returns "ABCDEF"

(STRSORT <list of strings>)

The function takes a list of strings as argument and returns the list sorted in raising order

(STRSORT (LIST "B" "A" "C" "E" "D"))		returns ("A" "B" "C" "D" "E")

(STRSPLIT <text> <delimiter>)

This function takes 2 strings and splits the text at delimiter

It returns a list of strings.

(STRSPLIT "example1.dxf\nexample1.dxf\n"  "\n") returns ("example1.dxf" "example2.dxf")

(STRTRIM text [<leading> [<ending>]])

This function removes leading or ending whitespaces in a string. The <leading> and <ending> argument is by default set to all whitespaces. This means ASCII 9, 10, 13 and 32. If you don't want to remove <leading> or <ending> the set the argument to ""

(STRTRIM "   IGEMS   ")			returns "IGEMS"
(SETQ default (STRCAT (CHR 9) (CHR 10) (CHR 13) (CHR 32))
(STRTRIM "   IGEMS   " default "")	returns "IGEMS   "
(STRTRIM "   IGEMS   " "" default)	returns "   IGEMS"

(SUBSCRIPTION)

This function check if the program has a valid subscription. It returns 0 if no subscription is available 1 if it is a 2D license and 3 if it is a 3D-licens.

(SUBST <new> <old> <list>)

This function searches <list> for <old>, and returns a copy of <list> with <new> replaced in every occurrence of <old>. If <old> is not found in <list>, SUBST returns <list> unchanged.

(SUBST new 20 (LIST 20 21 22))			returns (nil 21 22)
(SUBST 'new 20 (LIST 20 21 22))			returns (NEW 21 22)
(SUBST (LIST 10 20) 20 (LIST 10 20 30 40))	returns (10 (10 20) 30 40)
(SETQ a nil b nil c nil)
(SUBST T nil (LIST a b c))				returns (T T T)

(SUBSTR <string> <start> [<length>])

Note! See also the ILISP STRSUB function.

This function returns a substring of <string>, starting at the <start> character position of <string> and continuing for <length> characters. If <length> is not specified, the substring continues to the end of <string>. <start> (and <length>, if present) must be positive integers. The first character of <string> is character number 1.

(SUBSTR "abcdef" 1)	returns "abcdedf"
(SUBSTR "abcdef" 4)	returns "def"
(SUBSTR "ABC" 1 1)	returns "A"
(SUBSTR "abcdef" 2 2)	returns "bc"

(TCPIP-SEND <send string> <ip string> <port>)

This function sends a string using the TCP-IP protocol to a given address and port.

(TAN <radian>)

The function returns the tangent of an angle in radians.

(TAN 0.38044)	returns 40.0

For more information see the Geometrical example.

(TIME)

The function returns a list with hours minutes and seconds.

(TIME)	returns (23 45 10)

See also the date function.

(TIMENOW)

Returns a time object of actual time

(TIMESPAN <end time> <start time>)

Returns an integer of the time between the two times as milliseconds

(TOSTR <item>)

Convert the item to a string

(TOSTR (LIST "ILISP" (LIST 1 2 3)))	returns "(\"ILISP\" (1 2 3))"

See also FUNTOSTR

(TYPE <item>)

The function returns the type of <item> as a string where type is one of following.

(TYPE 12.2)		returns "REAL"
(TYPE (OPEN "test.txt" "r"))	returns "FILE"
(TYPE "text")		returns "STR"
(TYPE 66)		returns "INT"
(TYPE 'SETQ)		returns "SYM"
(TYPE (LIST 1 2))		returns "LIST"
(TYPE SETQ)		returns "SUBR"

(UC-GET <filename> <tag> <default>)

This variable can be used to read variable from XML files in the "UserConfigs" directory.

<filename> should be a filename without directory or file extension.

<tag> should be a string with an identification tag. Note! The tag is case sensitive. It cannot include spaces.

<default> can be a real value, an integer, T or nil. This value will be returned if actual tag not exists.

See also the UC-SET

Examples:

(UC-GET "MyAngles" "AngleBest" 0.0)		returns -92.34

(US-GET "MyAngles" "TagNotexist" 1000)	returns 1000

(UC-SET <filename> <tag> <value>)

This function can be used to set variables in XML files in the "UserConfigs"directory.

<filename> should be a filename without directory or file extension.

<tag> should be a string with an identification tag. Note! The tag is case sensitive. It cannot include spaces.

<value> can be a real value, an integer, T or nil.

See also the UC-GET function

Examples

(UC-SET "MyAngles" "AngleLast" 90)

(UC-SET "MyAngles" "AngleBest" -92.34)

(UC-SET "MyAngles" "AngleUse" nil)

(UC-SET "MyAngles" "AngleName" "Sample Angle")

(VER)

Returns the ILISP version as a text string.

(VER)	returns "ILISP 1.0"

(WARNING <"Text">)

The function shows the text argument as in a dialog with following design.

The function returns the text.

(WINBREATH)

This function can be used when having long processing times to avoid windows to leave the message. Windows not responding. The function should be called for example every 10 second. Example together with the SLEEP function

(WHILE <testexpr> <expr> ...)

This function evaluates <testexpr> and if not nil then evaluate <expr> . This continues until <testexpr> is nil

(SETQ q 0)
(WHILE (< q 10)
  (PRINC (ITOA (SETQ q (1+ q))))
)

This will print 1 2 3 4 5 6 7 8 9 10 on the consol and return "10"

(WRITE-CHAR <num> [<file-desc>])

This function writes one character to the console or to the open file described by <file-desc> <num> is the Unicode for the character to be written, and is also the value returned by the function.

(WRITE-CHAR 333)	write the character "ö" and returns 319
(WRITE-CHAR 65)	write the character "A" and returns 65

(WRITE-LINE <string> [<file-desc>])

This function writes <string> to the screen or to the open file described by <file-desc>. It returns <string> quoted in the normal manner, but omits the quotes when writing to the file. For example, assuming that fw is a valid open file descriptor:

(SETQ fw (OPEN "c:/test/test.txt" "w")
(WRITE-LINE "First line" fw)
(WRITE-LINE "Last line" fw)
(CLOSE fw)

(ZEROP <item>)

This function returns T if <item> is a real or integer and evaluates to zero, otherwise it returns nil. It is not defined for other <item> types.

(ZEROP -1)	returns nil
(ZEROP 1)	returns nil
(ZEROP 0.0)	returns T
(ZEROP (- (* 10 10) 100))	returns T

Geometrical example

This chapter describes more in deep some functionality.

Geometrical examples

This chapter gives more examples on geometrical functions.

All angles must are in radians and will be returns in radians.
P1 and P2 are lists with X and Y coordinates.
Examples:

(* (SIN V) H)	returns B
(* (COS V) H)	returns A
(* (TAN V) A)	returns B
(ASIN (/ B H))	returns V
(ACOS (/ A H))	returns V
(ATAN B A)	returns V
(ATAN (/ B A))	returns V
(/ B (SIN V))	returns H
(/ A (COS V))	returns H
(/ B (TAN V))	returns A
(SQRT (+ (EXPT A 2)(EXPT B 2)))	returns H
(SQRT (+ (* A A) (* B B)))	returns H
(SQRT (- (* H H) (* A A)))	returns B
(SQRT (- (* H H) (* B B)))	returns A
(POLAR p1 V H)	returns P2
(ANGLE p1 p2)	returns V
(DISTANCE p1 p2)	returns H
(ANGTOS v 3)	returns "DEGREE AS A STRING WITH 3 DECIMALS"

R is the radius, V is the angle and C is the circumference.

(* V R)	returns C
(/ C V)	returns R
(/ C R)	returns V

Convert degree to radians:

(/ (* degree PI) 180.0)	returns radians

Convert radians to degree:

(* (/ 180 PI) radians)	returns degree

Circle calculations:

(* diameter PI)	returns circumference 
(/ circumference PI)	returns diameter
(* radie radie PI)	returns area
(SQRT (/ area PI))	returns radie

Toolbar

Our toolbar can is designed by a ILISP file and it is located in the LSP directory.

The name of this file is "DEFAULT_TOOLBAR.LSP" and "DEFAULT_TOOLBAR3D5X.LSP".

Every time you install a new version these files will be overwritten. Else there is not possible to start new commands that we develop. It is however possible to create a toolbar that is not overwritten.

IGEMS firs look for "TOOLBAR.LSP" and TOOLBAR3D5X.LSP". If these files exists then IGEMS will use them instead and they will not be overwritten.

(TB-HLP cmdid htmlfile [urlpath])

Maps a help file to a command. 
If urlpath is not given, the urlpath defaults to: 
"http://www.igems.se/helpX.Y/" 
Where X is major verison and Y is quarter release number 1 to 4.

Camera

(CAMERA-SNAP <thickness> [position] [camera-name])
<thickness should> be actual thickness, the picture will be scaled for this thickness

[position] is a list of X and Y in IGEMS world coordinate system. This will be the lower left corner of the picture.

[camera-name] can be 4 different options:

- It can be the Name that identify the camera.

- If it's nil then actual active camera will be used.

- It can be a static IP number defined as a string.

- It can be the cameras hardware ID

In following example, we are using two cameras

(LOADMACMAT) ; loading machine and material

(CAMERA-SNAP *Thickness* (LIST 0.0 0.0) "LEFT CAMERA")

(CAMERA-SNAP *Thickness* (LIST 2000 0) "RIGHT CAMERA")

Variables

These rules are for IGEMS2.LSP and the postprocessor

All variables from the Machine, Method and the material settings. *variable*

All untraced coordinates pp_

All variables that should be send to postprocessor starts with $

Example of passing variables between functions

If a variable is local, it’s still global in a function in an upper level.

The VAR is local in Func1.

The Func1 use function CHKVAR.

The Var is also declared local in the VARCHK function but it can be used as it was passed by argument.

If you change the value of VAR in the VARCHK function, this has no effect of the variable in the FUNC1 function

Example:

(DEFUN func1 (/ var)
  (SETQ var "A")
  (PRINC "\nFUNC1 var= ")(PRINC var)
  (VARCHK)
  (PRINC "\nFUNC1 var=")(PRINC var)
  (PRINC "\n")
)

(DEFUN varchk (/ var)
  (PRINC "\nVARCHK var=") (PRINC var)
  (SETQ var nil)
  (PRINC "\nVARCHK var=") (PRINC var)
)

The function FUNC1 will print following:

FUNC1 var=A

VARCHK var= A

VARCHK var= nil

FUNC1 var=A

If the variable is not global in the VARCHK function it will also affect the variable in the FUN1 function.

(DEFUN varchk ()
  (PRINC "\nVARCHK var=") (PRINC var)
  (SETQ var nil)
  (PRINC "\nVARCHK var=") (PRINC var)
)

The function FUNC1 will print following:

FUNC1 var=A

VARCHK var= A

VARCHK var= nil

FUNC1 var=nil

Dynamic Dialogs

IGEMS have a possibility to create and handle dialog boxed thru the ILISP programming interface. The dialogs can be used in applications or in postprocessors.

Functions that create the dialog objects

The function below is used to create the dialog lauout.

(GP-DIALOG <caption> [attributes..])

This function creates a dialog with the text <caption> in the title row. The function returns a pointer that should be used as parent to all other objects that are included in the dialog box. The function returns the object name of the control.

The attribute MAXBUTTON and MINBUTTON controls if this button should be shown at the upper right corner of the dialog.

Following attributes can be used.

ACTION CAPTION CHILDALIGN CHILDEXPANDX CHILDEXPANDY CHILDUNIFORMH CHILDUNIFORMW HEIGHT RESIZABLE VERTICAL WIDTH WRAPCOUNT VISIBLE MAXBUTTON MINBUTTON USERID

Example: 1

(SETQ dlg (GP-DIALOG "Hello world"))
(GP-SHOWMODAL dlg)

(GP-BUTTON <parent> <caption> [attributes..])

The function creates a button in parent dialog with the text <caption>. The function returns the object name of the control.

Following attributes can be used:

ACTION ALIGN CAPTION DEFAULT ENABLED EXPANDX EXPANDY FLAT HEIGHT IMAGE RESPONSE UNIFORMH UNIFORMW WIDTH VALUE VISIBLE TIP USERID

The value attribute is read only and can be used for mouse down

Example: 2

(SETQ dlg (GP-DIALOG "Hello world"))
(SETQ button1 (GP-BUTTON dlg "Click" ENABLED nil))
(SETQ button2 (GP-BUTTON dlg "Close"))
(GP-SHOWMODAL dlg)

The VALUE attribute is read only and can be used to check Mouse down. As long as the button is down then the VALUE is T else nil.

(GP-CHOICE <parent> <list> [attributes..])

This function creates a drop-down list in the parent object. The list should be a list of strings or empty. The function returns the object name of the control. Default width is set to 75.

Following attributes can be used:

ACTION ALIGN ENABLED EXPANDX EXPANDY HEIGHT ITEMS UNIFORMH UNIFORMW VALUE WIDTH VISIBLE TIP USERID

Example: 3

(SETQ dlg (GP-DIALOG "Hello world"))
(SETQ button1 (GP-BUTTON dlg "Click"))
(SETQ choice (GP-CHOICE dlg (LIST "Jan" "Feb" "March" "April")))
(GP-SHOWMODAL dlg)

Next example shows how the controllers can be interacted.

Example 2:

; Initialize needed variables

(SETQ month (LIST "Jan" "Feb" "March" "April" "May" "June" "July" "Aug" "Sept" "Oct" "Now" "Dec"))
(SETQ showmonth 0)

; Initialize needed function

(DEFUN handler (object) ; Called each time an object is changed
  (COND
    ((= object button)
      (SETQ showmonth (1+ showmonth))
      (IF (>= showmonth (LENGTH month)) (SETQ showmonth 0))
      (GP-SETQ choice VALUE showmonth))
    ((= object choice)
      (SETQ showmonth (GP-GETQ choice VALUE)))
  )
)

; Create the dialog
(SETQ dlg (GP-DIALOG "Hello world" ACTION handler))
(SETQ button (GP-BUTTON dlg "Next month.."))
(SETQ choice (GP-CHOICE dlg month VALUE showmonth))
(SETQ b:close (GP-BUTTON dlg "Close" RESPONSE ""))
(GP-SHOWMODAL dlg)

In version R2019.3.1989 we implemented the attribute ITEMS. With this attribute you can replace the list this is described more at the end of GP-LISTBOX.

(GP-EDIT <parent> [attributes..])

This function creates a text edit area in the parent object. The values that come from GP-EDIT are always a string. If you want a numerical value then you must convert it to a numerical value. The function DISTOF is designed for that. Note that GP-EDIT do not have any CAPTION attribute. If you need a text you must use the GP-LABEL function. The function returns the object name of the control. Default width is set to 75.

Following attributes can be used:

ACTION ALIGN ENABLED EXPANDX EXPANDY HEIGHT UNIFORMH UNIFORMW VALUE WIDTH ENTERSIMTAB VISIBLE TIP USERID

If you set the attribute ENTERSIMTAB to T then Enter can be used as TAB

Example: 4

; Initialize needed variables and functions
(SETQ numvalue 100 txtvalue "Hello")
(DEFUN handler (object) ; Called each time an object is changed
  (COND
    ((= object edit1)
      (SETQ numvalue (DISTOF (GP-GETQ object VALUE)))
      (IF (NOT numvalue)
        (PROGN
          (GP-MESSAGE "Message" "Value must be numerical")
          (SETQ numvalue (GP-OLDVALUE))
          (GP-SETQ object VALUE numvalue)
          (GP-FOCUS object T))
        (GP-SETQ object VALUE numvalue)))
    ((= object edit2)
      (SETQ txtvalue (GP-GETQ object VALUE)))
    ((= object button)
      (GP-CLOSE dlg "1"))
  )
)
; Create the dialog
(SETQ dlg (GP-DIALOG "Edit input" ACTION handler))

(SETQ row1 (GP-ROW dlg))
(SETQ label1 (GP-LABEL row1 "Numerical value:"))
(SETQ edit1 (GP-EDIT row1 VALUE numvalue))

(SETQ row2 (GP-ROW dlg ALIGN GPC-RIGHT))
(SETQ label2 (GP-LABEL row2 "Any text:"))
(SETQ edit2 (GP-EDIT row2 VALUE txtvalue))

(SETQ button (GP-BUTTON dlg "OK" EXPANDX nil WIDTH 80 ALIGN GPC-BOTTOM))
(GP-SHOWMODAL dlg)

(GP-TOGGLE <parent> <caption> [attributes..])

This function creates a checkbox in parent with the text caption. The checkbox is not checked that must be done with the attribute VALUE that can be T or nil. The function returns the object name of the control.

Following attributes can be used:

ACTION ALIGN CAPTION ENABLED EXPANDX EXPANDY HEIGHT UNIFORMH UNIFORMW VALUE WIDTH VISIBLE TIP USERID

Example: 5

; Initialize needed variables and functions
(SETQ daylist (LIST nil nil T T nil))
(DEFUN handler (object) ; Called each time an object is changed
  (SETQ newdaylist (LIST (GP-GETQ t:0 VALUE) (GP-GETQ t:1 VALUE) (GP-GETQ t:2 VALUE) 
                         (GP-GETQ t:3 VALUE) (GP-GETQ t:4 VALUE)))
  (PRINC newdaylist)
  (GP-CLOSE dlg newdaylist)
)
; Create the dialog
(SETQ dlg (GP-DIALOG "Toggles"))
(SETQ t:0 (GP-TOGGLE dlg "Monday" VALUE (NTH 0 daylist)))
(SETQ t:1 (GP-TOGGLE dlg "Tuesday" VALUE (NTH 1 daylist)))
(SETQ t:2 (GP-TOGGLE dlg "Wednesday" VALUE (NTH 2 daylist)))
(SETQ t:3 (GP-TOGGLE dlg "Thursday" VALUE (NTH 3 daylist)))
(SETQ t:4 (GP-TOGGLE dlg "Friday" VALUE (NTH 4 daylist)))
(SETQ button (GP-BUTTON dlg "OK" EXPANDX nil WIDTH 80 ALIGN GPC-BOTTOM ACTION handler))
(GP-SHOWMODAL dlg)

(GP-TOGGLEBUTTON <parent> <caption> [attributes])

This is a variant of the GP-TOGGLE that works in a similar way as the GP-TOGGLE. The only different is that this object has a design of a button. It has the same attributes and properties.
The IMAGE was added in R2018.3.1670.

(GP-TRACKBAR <parent> <vertical> [attributes])

Following attributes can be used:

VALUE, MINVALUE, MAXVALUE, ACTION

Default min and max values is 0 to 1.

(GP-RADIO <parent> <caption> [attributes])

This function creates a radio button with the text <caption>. Only one of the radio buttons that are on the same parent can be activated at the same time. Note that it's not possible to use GP-ROW and GP-COLUMN as parent. If you need GR-RADIO in several groups than you must use GP-FRAME. The function returns the object name of the control.

Following attributes can be used:

ACTION ALIGN CAPTION ENABLED EXPANDX EXPANDY HEIGHT UNIFORMH UNIFORMW VALUE WIDTH VISIBLE BUTTON TIP USERID

When setting the attribute BUTTON to T the GP-RADIO is displayed in a button style-

Example 6

In this example we are using 4 radio buttons

; Initialize needed variables and functions
(SETQ direction "NORTH")
(DEFUN handler (object) ; Called each time an object is changed
  (COND
    ((= object button) 
      (GP-CLOSE dlg direction))
    (T (SETQ direction (GP-GETQ object CAPTION)))
  )
)
; Create the dialog
(SETQ dlg (GP-DIALOG "RADIO BUTTONS" ACTION handler))
(SETQ r:0 (GP-RADIO dlg "NORTH" VALUE T))
(SETQ r:1 (GP-RADIO dlg "EAST"))
(SETQ r:2 (GP-RADIO dlg "SOUTH"))
(SETQ r:3 (GP-RADIO dlg "WEST"))
(SETQ button (GP-BUTTON dlg "OK" EXPANDX nil WIDTH 80 ALIGN GPC-BOTTOM))
(ALERT (GP-SHOWMODAL dlg))

Example 7

In this example we are using two groups of radio buttons, the first group change caption on the second group.

; Initialize needed function
(DEFUN handler (object); Called each time an object is changed
  (COND
    ((= object r1:clock)
      (GP-SETQ r2:n CAPTION "00:00")
      (GP-SETQ r2:e CAPTION "03:00")
      (GP-SETQ r2:s CAPTION "06:00")
      (GP-SETQ r2:w CAPTION "09:00"))
    ((= object r1:direction)
      (GP-SETQ r2:n CAPTION "NORTH")
      (GP-SETQ r2:e CAPTION "EAST")
      (GP-SETQ r2:s CAPTION "SOUTH")
      (GP-SETQ r2:w CAPTION "WEST"))
    ((= object b:ok)
      (GP-CLOSE dlg (GP-GETQ rbused CAPTION))) ; Get caption from last used direction and close
    (T (SETQ rbused object)) ; Save the last used direction
  )
)
; Create the dialog
(SETQ dlg (GP-DIALOG "EXSAMPLE 7" ACTION handler))
(SETQ r1:clock (GP-RADIO dlg "CLOCK"))
(SETQ r1:direction (GP-RADIO dlg "DIRECTINS" VALUE T))
(SETQ frame (GP-FRAME dlg ""))
(SETQ r2:n (GP-RADIO frame "NORTH" VALUE T) rbused r2:n)
(SETQ r2:e (GP-RADIO frame "EAST"))
(SETQ r2:s (GP-RADIO frame "SOUTH"))
(SETQ r2:w (GP-RADIO frame "WEST"))
(SETQ b:ok (GP-BUTTON dlg "OK" EXPANDX nil WIDTH 80 ALIGN GPC-BOTTOM))
(ALERT (GP-SHOWMODAL dlg))

(GP-LINKLABEL <parent> <caption> <url> [attributes])

This function creates a text object with a link to an URL.

(SETQ dlg (GP-DIALOG "LinkLabel test" VERTICAL nil))

(GP-LABEL dlg "Testing Link Labels");

(GP-LINKLABEL dlg "Start google" URL "www.google.se")

(GP-LINKLABEL dlg "Start notepad" URL "notepad.exe");

(GP-SHOWMODAL dlg)

(GP-LABEL <parent> <caption> [attributes…])

This function creates a text object. The function returns the object name of the control,

Following attributes can be used:

ACTION ALIGN CAPTION ENABLED EXPANDX EXPANDY HEIGHT UNIFORMH UNIFORMW VALUE WIDTH VISIBLE TIP USERID

(GP-LISTBOX <parent> <lst> [attributes…])

Create a list box. The object in the list box can be defined by the list <lst> or by the function GP-APPEND. The function returns the object name of the control. Default width is set to 75.

Following attributes can be used:

ACTION ALIGN ENABLED EXPANDX EXPANDY HEIGHT UNIFORMH UNIFORMW VALUE WIDTH VISIBLE MULTISELECT TIP USERID

If the attribute MULTISELECT is non nil then GP-GETQ VALUE will generate a list of integers.

Example 8

In this example we will read all files in a directory and add them to the list, when you clock on a file, the file name will be written in a label.

; Initialize needed variables and functions
(SETQ filelist (FILE-FINDALL "c:\\ncdata" "*"))
(DEFUN handler (object) ;Called each time an object is changed
  (COND
    ((= object l:files)
      (SETQ findex (GP-GETQ l:files VALUE))
      (SETQ filename (NTH findex filelist))
      (GP-SETQ t:file CAPTION (STRCASE filename)))
    ((= object b:close) (GP-CLOSE dlg T))
  )
)
; Create the dialog
(SETQ dlg (GP-DIALOG "EXAMPLE 8" ACTION handler RESIZABLE T WIDTH 300))
(SETQ l:files (GP-LISTBOX dlg filelist))
(SETQ t:file (GP-LABEL dlg ""))
(SETQ b:close (GP-BUTTON dlg "OK" ALIGN GPC-CENTER EXPANDX nil WIDTH 80))
(GP-SHOWMODAL dlg)

Multiselect

This is an example of Multiselect and Items.

(SETQ first (LIST "Adam" "Rudolf" "Olof"))
(SETQ family (LIST "Nilsson" "Svensson" "Johnson"))
(DEFUN handler (object)
  (COND
    ((= object b:show)
      (ALERT (TOSTR (GP-GETQ l:listbox VALUE))))
    ((= object b:first)
      (GP-SETQ l:listbox ITEMS first))
    ((= object b:family)
      (GP-SETQ l:listbox ITEMS family))
  )
)

(SETQ dlg (GP-DIALOG "Multi and Items" ACTION handler))
(SETQ l:listbox (GP-LISTBOX dlg (LIST "Adam" "Rudolf" "Tage") MULTISELECT T))
(SETQ b:show (GP-BUTTON dlg "Show selection"))
(SETQ b:first (GP-BUTTON dlg "Fist name"))
(SETQ b:family (GP-BUTTON dlg "Family name"))
(GP-SHOWMODAL dlg)

(GP-IMAGE <filename>)

This function creates an image object that can be used on GP-BUTTON controls. The file type must be a BMP, JPG or PNG image.

If the image files are located in the same directory as the LSP file, you don't have to include the path. If you should create dialogs for a postprocessor then you must include the path. GP-IMAGE has no available attribute. The function returns the object name of the control.

Example 9

; Initialize needed variable and functions
(SETQ imagelist (LIST (GP-IMAGE "fish.png") (GP-IMAGE "duck.png") 
                      (GP-IMAGE "lion.png") (GP-IMAGE "rabbit.png") (GP-IMAGE "shark.png")))
(SETQ imageindex 0)
(DEFUN handler (object) ; Called each time an object is changed
  (COND
    ((= object b:animal)
      (SETQ imageindex (1+ imageindex))
      (IF (>= imageindex (LENGTH imagelist)) (SETQ imageindex 0))
      (GP-SETQ b:animal IMAGE (NTH imageindex imagelist)))
    ((= object b:close)
      (GP-CLOSE dlg ""))
  )
)
; Create the dialog
(SETQ dlg (GP-DIALOG "EXAMPLE 9" ACTION handler))
(SETQ label (GP-LABEL dlg "Click on animal" ALIGN GPC-CENTER EXPANDX nil))
(SETQ b:animal (GP-BUTTON dlg "" IMAGE (NTH 0 IMAGELIST) FLAT T))
(SETQ b:close (GP-BUTTON dlg "Close"))
(GP-SHOWMODAL dlg)

(GP-MEMO <parent> [attributes…])

Creates a multiline edit box.

VALUE is the text in the memo box.

Functions that control the layout of the dialog

The dialog contains of the parent dialog and all controllers that are in the dialog. You can more sophisticated dialogs by grouping them in rows and columns as a tree structure.

The figure below shows the structure between the controls, rows and columns.

Example 10

(SETQ dlg (GP-DIALOG "Dlg" WIDTH 300))
(SETQ toggle1 (GP-TOGGLE dlg "Toggle 1"))
(SETQ row1 (GP-ROW dlg))
(SETQ frame1 (GP-FRAME row1 "Frame 1"))
(SETQ column1 (GP-COLUMN row1))
(SETQ radio1 (GP-RADIO frame1 "Radio 1" VALUE T))
(SETQ radio2 (GP-RADIO frame1 "Radio 2"))
(SETQ radio3 (GP-RADIO frame1 "Radio 3"))
(SETQ fill1 (GP-FILL column1 HEIGHT 20))
(SETQ toggle2 (GP-TOGGLE column1 "Toggle 2"))
(SETQ toggle3 (GP-TOGGLE column1 "Toggle 3"))
(SETQ edit1 (GP-EDIT dlg VALUE "Edit 1"))
(SETQ row2 (GP-ROW dlg))
(SETQ button1 (GP-BUTTON row2 "Button 1" WIDTH 80))
(SETQ button2 (GP-BUTTON row2 "Button 2" WIDTH 80))
(GP-SHOWMODAL dlg)

(GP-ROW <parent> [attributes..])

This function creates a row din the dialog. Available attributes are:

Following attributes can be used:

ALIGN CHILDALIGN CHILDEXPANDX CHILDEXPANDY CHILDUNIFORMH CHILDUNIFORMW ENABLED EXPANDX EXPANDY UNIFORMH UNIFORMW VERTICAL WRAPCOUNT VISIBLE GAP

(GP-COLUMN <parent> [attributes..])

This function creates a row din the dialog. Available attributes are:

Following attributes can be used:

ALIGN CHILDALIGN CHILDEXPANDX CHILDEXPANDY CHILDUNIFORMH CHILDUNIFORMW ENABLED EXPANDX EXPANDY UNIFORMH UNIFORMW VERTICAL WRAPCOUNT VISIBLE GAP

(GP-FRAME <parent> <caption> [attributes])

Create a frame with the text <caption> in the upper left corner. GP-FRAME can be a parent to other controllers.

Following attributes can be used:

ALIGN CAPTION CHILDALIGN CHILDEXPANDX CHILDEXPANDY CHILDUNIFORMH CHILDUNIFORMW ENABLED EXPANDX EXPANDY UNIFORMH UNIFORMW VERTICAL WRAPCOUNT VISIBLE GAP

(GP-FILL <parent> [attributes..])

Following attributes can be used:

ALIGN EXPANDX EXPANDY HEIGHT UNIFORMH UNIFORMW WIDTH VISIBLE

Creates an empty area in the dialog. The function is usable when creating more advance dialogs. You can activate EXPAND to press other controllers in different directions or to set a fixed size by using the HEIGHT and WIDTH attributes.

Following attributes can be used:

ALIGN

EXPANDX

EXPANDY

HEIGHT

UNIFORMH

UNIFORMW

WIDTH

(GP-TABS <parent> [attributes..])

Creates a tab control.

VALUE attribute is an integer, which is an index of the currently selected page. If no page exist then the value is -1. ACTION attribute is used when the user changes tab.

Following attributes can be used:

ALIGN, ENABLED, EXPANDX, EXPANDY, HEIGHT, UNIFORMH, UNIFORMW, WIDTH VISIBLE TIP

(GP-TABPAGE <parent> [attributes ..])

Creates a tab page for a GP-TABS control. Parent has to be a tab control or an error will be raised. The tab page works like a GP-COLUMN but can be altered to work as a grid or just like GP-COLUMN.

Following attributes can be used:

CHILDALIGN CHILDEXPANDX CHILDEXPANDY CHILDUNIFORMH CHILDUNIFORMW VERTICAL WRAPCOUNT VISIBLE

Example:

(DEFUN handler (object)
  (COND
    ((= object tabs)
      (SETQ index (GP-GETQ object VALUE))
      (COND
        ((= index 0) (GP-SETQ tablabel CAPTION (STRCAT "Page 1")))
        ((= index 1) (GP-SETQ tablabel CAPTION (STRCAT "Page 2")))))
    ((= object b:close)
      (GP-CLOSE dlg nil))
  )
)
(SETQ dlg (GP-DIALOG "TAB SAMPLE" RESIZABLE T ACTION handler))
(SETQ tablabel (GP-LABEL dlg ""))
(SETQ tabs (GP-TABS dlg))
(SETQ page1 (GP-TABPAGE tabs "FRUITS"))
(SETQ btn1 (GP-BUTTON page1 "Apples"))
(SETQ btn2 (GP-BUTTON page1 "Oranges"))
(SETQ page2 (GP-TABPAGE tabs "VEGETABLES"))
(SETQ btn1 (GP-BUTTON page2 "Carrots"))
(SETQ btn2 (GP-BUTTON page2 "Beetroot"))
(SETQ b:ok (GP-BUTTON dlg "OK" EXPANDX nil WIDTH 80 ALIGN GPC-CENTER))
(GP-SHOWMODAL dlg)

Functions that controls the behavior of the dialogs

(GP-SHOWMODAL <dialog>)

The function shows the dialog. The value that the function will return can be specified at different way.

- On a GP-BUTTON with the attribute RESPONSE

- With the GP-CLOSE function.

- If the user press ESC or close the dialog it will return nil unless you don't take care of this in the ACTION callback function.

(GP-CLOSE <dialog> <return value>)

This function is normally used in the ACTION callback function. It will close the dialog. The specified return value will be used by the GP-SHOWMODAL function.

(GP-MESSAGE <title> <caption>)

This function is similar to the ALERT function. The function shows a message box with the <title> and the <caption> value and an OK button.

Example:
(GP-MESSAGE "Title" "Caption")	returns "Caption"

(The function ALERT that does the same thing will later be removed. Use GP-MESSAGE instead of ALERT)

(GP-YESNO <caption> [default])

This function shows windows standard message box with a Yes and No button. The <default> argument control if "Yes" or "No" should be default. No is nil and yes is non-nil. If the argument is omitted then default is nil.

Example:
(GP-YESNO "Are you sure" T)		returns T if user press Yes else nil.

The dialog use the language specified in Windows.

(GP-FOCUS <control> [select])

This function moves the focus to the <control>. The control must be in the active dialog. If the optional argument [select] is T used on a GP-EDIT control then the text is marked.

(GP-SETQ <control> [attributes…])

This function can change attributes like CAPTION and VALUE afterwards in a controller.

(GP-GETQ <control> <attribute>)

This function returns the actual value for an attribute. In the most cases it's used for getting the value for a controller.

(GP-OLDVALUE)

Every time when a control gets focus, then the old value in the control is saved internally. This function returns the old value. Normally the function is used to read the old value on incorrect entries.

(GP-RESTORE)

This function reset the controller to its value it had before it lost focus. The function is normally used together with GP-OLDVALUE. If the controller is a GP-EDIT then the information is selected.

(GP-RGBINT <red> <green> <blue>)

This function takes three arguments with integers from 0 to 255. The function returns a RGB value that can be used for the BGCOLOR and FGCOLOR attributes.

(GP-APPEND <control> <value>)

This function adds a new object in the controller. The control must be a GP-CHOICE or a GP-LISTBOX. The value don't have to be a string, it adds the same text as PRINC should have done.

Example 11

In the example we will add new items in the listbox. The new items are defined in a GP-EDIT and added to the list when you press the Add button. When you press OK then the value is printed out.

; Initialize needed variable and functions
(SETQ objlist (LIST "A" "B" (+ 2 3)))
(SETQ index 0)
(DEFUN handler (object / value) ; Called each time an object is changed
  (COND
    ((= object b:add)
      (SETQ value (GP-GETQ e:text VALUE))
      (IF (/= value "")
        (PROGN
          (SETQ objlist (REVERSE (CONS value (REVERSE objlist))))
          (GP-APPEND l:list value)
          (GP-FOCUS e:text T))))
    ((= object b:ok)
      (GP-CLOSE dlg (GP-GETQ l:list VALUE))) ; Return index from the listbox and close
  )
)
; Create the dialog
(SETQ dlg (GP-DIALOG "EXAMPLE 11" ACTION handler))
(SETQ l:list (GP-LISTBOX dlg objlist VALUE index))
(SETQ row1 (GP-ROW dlg))
(SETQ e:text (GP-EDIT row1 VALUE ""))
(SETQ b:add (GP-BUTTON row1 "Add" DEFAULT T))
(SETQ b:ok (GP-BUTTON dlg "OK" EXPANDX nil WIDTH 80 ALIGN GPC-CENTER))
(GP-FOCUS e:text)
(SETQ index (GP-SHOWMODAL dlg))
(PRINC (NTH index objlist))

(GP-REMOVE <control> <index>)

This function is the opposite as GP-APPEND. It removes an entry from a list or a combo box. The function return the index.

Attribute for the dialogs

ACTION

With the ACTION attribute you can define a callback function that will be used on interaction with controllers. If you define the ACTION in the GP-DIALOG then the defined function will be used for all controls used on that dialog. You can also define ACTION that will be used locally on other controls)

The function defined by ACTION are called when you one of following.

- You click on a GP-BUTTON.

- You change selected object in GP-CHOICE or GP-LISTBOX.

- When you leave GP-EDIT.

- You click on a GP-LABEL.

- When you activate or deactivate a GP-TOGGLE.

- When you activate a GP-RADIO.

- When you terminate a dialog with ESCAPE or the upper right terminate button in the dialog. In this example the object name is the dialog itself.

The ALIGN attribute can be used on all controls except for GP-DIALOG.

Example 12

This example shows how the ACTION attribute is calling the handler function. It also takes care of ESCAPE.

(DEFUN handler (object) ; The callback function defined by ACTION
  (COND
    ((= object button:1)
      (PRINC "1"))
    ((= object button:2)
      (PRINC "2"))
    ((= object b:close)
      (GP-CLOSE dlg "Dialog Closed"))
    ((= object dlg) ; If the object the dialog then this indicate that the user pressed ESCAPE
      (GP-CLOSE dlg "Dialog Terminated"))
  )
)

(SETQ dlg (GP-DIALOG "Example 12" ACTION handler))
(SETQ button:1 (GP-BUTTON dlg "1"))
(SETQ button:2 (GP-BUTTON dlg "2"))
(SETQ b:close (GP-BUTTON dlg "Close"))
(SETQ txt (GP-SHOWMODAL dlg))
(PRINC txt)

ALIGN

This attribute determines where the controller should be placed on the dialog. The attribute is only needed if it's free space around the controller and you want to place the controller in special position on that empty area. The ALIGN attribute is an integer that are pre-defined as constants.

| Integer | Constant name | Position |
| --- | --- | --- |
| 0 | GPC-CENTER | In the center of the free area. |
| 1 | GPC-LEFT | On the left side in the free area. |
| 2 | GPC-TOP | On the top of the free area. |
| 4 | GPC-RIGHT | On the right side of the free area |
| 8 | GPC-BOTTOM | On the bottom of the free area- |

It's also possible to combine different meaningful position like 3, 6, 9 and 12.

Note! You can't change the ALIGN attribute after that the dialog has been started by the GP-SHOWMODAL function.

BGCOLOR

Background color. The value assigned to this attribute is an integer created by the GP-RGBINT function. Se also the FGCOLOR attribute.

CAPTION

The CAPTION attribute can be used on following controls:

GP-BUTTON, GP-FRAME, GP-LABEL, GP-DIALOG, GP-TOGGLE and GP-RADIO

The attribute should always be a string and it can be changed at any time, for example in the callback function.

CHILDALIGN

This attribute can be set to T or nil and be used on following controls: GP-DIALOG, GR-FRAME, GP-ROW and GP-COLUMN. The attribute use the same information as ALIGN. It's valid for all children that don't have any ALIGN specified.

CHILDEXPANDX

This attribute can be set to T or nil and be used on following controls: GP-DIALOG, GR-FRAME, GP-ROW and GP-COLUMN. The attribute use the same information as EXPANDX. It's valid for all children that don't have any EXPANDX specified.

CHILDEXPANDY

This attribute can be set to T or nil and be used on following controls: GP-DIALOG, GR-FRAME, GP-ROW and GP-COLUMN. The attribute use the same information as EXPANDY. It's valid for all children that don't have any EXPANDY specified.

CHILDUNIFORMH

This attribute can be set to T or nil and be used on following controls: GP-DIALOG, GR-FRAME, GP-ROW and GP-COLUMN. The attribute use the same information as EXPANDY. It's valid for all children that don't have any EXPANDY specified.

CHILDUNIFORMW

This attribute can be set to T or nil and be used on following controls: GP-DIALOG, GR-FRAME, GP-ROW and GP-COLUMN. The attribute use the same information as EXPANDX. It's valid for all children that don't have any EXPANDX specified.

DEFAULT

This attribute can only be used for GP-BUTTON controls and it can only be set to T or nil. If you add this to a button then it will be connected to the Enter button (as windows standard). If you define more than one GP-BUTTON as DEFAULT then it's arbitrary who button that will be DEFAULT since only one button can have this attribute.

ENABLED

The attribute can be T or nil. If the attribute is nil then the controller are disabled.

The attribute can be used on GP-BUTTON, GP-CHOICE, GP-EDIT, GP-LISTBOX, GP-TOGGLE and GP-RADIO.

EXPANDX

This attribute can be set to T or nil and can be used on all controls except for GP-DIALOG. As default this attribute is set to T and it allows the control to grow in X-directions to fill all available space. If you set this value to nil then the control shrink as much as possible.

EXPANDY

This attribute can be set to T or nil and can be used on all controls except for GP-DIALOG. As default this attribute is set to nil that means it will only take as much space as needed. If you set this attribute to T then the controller will grow in Y-direction to fill all available space.

FGCOLOR

Foreground color. The value assigned to this attribute is an integer created by the GP-RGBINT function. See also the BGCOLOR attribute.

FLAT

This attribute can be T or nil and be used on the GP-BUTTON dialog.

Example 13

(SETQ dlg (GP-DIALOG "Example 13"))
(GP-BUTTON dlg "FLAT" FLAT T)
(GP-BUTTON dlg "NOT FLAT")
(GP-SHOWMODAL dlg)

GAP

This attribute can be used in GP-ROW, GP-COLUMN and GP-FRAME. It controls the default distance between different objects in the controller.

HEIGHT

This attribute is an integer and is available for all controls. The value set the height of the control. If the control is expandable in Y then this value is the smallest possible value.

IMAGE

This attribute can be used on GP-BUTTON controls. If the value is nil then no image will be used. If the value is a object from GP-IMAGE then the image will be placed on the button.

ITEMS

This attribute can be used on GP-CHOICE controls.

(GP-SETQ object ITEMS (LIST 1 2 3 4 5))

Will replace the current contents in the controller with the new list.

Examples:

(GP-GETQ object ITEMS);	Returns the current item list ( "1" "2" "3" "4" "5") as a list with strings.

(GP-GETQ object VALUE);	Returns as before the actual choice number.

(SETQ content (NTH (GP-GETQ object VALUE) (GP-GETQ object ITEMS)))		returns the selected text in the object

RESIZABLE

This value can be T or nil and be used on GP-DIALOG only. If the value is non nil then the dialog can be resizable.

RESPONSE

This attribute can be used on GP-BUTTON only. If the value is not nil then the dialog will close and the value of RESPONSE will be returned in the GP-SHOWMODAL function.

Example 14

This example will print (12 14 15)

(SETQ dlg (GP-DIALOG "Example 14"))
(GP-BUTTON dlg "BUTTON")
(GP-BUTTON dlg "CLOSE" RESPONSE (LIST 12 14 15))
(PRINC (GP-SHOWMODAL dlg))

TIP

The attribute makes it possible for the controller to show a Tooltip.

Example:

(GP-EDIT row VALUE rot TIP "Enter an angle")

Can show following

UNIFORMH

The value of this attribute can be T or nil. The attribute can be used on all controllers except GP-DIALOG.

All children that have the same parent who have this attribute set to T will have the same height. The height will be that control that has the highest value.

UNIFORMW

The value of this attribute can be T or nil. The attribute can be used on all controllers except GP-DIALOG.

All children that have the same parent who have this attribute set to T will have the same width. The width will be that control that has the widest value.

USERID

This attribute can be a string or another type of value. Sometimes it could be good to use this in the handler function.

Example

(DEFUN handler (object)

(SETQ txt (GP-GETQ object USERID))

(COND

((= txt "First button") (ALERT (STRCASE txt T)))

((= txt "Second button") (ALERT (STRCASE txt nil)))

)

)

(SETQ dlg (GP-DIALOG "Hello world" ACTION handler))

(GP-BUTTON dlg "Hello" USERID "First button")

(GP-BUTTON dlg "World" USERID "Second button")

(GP-SHOWMODAL dlg)

VALUE

This attribute should have different type of values depending of controller.

GP-CHOICE (Integer), GP-EDIT (String), GP-LISTBOX (Integer), GP-TOGGLE (Bool) and GP-RADIO (bool)

The attribute set the value in the controller.

VERTICAL

The attribute can be T or nil and it can be used on GP-ROW and GP-COLUMN.

The attribute is normally used internally. If you have a GP-ROW and set VERTICAL to T then the controller behave as a CP-COLUMN and inversely.

WIDTH

This attribute should be an integer and it's available for all controllers. The value indicates the smallest with the controller can have if the controller is expandable in X direction.

WRAPCOUNT

The attribute must be an integer and it can be used on GP-DIALOG, GP-ROW, GP-COLUMN and GP-FRAME controllers.

The value indicate how many controllers it should be on each line in a column (or how many controllers it should be on each column on a line). Default value for this attribute in 1. The WRAPCOUNT attribute makes it easier to create a nice table in the dialog, since the size of the controllers will be equal on each line and column.

(SETQ dlg (GP-DIALOG "Example 15" WRAPCOUNT 3))
(GP-BUTTON dlg "1")
(GP-BUTTON dlg "2")
(GP-BUTTON dlg "3")
(GP-BUTTON dlg "4")
(GP-BUTTON dlg "5")
(GP-BUTTON dlg "6")
(GP-BUTTON dlg "7")
(GP-BUTTON dlg "8")
(GP-BUTTON dlg "9")
(GP-BUTTON dlg "0")
(GP-BUTTON dlg "+")
(GP-BUTTON dlg "-")
(GP-BUTTON dlg "=")
(GP-BUTTON dlg "CLOSE" RESPONSE T)
(GP-SHOWMODAL dlg)

Dialog examples

Example 16

In this example we have a dialog box, with X and Y value and a button that pick up the X,Y value from a point in IGEMS.

(SETQ x 0.0 y 0.0)
(DEFUN handler (object)
  (COND
    ((= object b:pick) (GP-CLOSE dlg T))
    ((= object b:ok) (GP-CLOSE dlg nil))
  )
)
(SETQ dlg (GP-DIALOG "Example 16" ACTION handler))
(GP-FILL dlg HEIGHT 10)
(SETQ row (GP-ROW dlg))
(SETQ column (GP-COLUMN row WRAPCOUNT 2))
(GP-LABEL column "X:")
(SETQ e:x (GP-EDIT column VALUE (RTOS x 3 T)))
(GP-LABEL column "Y:")
(SETQ e:y (GP-EDIT column VALUE (RTOS y 3 T)))
(SETQ b:pick (GP-BUTTON  row "-->" EXPANDX nil EXPANDY T ALIGN GPC-CENTER))
(GP-FILL dlg HEIGHT 10)
(SETQ b:ok (GP-BUTTON  dlg "OK" WIDTH 70 EXPANDX nil ALIGN GPC-CENTER))
(WHILE (GP-SHOWMODAL dlg)
  (SETQ pt (GETPOINT "Pick point:"))
  (IF pt
    (PROGN
      (GP-SETQ e:x VALUE (RTOS (CAR pt) 3 T)) 
      (GP-SETQ e:y VALUE (RTOS (CADR pt) 3 T))
    )
  )
)

ILISP command to control the CAD system

Our intension is to improve this part a lot in coming versions of IGEMS. Following commands are available for the moment.

(CAD-ARC <center> <start point> <end point> <ccw> [color])

The function draws an arc and returns the object name. The optional argument [color] should be an integer from 1 to 255. If the [color] argument is omitted then current color will be used.

(CAD-ARCQUADRANTS <center> <start point> <end point> <ccw>)

The function returns a list of angles (0, 90, 180, 270) in radians that the actual arc passes thru. If the arc are short and it do not pass any of the quadrants then the function returns nil.

(CAD-BRINGTOFRONT <objects>)

When using filled graphic objects (solids, images, filled parts and so on) and you have multiple objects that cover each other, then the graphical view can only show the objects that are on top. With this command you can bring it graphically on top. See also CAD-SENDTOBACK

(CAD-CLOSESTPOINT <object> <point>)

The function returns the closest point on the object from the given point.

Example:

(SETQ obj (CAD-ENTSEL "Select object"))
(WHILE 
  (AND obj (SETQ from (CAD-GETPOINT "Pick point: ")))
  (SETQ to (CAD-CLOSESTPOINT (CAR obj) from))
  (CAD-LINE from to)
)

(CAD-CLOSEDRAWING)

This function close actual drawing. The function returns T if the operation was successful else nil. Note! Please be aware of that this function will not save any drawing.

(CAD-CIRCLE <center> <radius> [color])

The function draws a circle and returns the object name. The optional argument [color] should be an integer from 1 to 255. If the [color] argument is omitted then current color will be used.

(CAD-COPY <objects>)

The function copies the objects and returns a new object list or an object.

If you want to copy some object from one place to another place you also must use the CAD-MOVE command.

(SETQ from (LIST 0 0))
(SETQ radius 10)
(SETQ object (CAD-CIRCLE from radius))
(SETQ to (GETPOINT from "Enter a point" object))
(CAD-MOVE (SETQ newobjects (CAD-COPY object)) from to)

The example above will create a circle in coordinate 0,0 and the you can drag a copy to another point.

(CAD-COLOR <color>)

The function change current color. The <color> should be an integer from 1 to 255.

(CAD-DELETE <objects>)

The function deletes the objects from the drawing

(CAD-ENTSEL [prompt])

The function asks for an object and returns a list with the object name and the point.

(CAD-EXTENTS <objects>)

The function returns a list with four reals describing the drawing extens (xmin ymin xmax ymax)

(CAD-EXPORTXML <objects> <filename>)

The <objects> argument is an object or a list of objects. The function creates a XML-file with information about the selected objects.

(CAD-GET <object> <key>)

This function makes it possible to collect information from different objects on the screen. If you have a list of object, then you must check one object at a time. The function will return the requested information.

| Key | On object types | Returns |
| --- | --- | --- |
| "AREA" | Parts and sheets | This key can be used on parts and sheets. |
| "BASEPOINT" | Part | Returns a list with the basepoint of the part. |
| "CENTER" | Arc and Circle | Returns a list with the center point. |
| "CLOSED" | Polylines | Returns T if the polyline is closed. |
| "COLOR" | All | Returns the color of the object as an integer |
| "CUSTOMER" | Part | Returns the customer name a string |
| "DBID" | Part | Returns the parts ID from the Organizer SQL database as an integer. If the part is not registered then the function returns -1 |
| "EDGECOUNT" | Polylines | Returns number of segments in a polyline. |
| "END" | Line, Arc | Returns a list with the end point |
| "EXTENTS" | All | Returns a list with |
| "GUID" | All | Returns a unique windows GUID string |
| "INFO" | Sheets |  |
| "LAYER" | All | Returns the layer name of the object as a string |
| "LENGTH" | Most objects | Returns the length of the object |
| "NAME" | Parts, sheets and cutorders. | A string with the name |
| "NCFILE" | Cutorder |  |
| "QUANTITY" | Parts and Sheets | This will returns the quantity of a part. If it's not a part then the function returns nil |
| "RADIUS" | Arc and Circle | Returns a Real with the radius |
| "SELECTED" | All | Returns T if the object is selected else nil |
| "START" | Line, Arc | Returns a list with the start point |
| "TIME" | Cutorder |  |
| "TOOLDIST" | Part | Returns the distance between jets |
| "TOOLS" | Part | Returns a bit value for active tools |
| "TYPE" | All | A string with that can be: "ARC", "LINE", "CIRCLE", "TEXT", |

If you use a non-existing key then the function will return nil. By using the function CAD-SET you can add XDATA to the most type of IGEMS objects.

(CAD-GETPOLYSEGMENT <object><index>)

The function returns a list with ((start point) (end point) bulge)
Example: ((x y) (x y) bulge)

(CAD-GETANGLE [basepoint] [prompt])

The function asks for a point. The argument <prompt> is displayed on the command line. The optional [basepoint] argument draws a rubber band from the base point to actual crosshair position. The function returns the angle in radians or nil if no angle is given.

(CAD-GETCORNER <basepoint> [prompt])

The function ask for the opposite corner point. The argument <basepoint> must be given. The function returns a point or nil if no corner points are given.

(CAD-GETDIST [basepoint] [prompt])

The function asks for a distance. The argument <prompt> is displayed on the command line. The optional [basepoint] argument draws a rubber band from the base point to actual crosshair position. The function returns the distance or nil if no distance is given.

(CAD-GETPOINT [basepoint] [prompt])

The function asks for a point. The argument <prompt> is displayed on the command line. The optional [basepoint] argument draws a rubber band from the base point to actual crosshair position. The function returns the point or nil if no points are given.

(CAD-HATCH objects [spacing [radians [crosshatch [basepoint [color]]]]] )

The function create a hatch from other geometries.

<objects> This is a list of objects. If no other arguments are given then the type of hatch will be a solid.

<spacing> If the value is zero then the type of hatch will be a solid. If it is a value then this is the distance between the lines in the hatch.

<radians> This is the angle of the hatch. If this value is not given then the angle is 45 degree. The argument should be given as radians.

<crosshatch> Should be T or nil

<basepoint> One of the hatch line will go thru this point. If no argument is given or if the argument is nil then the function will use the centroid.

<color> The color of the hatch. If no argument is given then the color will be actual color in the system

Example:

(SETQ entities (CAD-SSGET))
(CAD-HATCH entities 5.0 2.0 nil nil 2)
(CAD-DELETE entities)

(CAD-IMAGE <filename> <pos> <xsize> <ysize>)

The function will insert an image on the screen.

<filename> must be a valid existing image file.

<pos> is a point the lower left corner if the image, for example '(100 100)

<xsize> and <ysize> can be nil or a value. If both are nil the image is inserted the same way as the Image command in IGEMS If you for example set xsize to 500 and ysize to nil the width will be 500 and the height will keep the aspect ratio of the image.

(CAD-INSERTFILE <filename> <blkname> <inspos> <userpos> <explode> <insposlowleft>)

<filename> The file to be inserted in current drawing.

<blkname> The preferred name for the blocks to be inserted. If not possible the blockname will be decorated to a unique name. If "*" is used then IGEMS will set an automatic name for the block.

<inspos> The position the file is inserted at. Either this is the lower left point inserted drawing or the inserted point stored in the drawing is depending on the parameter <insposleft>.

<userpos>

<explode> If this argument is not nil then IGEMS will explode the inserted blocks before the functions returns.

<insposlowleft> If this argument is non nil then the lower point of the inserted drawing is used as insertion points, otherwise the insertion point stored in the drawing (normally 0,0) is used as insertion point.

The function returns a list of objects that was inserted.

Tip* If <inspos> and <userpos> is nil automatic insertion is done with 10mm spacing

(CAD-LAYER <name> [color])

The function changes the active layer. If the layer does not exist then the layer will be created. The optional <color> command should be an integer from 1 to 255.

(CAD-LINE <start point> <end point> [color])

The function draw a line between the two points and returns the object name. The optional argument [color] should be an integer from 1 to 255. If the [color] argument is omitted then current color will be used.

(CAD-LINEPARAM <x> <y> <lineorgx> <lineorgy> <angle>)

The function will return the length P from the ORG to XY in the direction of A.

(CAD-MARKER <x> <y> [rgb-color])

Sets a visual marker at coordinate x,y. Also, optionally set the color of the marker to an RGB encoded 32-bit integer 00RRGGBB. Default color is white.

(CAD-MARKERS <x> <y> <index> [rgb-color])

Sets a visual marker at coordinate x,y. create up to 6 markers with index (0-5) Also, optionally set the color of the marker to an RGB encoded 32-bit integer 00RRGGBB. Default color is white.

(CAD-MIRROR <objects> <point1> <point2>)

The function mirror the <objects> thru <point1> and <point2>

(CAD-MOVE  <objects> <from> <to>)

The function moves the objects <from> <to>

(CAD-NEW [file name])

The function creates a new drawing. If the optional argument [file name] is given then this will be the default name of the drawing. If no name are given then the system will use a standard name like Noname1, Noname2 and so on. Note! The function do not create the file on the hard disk. It has to be saved as normal.

(CAD-OPEN <file name> [point])

The file must exist else you will have an alert box with a warning.

If the argument [point] is missing, the function returns nil. The drawing will be open in a new window.

If the [point] argument is mentioned and nil, the user must pick the insertion point. The drawing will be inserted in existing drawing. The function returns a list with all objects

If the [point] argument is a valid list, the drawing will be inserted at that position in existing drawing. The function returns a list with all objects.

(CAD-POINT <point> [color])

The function draws a point in current point style at the <point> argument. The optional argument [color] should be an integer from 1 to 255. If the [color] argument is omitted then current color will be used.

(CAD-POLYBEGIN)

Returns a new polyline object. See also the CAD-POLYPOINT and CAD POLYEND functions.

(CAD-POLYPOINT <object> <x> <y> [<x-center> <y-center> <ccw>])

Add a segment to the polyline created with the CAD-POLYBEGIN function. If the argument <x-center> and <y-center> is used then the segment will be an arc.

(CAD-POLYEND <object> <color> [close])

Finish the polyline object created with the CAD-POLYBEGIN and CAD-POLYPOINT functions. The function returns the object name.

The example below will create a polyline and then delete it.

(SETQ obj (CAD-POLYBEGIN))
(CAD-POLYPOINT obj 0 0) 		; startpoint of the polyline
(CAD-POLYPOINT obj 100 0 50 0 T) 	; adds a CCW arc segment to the polyline
(CAD-POLYPOINT obj 100 200) 	; adds a line segment to the polyline
(CAD-POLYPOINT obj 0 200 50 200 nil) ; adds a CW arc segment to the polyline
(SETQ handle (CAD-POLYEND obj 2 T)) 	; Finish the closed yellow polyline.
(ALERT "The polyline object will be deleted")
(CAD-DELETE handle)
(CAD-REGEN)

(CAD-RECTANGLE <lowerleft> <topright> [color])

The function draw a rectangle between the corner points and returns the object name. The optional argument [color] should be an integer from 1 to 255. If the [color] argument is omitted then current color will be used.

(CAD-REDRAW)

After you have added object you must run this function to have them visible on the CAD screen.

(CAD-REGEN [objects])

The function regenerates the objects in the [object list]. If the [object list] is omitted then all objects are regenerated.

(CAD-REPLACE <objects>)

This function must be used to update the UNDO function.

Replace an exist object with a new object.

(CAD-ROTATE <objects> <angle> <center>)

The function rotate the <objects> in an <angle> from the <center>.

(CAD-SENDTOBACK <objects>)

When using filled graphic objects (solids, images, filled parts and so on) and you have multiple objects that cover each other, then the graphical view can only show the objects that are on top. With this command you can send objects to back. See also CAD-BRINGTOFRONT

(CAD-SET <object> <key> <value>)

This function makes it possible to change information on different objects on the screen. If you have a list of object then you must check one object at a time. The function will return the value.

| Key | Returns |
| --- | --- |
| "COLOR" | Set the color of the object as an integer |
| "CUSTOMER" | Set the customer name as a string. This should p |
| "LAYER" | Set the layer name of the object as a string |
| "NAME" | Sets the name of the part |
| "QUANTITY" | Set the quantity of a part. If it's not a part then the function returns nil |
| "SELECTED" | Select object if the value is T. |

If you use CAD-SET function with a non-existing key then this key will be added to the object. We call this information XDATA. You can save strings on all kind of IGEMS objects. It’s a special handling for parts and parts definitions. If the key starts with a dot. Example: ".PAINTED" then the same XDATA will be added for all identical parts. If you add "PAINTED" it will be added to the selected part only. See also the CAD-GET function

(CAD-SSGET [mode] [pt1 [pt2]])

The function let the user select objects on the screen, if object are selected then the object names are returned in a list. The mode option has following alternatives.

If no argument is given then the user must select the objects

| Mode | Meaning |
| --- | --- |
| "L" | Return the last object added to the drawing database. |
| "C" | Use Crossing type and need two points. |
| "W" | Use Windows type and need two points |
| "X" | Take all objects on the screen |

(CAD-SCALE  <objects> <xscale> <yscale> <basepoint>)

The function scales the select objects in <xscale> and <yscale> around the <basepoint>

(CAD-SAVE <file name> [objects])

Save the objects specified as <objects> as a <filename>

(CAD-SAVEAS <filename>)

Save the drawing as the specified filename.

(CAD-SAVEIMAGE <objects> <filename> <width> <height> [border] [background])

The function creates an image. If no border is given the then the border is 3 pixels. If no background is given or zero then the background is transparent. The color is the same as the color number in the color command.

(CAD-TEXT <txt> <pos> <height> <angle> [color] [basepoint] [font])

Insert a text on the drawing. <txt> is the text to add,<pos> is the insertion point, <angle> is the angle in radians. The argument [color] is optional if it’s nil then actual color will be used.
The [basepoint] is a value that controls where the text base point is placed. If the argument is omitted or nil then default base point 9 will be used.
[font] is an optional argument for the text font default is "ISOCP" which is the built-in single font You can also use one of the installed windows font, for example "Arial".

(CAD-ZOOMCH <center> <height>)

The function centers the view at <center> and a viewport of height <height>.

<center> is a list of the point that will be the new center of the screen.

<height> is a value that describe the height of zoom in actual units.

(CAD-ZOOMEXTENTS)

Zooms out that everything on the screen will be visible

(CAD-ZOOMWINDOW <pt1> <pt2>)

This function zooms a window given by two points.

<pt1> and <pt2> is lists of two reals that describe the position in X and Y

(UC-GET <filename> <key> <default>)

With this function you can read values from a User Config File.

<filename> The argument must be a valid filename without path and extension. The file will be located in the UserConfigs folder and have the extension .XML

<key> The argument must be a string.

<value> the argument van be a value or a string

<default> If the key does not exist then the function will return this value.

(UC-SET <filename> <key> <value>)

With this function you can save values in a User Config File.

<filename> The argument must be a valid filename without path and extension. The file will be located in the UserConfigs folder and have the extension .XML

<key> The argument must be a string.

<value> the argument van be a value or a string

(TOOLS-CLEANUP <objects> <tolerance>)

returns a list of objects

Sheet value calculation script

The value

Following variables are available in the script:

| *SHEET-WEIGHT* | Weight in Kg. |
| --- | --- |
| *SHEET-AREA* | Sheet Geometry Area. The holes in the sheet is removed from the external area. The units are in m² |
| *SHEET-THICKNESS* | The Thickness of the material. The units are in mm. |
| *SHEET-DENSITY* | The weight of 1m³ |
| *SHEET-GROUP* | The name of the material group |
| *SHEET-QUALITY* | The name of the material quality |
| *SHEET-COST* | The cost per kilo |
| *SHEET-DX* | Sheet length in X units mm |
| *SHEET-DY* | Sheet length in Y units mm. |
| *SHEET-ISRECTANGULAR* | T or nil. |
| *SHEET-NAME | The name of the sheet as a string. |
| *SHEET-INFO* | The info on the sheet as a string. |
| *SHEET-REFERENCE* | The Reference on the sheet as a string |
| *SHEET-STORAGE* | The Storage on the sheet as a string. |
| *SHEET-SMALLSIZE* | The Minimum rectangle size in Sheet Inventory |
| *SHEET-MEDIUMSIZE* | The Medium rectangle size in Sheet Inventory |
| *SHEET-LARGESIZE* | The Large rectangle size in Sheet Inventory |

(SHEET-INVENTORY [<small> <medium> <large>])

Returns a list with the weight in kilo of each areas.

If the arguments are not given then the system uses the default values that can be found in the Settings option in the Sheet Inventory command.

Connection IGEMS/Organizer with external ERP

Installation and workflow

Today date 2015.05.14

- Download and install version R2015.2.692 or newer.

- Copy the file "OrganizerRegister.lsp" to folder "…/Shared/Algorithms/OrganizerRegister"

- Copy the file "OrganizerStart.lsp" to the folder "…/Shared/Algorithms/OrganizerStart"

- The best is to start with a complete new database. Delete the “Organizer.db” file located in the Organizer folder in Shared.

- When registering a finish job, just select one sheet at a time.

- Inside the files on the first line is a variable TARGETFOLDER
(SETQ targetfolder "C:/ERP/") ; Set this variable to the communication folder
Set this variable to a valid existing folder for the communication with the ERP.

- In the Various tab in the machine settings, you can activate a script that will be loaded every time you register a part and every time you start Organizer.

Registration new parts in IGEMS/Organizer

When you convert basic geometries (lines, arcs, circles) to a part than the part will also get some production properties.

Those properties is following: Name of the part, Quantity, Date and Customer name.

If the checkbox “Add to Organizer” is activated then the part will automatically be stored in a special folder on the computer and the other information in a SQL database.

At the same time the program will start a LISP script. This script will open an output registration file. The name of this file is OrganizerNewParts.txt. The Script will add information in this file. The file will have one line for each part that has been registered. The file contain following information.

<ID> <"Name"><Quantity><"Date"><"Customer"> <"Material"> <"Quality"> <Thickness> <Area> <Length> <Estimated time>

Each field in are separated by a space, strings are between quotation marks.

Example:

6 "PART332" 0 "YYYY.MM.DD" "VOLVO" "Aluminum" "Standard" 20.0 739300 3549.3 420.2

7 "PART327" 0 "YYYY.MM.DD" "SKODA" "Aluminum" "Standard" 20.0 543772 2345.1 398.7

8 "PART334" 0 "YYYY.MM.DD" "AUDI" "Stainless Steel" "Standard" 12.0 662634 4210.9 1214.9

Since the ERP system will be the master in the system the script will always set the quantity to zero. It will always be the ERP system that desire when something should be produced. The ID is the IGEMS identification number. This number must be saved in the ERP system.

The ERP-system will probably need a unique internal "Article number". It’s up to the ERP-system to rename the part name (above "PART332" "PART327" "PART334").

Register of the parts in the ERP system

It’s now up to the ERP system to open the OrganizerNewParts.txt file read the information and add information into the ERP system. When the information is stored, then the ERP system will delete the file. When IGEMS will add new parts again, it will be in an empty file.

Important about ID and ArticleName

The ID is the IGEMS identification number. This number must be saved in the ERP system.

The ERP-system will probably need a unique internal "ArticleName". It’s up to the ERP-system to rename the part name (above "PART332" "PART327" "PART334") to a correct name.

Organizer

In Organizer it’s possible to see information about all parts that are saved. The operator will use Organizer in a normal way and this document do not cover this workflow.

The field "Remain" are set to zero and there is nothing to produce.

Creating a cutting order in the ERP system

The ERP system is the master in the system. If something should be produced then this is a decision from the ERP. When the system generate a production order then the ERP write information in a file named OrganizerProduce.txt. This file contains following information:

<ID> <nil><Quantity><nil><nil> <nil> <nil> <nil>

Example:

6 "ArticleName" 55 nil nil nil nil nil

8 "ArticleName" 34 nil nil nil nil nil

The meaning of this instruction is:

Produce 55 copies of part ID 6

Produce 34 copies of part ID 8.

Other instructions

It’s also possible for the ERP system to change other data in Organizer.

<ID> <"Name"><Quantity><"Date"><"Customer"> <"Material"> <"Quality"> <Thickness>

In the example above the correct "ArticleName" will be transferred to Organizer.

If the ERP system send any other information than nil then the part information in organizer will be updated with that information. 
It’s an extra possibility with the quantity instruction. By adding a plus (+) or minus (-) sign before the value the ERP system can add or reduce the number of parts that already are stored in organizer (incremental mode).

6 "ArticleName" +55 nil nil nil nil nil

8 "ArticleName" -2 nil nil nil nil nil

Receiving the production order in Organizer

Every time the operator start Organizer the program will look for the file OrganizerProduce.txt. If this file exists then the database is updated. When the database is updated then the file will be deleted. Organizer have now the updated information from the ERP system

(You can see that ID 6 and 8 now have parts to produce)

The operator will always work with Organizer in normal way. The connection with the ERP system is completely invisible.

When setting the profile to INSERT and press the Insert button the parts will be inserted in IGEMS.

Registering that parts has been cut

You can nest and add the toolpath and make the CNC-file in normal way.

To register that the part has been included in a toolpath, press the “Register button”

This will show the “Register” dialog box. Here you can see short information about each part. If you accept then the organizer count down the number of copies to produce. The event will also start a script that will report to the ERP-system that the part has been added to a job. This script will create a text file with the name OrganizerRegister.txt

The file has following format:

<ID> <ArticleName><CountDown> <"Name of CNC-file"> <"Name of sheet">

Example

31 "ABC1" 1 "100.cnc" "Sheet-10"

33 "ABC2" 1 "100.cnc" "Sheet-10"

32 "ABC3" 7 "100.cnc" "Sheet-10"

30 "ABC4" 1 "100.cnc" "Sheet-10"

29 "ABC5" 3 "100.cnc" "Sheet-10"

The ERP system can use either IGEMS-ID or ArticleName as identification for count down.

When the ERP system has imported the information, then the ERP system should delete the file.

Final

The ERP-system is the master in the system.

The Organizer will be used locally for the operator, to handle what parts should be cut together and to organize the work for the day.

The organizer register script

This is an example of a register script:

(SETQ targetfolder "C:/ERP/") ; Set this variable to the communication folder

(IF (= *OLDDBID* -1)

(PROGN

(SETQ fw (OPEN (STRCAT targetfolder "ORGANZERNEWPARTS.TXT") "a"))

(WRITE-LINE (STRCAT (ITOA *DBID*) " "

(CHR 34) *NAME* (CHR 34)

" 0 "

(CHR 34) (ITOA (CAR *DATE*)) "." (ITOA (CADR *DATE*)) "." (ITOA (CADDR *DATE*)) (CHR 34) " "

(CHR 34) *CUSTOMER* (CHR 34) " "

(CHR 34) *MATERIAL* (CHR 34) " "

(CHR 34) *ALLOY* (CHR 34) " "

(RTOS *THICKNESS* 2) " "

(ITOA *AREA*) " "

(RTOS *LENGTH* 1 T)) fw)

(CLOSE fw)

;(CAD-SET *object* "QUANTITY" 0) ; If you want to set the quantity to zero on the drawing

…                                                       .;(CAD-DELETE *object*) ; If you want to delete the part on the drawing

(SQL-SET *DBID* "quantity" 0)

(SQL-SET *DBID* "remains" 0)

)

)

(IF (> *PRODUCED* 0)

(PROGN

(SETQ fw (OPEN (STRCAT targetfolder "ORGANZERREGISTER.TXT") "a"))

(WRITE-LINE (STRCAT (ITOA *DBID*) " "

(CHR 34) *NAME* (CHR 34) " "

(ITOA *PRODUCED*) " "

(CHR 34) (CAR *NCFILES*) (CHR 34) " "

(IF *SHEETS* (STRCAT (CHR 34) (CAR *SHEETS*) (CHR 34))  "nil")) fw)

(CLOSE fw))

)

The Organizer start script

(SETQ targetfolder "C:/ERP/") ; Set this variable to the comunication folder

(DEFUN DateToList (datestr / year month day) ; The function convert a datestr example: "2015.3.15" to a list (2015 3 15)

(SETQ year "" month "" day "" q 0)

(FOREACH s datestr

(COND

((= s ".") (SETQ q (1+ q)))

((= q 0) (SETQ year (STRCAT year s)))

((= q 1) (SETQ month (STRCAT month s)))

((= q 2) (SETQ day (STRCAT day s)))

)

)

(LIST (ATOI year) (ATOI month) (ATOI day))

)

(IF (FILE-EXISTS (STRCAT targetfolder "ORGANIZERPRODUCE.TXT"))

(PROGN

(SETQ fr (OPEN (STRCAT targetfolder "ORGANIZERPRODUCE.TXT") "r"))

(WHILE

(SETQ txt (READ-LINE fr))

(SETQ txtlist (READ (STRCAT "(" txt ")")))

(SETQ id (CAR txtlist))

(IF (SETQ name (NTH 1 txtlist)) (SQL-SET id "name" name))

; Se if there is a + or - sign

(SETQ fnuts 0)

(FOREACH tk name (IF (= tk (CHR 34)) (SETQ fnuts (1+ fnuts))))

(SETQ tk (ASCII (STRTRIM (STRSUB txt (+ (STRLEN (ITOA id)) (STRLEN name) 4 fnuts)))))

(SETQ absolut (IF (OR (= tk 43) (= tk 45)) nil T))

(COND

((NOT (NTH 2 txtlist)) nil)

(absolut

(SETQ quantity (NTH 2 txtlist))

(SQL-SET id "quantity" quantity)

(SQL-SET id "remains" quantity)

(SQL-SET id "produced" 0))

((NOT absolut)

(SETQ addons (NTH 2 txtlist))

(SQL-SET id "quantity" (+ (ATOI (SQL-GET id "quantity")) addons))

(SQL-SET id "remains" (+ (ATOI (SQL-GET id "remains")) addons)))

)

(IF (SETQ datum (NTH 3 txtlist))

(PROGN

(SETQ datelist (DATETOLIST datum))

(SQL-SET id "productiondate" (SQL-DATE datelist))))

(IF (SETQ customer (NTH 4 txtlist)) (SQL-SET id "customer" customer))

(IF (SETQ material (NTH 5 txtlist)) (SQL-SET id "material" material))

(IF (SETQ alloy (NTH 6 txtlist)) (SQL-SET id "quality" alloy))

(IF (SETQ thickness (NTH 7 txtlist)) (SQL-SET id "thickness" thickness))

)

(CLOSE fr)

(FILE-DELETE (STRCAT (STRCAT targetfolder "ORGANIZERPRODUCE.TXT")))

)

)

Following variables are automatically set every time a part is registered and the OrganizerRegister.LSP file is loaded.

| *ALLOY* | The Quality of the material |
| --- | --- |
| *AREA* | The area of the part in mm2 |
| *CUSTOMER* | The customer name |
| *DATE* | A list with 3 integers containing (Year month day) |
| *DBID* | The database ID that the part will have in Organizer. |
| *LENGTH* | The total circumference of the part |
| *MACHINE* | The name of the machine |
| *MACHINETYPE* | The machine type: 0=AWJ, 1= Pure water |
| *MATERIAL* | The material |
| *NAME* | The name of the part as a string as defined in the Create part command |
| *NCFILES* | A list of NC-files |
| *OBJECT* | The object id in the CAD system. This can be used to delete the part from the drawing or modify the quantity |
| *OLDDBID* | If it’s a new part that has not been registered in organizer then this value is always -1. Else the value is the same as *DBID* |
| *PRODUCED* | Number of parts that has been produced. If it’s a registration of a new part then this value is 0. |
| *QUANTITY* | Quantity |
| *SHEETS* | A list of sheets |
| *THICKNESS* | Material thickness |
| *XSIZE* | The external size in X of the part. |
| *YSIZE* | The external size in Y of the part. |
| *CUTTIME* | Roughly estimated cut time of the part. |
| *WEIGHT* | The weight of the part. Only correct in flat material. |

(SQL-DATE <year month day>)

The date format in the SQL server is different from normal way of entering dates. To convert to a SQL-DATE format you must use this function. The function takes a list of three integers and returns a string with the SQL-DATE format. This format must be used when modifying dates in the database. Example

(SQL-DATE (LIST 2015 04 15))	returns  "42109.0"

Note! The function SQL-STRINGTODATE makes an opposite conversion.

(SQL-GET <id> <key>)

This function returns the value from the key column on the id row in the database. Se list later in this chapter. All values will be returned as strings.

(SQL-RUN <string>)

This function execute a SQL command and return the value.

(SQL-SET <id> <key> <value>)

The function set values in the SQL database. The value do not have to be strings, since this will be converted automatically.

ID is the database id of the part,

KEY is one of the keys (se later in this chapter).

VALUE the value that should be set.

(SQL-STRINGTODATE <SQL-DATE>)

The date format in the SQL server is different from normal way of entering dates. To convert the special format to a list of three integers (year month day) use this function. 
Example:

(SQL-STRINGTODATE "42109.0")	returns (2015 4 15)

Note! The function SQL-DATE makes an opposite conversion

Standard key field in the Organizer database

| Key | Description |
| --- | --- |
| NAME | The name of the part |
| CUSTOMER | Name of the customer |
| SIZEX | The size in X of the part |
| SIZEY | The size in Y of the part |
| AREA | The area of the part in mm2 or inch2 |
| WEIGHT | The weight of the part as Kg or lb |
| LENGHT | The total circumference of the part. |
| CONTOURS | The number of contours of the part |
| MATERIAL | The material name |
| QUALITY | This is the same as alloy that are used in other functions |
| THICKNESS | The material thickness |
| MACHINE | The name of the machine |
| MACHINETYPE | Machine type default =0 = AWJ |
| QUANTITY | The number of parts to produced |
| PRODUCED | The number of parts that has been produced |
| REMAINS | The number of parts that remains to produce |
| PRODUCTIONDATE | This is the date that was entered in the Create part command. |
| REGDATE | This is the date that the part was registered in the database. |
| NOTE | The Note in the Organizer |
| RELEASED | The value can be 0 or 1 |
| All user fields that has been added in organizer can be used |  |

ILISP functions for the CAM module

(CAM-TIME4LINEAR <distance> <max speed> <acceleration>)

The function returns the time in seconds for a motion in entered <distance> in <max speed> with acceleration and retardation in <acceleration> as mm/s².

(CAM-TIME4ROTATE <radians> <max speed> <acceleration>)

The function returns the time in seconds for a motion <radians> rotation with <max speed> as degrees per second with <acceleration> in degree/sec²

NOTE! See also CAM-MAXFEED4ROTATE

(CAM-AWJPAR <quality> <row>)

This function will read values from the AWJ database for actual loaded material.

Example:
(CAM-AWJPART 3 1)	returns 228.3
(CAM-AWJPART 2 5)	returns 6

(CAM-CANVAS <part>)

Create a canvas image

(CAM-CONVERTRAPIDS <parts> <cutfeed> <rapidfeed> <maxdist>)

This function will convert all rapids in <parts> that are shorter than <maxdist> to normal cutting with the speed of <rapidspeed>. All other cutting will be done by <cutspeed>.

(CAM-CONTOURMULTI <list of parts>)

This function will add toolpath on all parts in the list. It will always use the current settings of the Contour command.

<list of parts> This is normally the list that are returned from CAM-CREATEPART. The argument can also be a part

(CAM-CUTORDER <list of parts>)

This will create a cut order. It will use the settings in the cutorder dialog.

(CAM-CUTORDERSHEETS <list of parts>)

This will create multiple cut orders. One for each sheet. Will return a list of cut order entities.

(CAM-CUTORDERRAW <list of parts>)

This will create a cut order, the parts will be selected as the list is sorted.

(CAM-CREATEPART <objects> <other objects> <name> <quantity> <date> <customer> <ext qua> <int qua> <other qua> <geometry opt > <curve fit>)

This function create a part in IGEMS and returns a list of created parts.

<Objects>	A list of objects that should be used for the parts

<Other objects> A list of objects that should be used as other objects.

<Name> A string that describes the name. The name can have "#" and use automatic numbering.

<Quantity> the number of parts that should be cut

<Date> if you want to use the date if the day, then set this variable to nil. If you want to set another date then use a list of three integers that describe the year month and day. Example: (2015 3 27)

<Customer> a string with the name. If no customer should be given then use an empty string "".

<ext qua> The quality for external quality between 1 to 5. 1 is X-rough and 5 is X-fine.

<int qua> The quality for internal quality between 1 to 5. 1 is X-rough and 5 is X-fine.

<Other qua> The quality for other objects between 1 to 5. 1 is X-rough and 5 is X-fine.

<Geometry optimizing> T if geometry optimation should be used else nil

<Curvefit> T if Curvefit should be used else nil.

(CAM-CREATESHEET <name> <xsize> <ysize> <collar>)

Creates a rectangular sheet at with <xsize> and <ysize> and <collar>

(CAM-DISCONNECT <part> <breakup>)

The function will disconnect the toolpath on the part and create disconnected toolpaths. The function will return T if it's OK else nil.

If the argument <breakup> is not nil, then it will be separate disconnected toolpaths for each geometry on the part. If the argument is nil then all geometry's will be located in the same disconnected toolpath.

(CAD-GETBOTTOMARC from mid to oldvector $vector)

The function returns a list of the center and the midpoint of the bottom arc from a 3D-5X part.

<from> is the start point of the top arc in x y z

<mid> is the midpoint of the top arc in x y z

<to> is the endpoint of the top arc in x y z

<oldvector> id the $vector from previous postprocessor function

<$vector> is the jet vector in the ARC function in the postprocessor.

(CAM-GETMACVAR <variable>)

This function returns the value from the machine configuration. If the variable does not exist, then it will return nil.

(CAM-GETMATVAR <variable>)

This function returns the value from the material settings. If the variable does not exist, then it will return nil.

(CAM-INDEXTOOBJ <index>)

Returns an object name that can be used by CAD-GET to get information about the part while postprocessing. The index number must come from the cut order in the postprocessor

(CAM-LISPGEOMB64 <point list> <trans>)

This function returns a Base64 string that can be used as a sheet geometry in IGEMS cnc.

Each list has following information (X Y bulge), if the bulge is 0 it will be a line, if bulge is 1 it will be a 180-degree arc. If the argument <trans> is T then the X, Y coordinated is switched according to the actual axis direction of the machine. (The table layout in machine setting).

Example:

(SETQ sheetlist (LIST (LIST '(450 0 0) '(450 450 0) '(0 450 0) '(0 0 0)) (LIST '(350 100 0) '(350 350 0) '(100 350 0) '(100 100 0))))

(WRITE (STRCAT "$SHEETGEOMETRY:" (CAM-LISPGEOMB64 sheetlist T)))

The INC file will look as follows:

$SHEETGEOMETRY:AgAAAAQAAAAAAOFDAAAAAAAAAAAAAOFDAADhQwAAAAAAAAAAAADhQwAAAAAAAAAAAAAAAAAAAAAEAAAAAACvQwAAyEIAAAAAAACvQwAAr0MAAAAAAADIQgAAr0MAAAAAAADIQgAAyEIAAAAA

In IGEMS cnc HMI the sheet will look as follows:

(CAM-LOADMAC <machine>)

The function load actual machine with latest used cutting parameters.

From version R2017.2.1317 it also set all LISP variables that belong to the machine.

(CAM-LOADMACMAT <machine> <group> <quality> <thickness>)

(CAM-MATINFO)

The function returns a list with ("NAME" "QUALITY" "THICKNESS" "DENSITY")

(CAM-MACINFO)

The function returns a list with ("NAME" "TYPE")

(CAM-MAXFEED4ROTATE <radians> <max speed> <acceleration>)

The function returns the max reacted speed in degree per second for a motion <radians> rotation with <max speed> as degrees per second with <acceleration> in degree/sec²

Note! See also CAM-TIME4ROTATE

(CAM-OPENSHEETFILE)

Open matching sheet file.

(CAM-PARTGEOMB64 <$partid >)

This function will returns a Base64 string that describes the geometry for the HMI in IGEMS cnc.

The $partid argument should be the same as the $partid variable that are visible in the PART function in the postprocessor.

Example:

(WRITE (STRCAT "$PARTGEOMETRY: " (CAM-PARTGEOMB64 id)))

(CAM-PARTDATA <object>)

Returns a list (cutting time and cutting length)

(CAM-PARTTEXT <text height> <flag> [parts])

This function adds part text to parts on the drawing.

<text height> set the size of the text.

<flag> If the value is 0 then the text will be the name of the part. If the value is 1 the text will be an increasing number for each part.

The argument [parts] can be a part object or a list of objects. If the argument [parts] argument is omitted then the function will add part text to all parts on the drawing.

The example below will place

(SETQ objects (CAD-SSGET))

(CAM-PARTTEXT 10 0 objects)

(CAM-PARTIMAGE <objects> <filename><width> <height> [border] [backcolor])

The extension of the file name must be included and sets the type of file. PNG, JPG and BMP is suppoted

Width, height, border is size in pixels.

(CAM-PROCESS <cutorder><filename><option>)

The command run the postprocessor and creates a filename. The option should be 6. More documentation will come on different options.

(CAM-REGISTER <list of parts>)

Register the parts in Organizer and returns the database id as a list

(CAM-SAVEMAT)

This function can be used to save changes in actual material. It is used together with the function 
CAM-SETMATVAR

(CAM-SAVEMAC)

This function can be used to save changes in actual machine. It is used together with the function CAM-SETMACVAR

(CAM-SAVEPREPIMAGE <filename>)

This function saves the picture shown in the postprocessor window. You can include a full path and extension in the filename. The extension can be JPG, PNG, BNG. If no extension is given the default type is PNG. The function returns T if the operation success. In other case no. This function will only work if you postprocess from the normal postprocess command.

(CAM-SETMACVAR <variable> <value>)

This function can set any variable in the machine configuration. When using the function, the changes are only saved in the memory. By using the function (CAM-SAVEMAC) the changes are saved permanently on disk. The actual variable name must exist and it must be correct variable type. (STR, INT, REAL).

(CAM-SETMACVAR "*3D-5X*" "H30210 H20310")

(CAM-SAVEMAT)

(ALERT "Technology saved")

(CAM-SETMATVAR <name> <value>)

This function can set any variable in the material configuration. When using the function, the changes are only saved in the memory. By using the function (CAM-SAVEMAT) the changes are saved permanently on disk.

Example
(CAM-SETMATVAR "*Technology*" "H30210 H20310")

(CAM-SAVEMAT)

(ALERT "Technology saved")

(CAM-SETZEROPOINT <cutorder> <pt>)

The command modifies the zero point of the cut order. The coordinate is in the coordinate system from the CAD-system.

(CAM-SHEETGEOMB64)

This function generates a base64 string of the sheet geometry that can be used in IGEMS cnc.

Example:

(WRITE (STRCAT "$SHEETGEOMETRY: " (CAM-SHEETGEOMB64))))

(CAM-SORTIDTOPART <$sortid>)

The function is to use inside the postprocessor PART function. It takes the variable $sortid that are an integer and returns the entity name for the part. The following example will select the part.

(SETQ obj (CAM-SORTIDTOPART $sortid))

(CAD-SET obj "SELECTED" T)

(CAM-TOOLSET <part object> <toolbit> <distance>)

The function controls the multiple tool setting on a part. It returns the object of the new modified part.

(CAM-TVL <thickness> <speed>)

The returns a list with TAC VOC LAG for existing material. Note the values is in mm.

(CAM-UNIQUEPARTS <objects>)

The <objects> can be one object or a list of objects. The function will return a list of unique parts. Other objects than parts will not be member in the list.

Example:

If you have 10 parts A and 5 parts B and 1 part C then your objects list will contain 16 parts. The function will return a list of three parts (A B C)

(CAM-VOC <thickness> <speed>)

The function returns the actual VOC correction for actual material in mentioned <thickness> and <speed>

(TOOLS-CLEANUP <objects> <tolerance>)

The command runs the cleanup command on the objects in the given tolerance in mm.

(CALCAWJSPEED <quality> <thickness> <low speed>)

This function returns the speed from an already loaded machine/material setting.

Quality is an integer from 1 (X-rough) to 5 (X-Fine)

Thickness can be any thickness, if it is not mention in the material database it will interpolate or extrapolate between different existing thicknesses to find a theoretical value.

Low speed is nil or a non nil value.

ILISP functions for the Automatic Nesting module

(NEST-CREATE [part distance])

This function creates a nesting object and returns an argument to be used on the other nesting functions. The optional argument part distance can be entered. If not then the nesting will use the distance specified on the material.

Example:

(SETQ nestobj (NEST-CREATE 4.5))

(NEST-ADDPART <nestobj> <part> [quantity])

The function adds a part to the nesting. The optional argument quantity can be used. If not used the number of parts entered in the part will be used.

(NEST-ADDSHEET <nestobj> <sheet>)

The function adds a sheet to the nesting.

(NEST-SET <option> <value>)

The option argument must be a string the value argument have different objects.

| Option | Value |
| --- | --- |
| "PARTDISTANCE" | A real or integer that describe the distance |
| "CORNER" | An Integer with following meaning: Bottom Left = 0 Top Left = 1 Bottom Right = 2 Top Right = 3 by Machine = 4 |
| "DIRECTION" | An integer with following meaning: Automatic = 0 Horizontal = 1 Vertical = 2 Corner = 3 |
| "MIRROR" | An integer with following meaning: By Part=0 (not yet implemented in part) Yes=1 No=2 |
| "FILLERSAFTERFRONTLINE" | An integer with following meaning: 0=False 1=True |
| "MULTITORCHUSE" | If the nesting should be done with multiple jets: 0=False 1=True |
| "MULTITORCHMODE" | If the nesting should be done with a constant or dynamic tool distance: 0=Constant 1=Dynamic |
| "MULTITORCHQTY" | An integer with number of jets that should be used for the nesting. |
| "MULTITORCHDIST" | The distance between the jets if "Multitourchmode" is constant. |
| "MULTIPLICITY" | If the multiplicity flag should be activated. 0=False 1=True |

(NEST-RUN <nestobj> <seconds>)

The function runs the automatic nesting for entered number of seconds. The function returns a list of lists where the first object is the sheet and the other is the parts on the sheets.

Example:

(SETQ objects (NEST-RUN nestobj 10))
(PRINC (TOSTR objects))
(
(sheetobject partobject partobject partobject partobject)
(sheetobject partobject partobject)
(sheetobject partobject partobject partobject partobject partobject)
)

Lead script

By holding down the CTRL the same time as you press the Favorites button you can edit the script. The script may only be changed if you have experience of LISP programming.

Variables that holds information

This variables is always set when the script is in use.

| *LEAD-YSIZE* | Real | The size in Y in mm |
| --- | --- | --- |
| *LEAD-HOLE* | Bool | T if it's an internal geometry else nil |
| *LEAD-CLOSED* | Bool | T if it's a closed geometry else nil. |
| *LEAD-LENGTH* | Real | The perimeter of the geometry in mm |
| *LEAD-AREA* | Real | The area of the hole if it's a hole else the area of the part. |
| *LEAD-XSIZE* | Real | The size in X in mm |
| *LEAD-CIRCLE* | Bool | T if it's a circular hole else nil |

You can also use all machine and material variables in the script.

(LEAD-SET type name dynmax dynmin inradius inangle outlength outradius outangle overcut piercingtype)

Type : 0=Outer 1=Inner 2=Corner 3=Alternative

If dynmin < dynmax then the lead will dynamic

piercingtype 0-6

inangle och outangle are in degree.

Old reports system (before 2018.4)

Overview

When running a report in IGEMS a template report file is populated with data from IGEMS and generates a document that can be save and/or printed. The template files have extension .tig. For every type of report you can make your own customized reports of that type.

The reports are located in the IGEMS shared folder under the folder Reports. Each type of report has its own subfolder.

- CostEstimate

- Inlay

- Nest

- Organizer

- Postprocessor

- SheetInventory

These are the different types of reports shipped with IGEMS.
In each subfolder all the reports of that type are placed.
A report consists of the following files.

- filename.tig

  - This is the report template

- filename.xml

  - This is a settings file with all LISP tags and other settings

- filename.lsp

  - This is the startup-script that can be attached to a report

The report system has two states. The generated report and the edit state.
To switch between the generated report and the edit state press IGEMS/Toggle report view or press F5. 

IGEMS is shipped with a number of standard reports. To customize a report you need to manipulate the report template. It is not possible to change the standard reports. To create a custom report you can do that in several ways.

Create a custom report using the standard report as a template.

- Run a standard report and press F5 to switch to edit mode.

- Select File/Save as... and save the report template in the same folder under another name.

- You can now start editing your own custom report.

- When running the report from IGEMS a dialog will appear and prompt you to select which report to run.

Create a custom report from scratch

- Run a report and press F5 to switch to edit mode.

- Select File/New to create a new document.

- Select File/Save and save the file in the same folder.

APP reports

Reports can also be generated from the APP section in IGEMS. Here you can make your own totally custom reports. You have to write a LISP app to generate the report data and write a report template file. This will be covered in a separate section below.

Editor overview

The report editor is a simple word processor with built in report features. You can do most common word processing tasks. The generated report can be manually edited before saved or printed. You can export the document to several different formats including .rtf, .doc, .docx, .html, .pdf, .rtf, .xml, .txt. The normal word processing features will not be covered in this document, please contact our support if you need help. The exception is tables. Tables are used extensively when writing report templates so a few notes are in order.

Tables

To insert a table in a document select Table/Insert/Table from the menu.
Type in the number of rows and columns and press ok.
A table is now inserted. The table has all grid lines ON as default. To change the properties of a table place the cursor in a table cell and select Table/Select/Table and then select Table/Properties...
A dialog box is presented where you can alter the appearance of a table or individual cells of a table. You can set which grid lines of a table or cell are visible and change the background color etc... In the second tab “Size and formatting” there are two check-boxes that are important.

- Allow row to break across pages

  - If this box is checked a table row can be broken on multiple pages. If you want a table row to stay on one page you must uncheck this box.

- Repeat as header row at the top of each page

  - If you select a row in a table and check this box that row will be repeated at the top of each page if the table spans multiple pages.

Tags

A tag is a text field in the template that will be replaced with data when generating the report.
When you move the mouse over a tag the cursor will change to a “hand”. This indicates that the tag has special meaning. You can change the displayed name of the tag, you can delete a tag and you can edit a tag.

Double input position. When placing the cursor in a tag the tag is highlighted. As long as the tag is highlighted you can edit the displayed name of the tag. When the cursor is at the start or end of a tag you can press the Left Arrow key and Right Arrow key to leave the tag. This is called a double input position. 

When the report is generated the tag is replaced by the data in the tag. The formatting of the tag is retained in the report. All properties of the tag like Font, Size, Color, Bold, Italic, Alignment etc is retained.

Example: We have a tag called Name with data IGEMS Report
Name -> IGEMS Report

Multiple data tags

A tag can contain multiple data. For example if you run the CostEstimate report the tag Part.Name will contain the name of all the parts. If you place the tag in the main text of the document it will be replaced with the data for the first part. The only way to display all data of a tag is to place it in a table. If a tag is placed in a table the table will expand and show all data for that tag.
Example:

| Name | Quantity | Cutting cost | Material cost | Total cost |
| --- | --- | --- | --- | --- |
| Name | Quantity | CutCost | MatCost | TotalCost |

The first row is normal text and does not contain any tags.
The second row contains tags with multiple data.
When running the report this table will expand and will look like this.

| Name | Quantity | Cutting cost | Material cost | Total cost |
| --- | --- | --- | --- | --- |
| PART-36 | 5 | 87,93 kr | 106,70 kr | 973,16 kr |
| PART-37 | 12 | 113,01 kr | 447,32 kr | 6.723,90 kr |
| PART-38 | 2 | 148,44 kr | 289,35 kr | 875,58 kr |

Adding tags

To add a tag you must be in edit mode.
In edit mode you will have a tree structure on the right side in the editor.
This tree view contains all the data tags available for the report.
By double clicking on a tag in the tree view the tag is inserted in the document.

Deleting tags

To delete a tag just delete the whole tag using the Del or Backspace keys.

Editing tags

To edit the properties of a tag you double click the tag.
When double clicking a dialog will appear where you can change the properties of the tag.
What properties you can change depends on the type of the tag.
When editing properties of a tag that affects only the tag edited in the document.

Tag types

String

The tag is replaced by a string

Int

The tag is replaced by an integer

Currency

The tag is replaced by a currency using the currency settings in Windows

Double

The tag is replaced by a double

You can change the display name, number of decimals and if the unit should be visible.

Date

The tag is replaced by a date in the Short date format defined in Windows.

Time

The tag is replaced by a time in HH:MM:SS format

You can change to Decimal hour and then the time is shown in HH.## format.

Example 17:03:02 will be displayed as 17.05

Image

The tag will be replaced by an image

When an image tag is placed in the report you can use the grips to resize it to fit your needs.

LISP

A LISP tag or user defined tag is a special tag that the user creates. Above the tree view you have a + button called “Add user defined tag”. If you press this button a dialog appears where you can create your own tag and write a LISP script. LISP tags will be covered below.

Special tags

Special tags are placed under the IGEMS meny item. These tags are not editable.

| Tag |  |
| --- | --- |
| Date | Will be replaced by the current date |
| Time | Will be replaced by the current time |
| Pages | Will be replaced by the total number of pages |
| Pagenumber | Will be replaced by the current page number |

LISP tags

Press the “Add user defined tag” above the tree view in edit mode.
Enter the name of the tag to create.
Now the Edit LISP dialog will appear.
Here you can write your custom tag and use all the tags already defined for the report.
By double clicking on a tag on the right the correct LISP syntax is inserted in the editor.
The value of the LISP script will be the value of the tag. By pressing the Eval button the value will be displayed. 

Examples:
(+ (TAG "Part.CutCost") 10)
This will add 10 to the CutCost of each part

(strcat (TAG "Part.Name") "#")
This will append a # after the name of each part

(setq a 100)
This tag will have the constant value of 100

(TAGCNT "Part.Name")
This will return the number of multiple data for this tag

(TAG "Part.Name" 3)
This will return the name of the fourth data for the tag “Part.Name” (zero based index)
When adding the index this tag will be constant and always return the same value.

(TAG "Part.Name")
This will return the name of the data for the tag “Part.Name”
This value will change when this tag is placed in a table

So, by using LISP tags you can make calculations to fine tune your reports.
The user defined tags are added to the tree view under the LISP branch.

Conditional sections

You can add conditional segments to a report using the following syntax.
{IF TAG … ENDIF}
If the tag after the {IF is evaluated to 1 the part between {IF and ENDIF} will be visible in the report.

Example:
{IF ShowInfo
Part.Name
Part.Customer
ENDIF}

Startup script

Every report can have a startup script connected to it. To edit or create a startup script select IGEMS/Startup script 
Now a LISP editor is opened where you can write a LISP application that will automatically be run before the report is generated.

In this LISP application you can create tags that can be used in the report. 
You can display a dialog box if you like.
You can also do anything else you like, for example export data or communicate with other systems.

In the startup script you have the following special functions for handling reports.
(TAG Name Index)

- Name is the name of the tag

- Index is the index for multiple data tags

- Returns the value of the tag at Index

(TAGCNT Name)

- Name is the name of the tag

- Returns the number of data rows for the tag

(SETTAG Name Value [Description])

- Name is the name of the tag

- Value is the value of the tag and can be of the following types

  - String

  - Double

  - Int

- Description is optional and if used will be displayed in the tree view when the tag is selected.

So, by using a startup script you can create a set of custom tags that can be used in the report. You can for example create a startup script that displays a dialog box where the user can enter a “Preparation cost”. This “Preparation cost” can then be used in the report.

To create multiple data tags you just call SETTAG multiple times with different Values.
Example
(SETTAG “Special.Users” “Bo”)
(SETTAG “Special.Users” “Inge”)
(SETTAG “Special.Users” “Nils”)
(SETTAG “Special.Users” “Patrik”)

Example.
This example will create a tag with a modified Part.CutCost that adds 10 to each current Part.CutCost.

(setq idx 0)
(setq rows (TAGCNT "Part.CutCost"))
(repeat rows
(SETTAG "Part.CutCost2" (+ (TAG "Part.CutCost" idx) 10))
(setq idx (+ 1 idx))
)

Printing

Every report template can have a printer connected to it. This can be any installed printer. To select a printer for the report select IGEMS/Set printer 
The printer you select here will be preset when printing and used if quick printing this report.
It will also be used if running a report for direct printing.

APP reports

It’s possible to create a totally custom report from the APP section in IGEMS.
You have the following special functions to use

(REPORT-INIT)

- Must be called before using any of the other functions!

(SETTAG Name Value [Description])

- Name is the name of the tag

- Value is the value of the tag and can be of the following types

  - String

  - Double

  - Int

- Description is optional and if used will be displayed in the tree view when the tag is selected.

(SETTAGIMAGE Tagname Filename Width Height)

- Tagname

  - The name of the tag

- Filename

  - The name of an image file (.png, .bmp, .jpg...)

- Width

  - Size of the image in the report

- Height

  - Height of the image in the report

(SETTAGEMF Tagname Ents SizeX SizeY Width Height)

- Tagname

  - The name of the tag

- Ents

  - An entity or list of entities to create the image of

- SizeX

  - Width of the EMF image

- SizeY

  - Height of the EMF image

- Width

  - Size of the image in the report

- Height

  - Height of the image in the report

- Use 300 as a start value for SizeX, SizeY, Width and Height

(REPORT-RUN template [resfile] [print])

- Template

  - The report template to run

- Resfile

  - a .doc or .pdf filename

  - If used the report will not be visible but automatically saved to resfile

- Print

  - If a resfile has been defined and print=1 then the report is automatically printed to the printer set in the template

Converting old reports

Just replace the old <<TAG>> syntax with the new tags in the report template file.

New report system (from 2018.4)

Report

When a report is generated it is opened in a word processor. Here you can modify the report using the ribbon style user interface.

You can then print and/or save the report.

Template

The report is generated using a report template. The template is populated with data from IGEMS.

To modify the template, you have to enter the “Edit template” mode.

Go to the menu File and select Edit Template or press F5. F5 toggles between Report-mode and Template-mode

In template mode you can edit the template that controls the report. After editing you press F5 or the button “Generate report” to come back to Report mode with the updated template.

To the right is a panel with the data from IGEMS.

Merge field

A merge field is a tag from the database that will be populated with data when running the report.

To insert a merge field just place the cursor in the document and double click on the field in the right panel.

To delete a merge field just select the field with the mouse and press delete or the “Delete field” button.

To change the properties of a field, like number of decimals etc… just double click on the field or press the “Properties” button.

Merge block/Repeating table

A merge block repeats the data in a table. To create a repeating table you have to select the table in the right panel and press “Insert table”.

When pressing “Insert table” you get a selection of what type of table to insert.

For normal repeating tables choose “Table”. The two other types of tables are described under “Labels”.

You are prompted to select the number of columns in the table. The first row is not a repeating block and should be used for setting the name of the table column but you can also delete it.

After a merge block table has been inserted you can add merge fields to it.

Tables video

Table design

It is possible to select the style of a table from a predefined list of designs. Just double-click on a table or choose Table layout/Design.

This will bring up the tables design dialog. Select the design you like and press OK.

- Remove formatting

  - The design will be removed from the table.

- Reset table

  - The table will be restored to a simple default style.

Special fields

Date

The date field inserts the current date.

Next

The next field is a special field that can be used to advance to the next row in a table. This is useful for label reports. See Labels for more information.

New template

Select this to create a new template.

Default template

Select this to insert the default template shipped with IGEMS.

Labels

You can generate reports for label printing.

Labels are created using tables. So, you select the table you want to use and press "Insert table" in the right panel.

To create labels for a normal printer with a document of labels choose "Labels". To create labels for a label printer with one label per page choose "Label printer".

When choosing "Labels" the Labels configuration dialog is shown.

This dialog sets up the labels.

Width and Height is the with and height of the label.

Spacing is the space between two columns.

Columns are the number of labels on one row.

Page margins sets up the margins of the page.

Predefined have a list of sample labels.

You can save and delete labels from the predefined list.

When you press OK one row of the table is inserted. Now you can add fields as usual.

Then you have to add the "NEXT" field after the last field in each cell except in the last column.

When choosing “Label printer” then a table is created with just one table cell fitting the whole page. In this case you should not use the “NEXT” field.

This will create a report that can be used for special label printers. Before creating this table, you should clear the report and select the correct paper-size.

Barcodes

We now support barcodes. To insert a barcode, go to the tab Insert and select the barcode button.

Here you select the type of barcode to use. Double click on the barcode to get to the barcode properties page. Here you can connect the barcode to the database or set the text for a non-connected barcode. You can also change the type of the barcode as well as other settings.

Note! Every type of barcode has certain limitations. It can be a minimum and maximum length of the text to encode for example. If the data from the database is not valid for a certain barcode then the barcode will not be encoded correctly. So, it is important to format the data correctly for the chosen barcode.

Here is a video showing how to use barcodes both for fixed values and for barcodes connected to the report data:

BARCODES

Barcode properties

Charts

It is possible to create charts that are driven by the data from IGEMS.

First you insert a chart.

Then select the chart and go to the “Chart layout” tab.

Here you select “Set data relation” and select the data.

See this video for how to create this report:

Chart example video

Script

The report can be extended and modified using the report script.  The script is connected to the report and is executed before the report is generated. So, this is the place to modify/extend the data that can be used in the report. This is also a good place if you need to export data to another system using for example XML.

There are a number of LISP functions that can be used. Note that all rows have a zero-based index, this means that the first row has index 0, the second row index 1 etc. All tables and columns are accessed using strings representing the name of the tables and columns. To edit the script, select the “Script” menu item. A new editor will open where you can edit the script. The script is saved with the template.

LISP functions

Here is a list of special report LISP functions.

(REP-GETTABLES)

Returns a list of all tables in the report database.

(REP-GETCOLUMNS <table string>)

Returns a list of all columns in a table.

(REP-GETCELL <table string> <column string> <row int>)

Returns the value in a cell. Row starts with 0 (zero-based).

(REP-ROWCOUNT <table string>)

Returns the number of rows in a table.

(REP-ADDTABLE <table string>)

Creates a new table to the report database.

(REP-ADDCOLUMN <table string> <column string> <value object> <description string>)

Adds a new column to a table. The object value decides the type of the columns and is also the default value for all new rows added.

(REP-ADDROW <table string>)

Adds a new row to a table. After adding a row, you need to call REP-SETCELL for each column to set the values of the cells.

(REP-SETCELL <table string> <column string> <row int> <value object>)

Sets the value of a cell.

(REP-SETUNIT <table string> <column string> <unit string>)

Sets the unit of a column.

(REP-SETDECIMALS <table string> <column string> <decimals int>)

Sets the number of decimals of a column.

(REP-SAVEXML <filename string>)

Saves the database to an XML file. If the file exists it is overwritten. If does not exist it is created.

(REP-DATE [(year,month,day)])

Returns a date object that can be used in the reports.

If the optional list of year,month,day is not specified the todays date is returned.

Example:

(REP-DATE (LIST 2018 8 14))

(REP-TIME <seconds >)

Returns a time object that can be used as a value in the reports.

(REP-ENTS)

Returns the list of entities that the user selected when running the report. This list is only valid for reports that are created in APPS.

(REP-AUTOSAVE)

Autosaves the report. Only works if a result file has been set.

(REP-AUTOPRINT)

Prints the report.

(REP-SETRESFILE <filename string>)

Sets the result filename.

Sample scripts

Below are some sample scripts showing some basic things you can do with the report script system.

Modify an existing value

; Sample modifying an existing field in the database
; Adding 10.0 to CCostTotal in table PartPP
(SETQ rows (REP-ROWCOUNT "PartPP"))
(SETQ idx 0)
(WHILE (< idx rows)
  (SETQ cost (REP-GETCELL "PartPP" "CCostTotal" idx))
  (SETQ cost (+ cost 10))

(REP-SETCELL "PartPP" "CCostTotal" idx cost
  (SETQ idx (+ idx 1))
)

Adding a new column to an existing table and set the value

; Sample adding a column to an existing table and setting its value
; Adding column "Color" to table "Parts" and set it to "RED" as default
; Then we loop all rows and set every second part to "YELLOW"
(REP-ADDCOLUMN "Parts" "Color" "RED" "The color of the part")
(SETQ rows (REP-ROWCOUNT "Parts"))
(SETQ idx 0)
(while (< idx rows)
  (REP-SETCELL "Parts" "Color" idx "YELLOW")
  (SETQ idx (+ idx 2))
)

Adding a new table

; Sample adding a custom table to the report
; Adding a Countries table and fill it in with some values
(REP-ADDTABLE "Countries")
(REP-ADDCOLUMN "Countries" "Name" "" "The name of the country")
(REP-ADDCOLUMN "Countries" "Code" "" "The country code")
(REP-ADDROW "Countries")
(REP-SETCELL "Countries" "Name" 0 "Sweden")
(REP-SETCELL "Countries" "Code" 0 "SE")
(REP-ADDROW "Countries")
(REP-SETCELL "Countries" "Name" 1 "United states")
(REP-SETCELL "Countries" "Code" 1 "US")
(REP-ADDROW "Countries")
(REP-SETCELL "Countries" "Name" 2 "France")
(REP-SETCELL "Countries" "Code" 2 "FR")

Setting the column format

; Sample adding a column to an existing table and setting its unit, decimals and value

; Adding column "Volume" to table "Parts"

(REP-ADDCOLUMN "Parts" "Volume" 0.0 "The volume of the part")
(REP-SETDECIMALS "Parts" "Volume" 4)
(REP-SETUNIT "Parts" "Volume" "m³")
(SETQ rows (REP-ROWCOUNT "Parts"))
(SETQ thickness (REP-GETCELL "Material" "Thickness" 0)) ; Thickness is in mm
(SETQ thickness (* thickness 1e-3)) ; Convert thickness to meter
(SETQ idx 0)
(WHILE (< idx rows)
  (SETQ area (REP-GETCELL "Parts" "Area" idx)) ; Area is in m2
  (SETQ volume (* area thickness))
  (REP-SETCELL "Parts" "Volume" idx volume)
  (SETQ idx (+ idx 1))
)

Exporting to XML

(REP-SAVEXML "c:\\temp\\report.xml")

Application reports

It is possible to create custom reports from the APP system.

Just create an APP and call this function:

(REPORT-CREATE  <entities> <template string> [resultfile string] [autosave bool] [autoprint bool] [silentmode bool])

The template must end with .TX. If this file does not exist it is created.

The database will be populated with the Report, Machine, Material, Parts, Organizer and Sheets data. No JOB info or COST info will be available. If you need this data you should make a custom template in the relevant report instead.

When making a report from the APPS you also has access to the (REP-ENTS) function so that you can customize the database with data not only from parts and sheets.

The normal workflow here is:

- Create an APP and call the REPORT-CREATE function with a template file.

- Then run the APP and an empty template will be created

- Edit the template and save.

- Then you probably want to edit the template script to do customization to the database.

- Edit the template again if needed.

- Done!

Sample: Coming soon…

Other postprocessor functions

(HMZ <2D-point> [<to-point>])

The function will return a list with X, Y and Z coordinates and a tool vector.

(HMZ (LIST 100 100))	returns ((100  100  2.0)  (0  0 -1))

If you enter two points than you will have a list of list of points and vectors

(HMZ (LIST x y) (LIST x y))	returns (((x y z)(xv yv zv)) ((x y z) (xv yv zv)) ((x y z) (xv yv zv)))

The accuracy number of points are depending of the accuracy of the height map.

(HMZARC <startpoint> <midpoint> <endpoint> <acc>)

The function will take the information from a circular movement and returns a list of X,Y and Z coordinates and a tool vector. The accuracy is always in metric.

(HMZARC <startp> <midp> <endp> 0.1)	returns (((x y z)(xv yv zv)) ((x y z) (xv yv zv)) ((x y z) (xv yv zv)))

(CIRCULARPIERCINGTIME)

This function will calculate the circular piercing time by using a formula based on material properties and cutting parameters.

(STATIONARYPIERCINGTIME)

This function will calculate the circular piercing time by using a formula based on material properties and cutting parameters.

(LINEARPIERCINGDISTANCE)

This function will calculate the circular piercing time by using a formula based on material properties and cutting parameters.

(CIRCULARPIERCINGDIAMETER)

This function will calculate the circular piercing time by using a formula based on material properties and cutting parameters.

(CIRCULARPIERCINGSPEED)

This function will calculate the circular piercing time by using a formula based on material properties and cutting parameters.

Interfacing ILISP with a .NET language

A brief introduction to interfacing ILISP with a .NET language

It is possible to call a function written in a .NET language from ILISP. This is done by giving a function the attribute: LispSharp("LISPNAME"), where lispname is the function name from lisp.

The following steps are needed to create a complete lisp function in C#:

1. Create a new project for a class library assembly in Visual Studio

2. Create a public static class, containing a public static function decorated with the LispSharp attribute

3. Implement the function as needed and return a .NET value casted to an object:

null is used to return nil

double are used to return floating point numbers

TypeT.T is used to return T (true Booleans are not used and does not exist in lisp)

Any other object can be returned as wished, but for example float won’t be recognized as a floating point number in the lisp engine

4. In a lisp app, do (LOAD "mylispdll.dll") to load the assembly IGEMS, where after the function(s) is available

Here is an implementaion of function MULT that multiplies two numbers in c#:

[LispSharp("MULT")]

public static object MyMultFunc(LispEngine L, Cons args)

{

double a = 0.0, b = 0.0;

L.GetNumberArg(ref args, ref a);    // fetch first argument

L.GetNumberArg(ref args, ref b);    // fetch second argument

L.CheckArgSentinel(args);   // throw an error if there are more arguments

return (object)(a * b);

}

The class LispEngine has many utility functions for managing lisp lists as well as fetching arguments etc.

The most important ones, normally needed, is the Get...Arg(ref args, ref obj)

The class Cons is a standard LISP S-Expression, containing a CAR and a CDR as specified in any lisp internals documentation.

The object pointed to by CAR is in the ILISP case a general .NET object.

OEM-Labeling of IGEMS

You can change the look of IGEMS by modify or adding following files to the C:/Program/IGEMS_R2018/RES directory:

SPLASH.PNG

If this file exists then it will be shown when you start IGEMS. It will be shown 2 seconds while the program load into the memory.

SYSBUTTON.PNG

This file replaces the standard system button design.

I good size of this file may be approximately 64 x 15 pixels

.

SYSICON.ICO

This file replaces the standard system icon at the upper left corner.

PROGNAME.TXT

This file should only contain one line. This line will be at the title line.

The structure of the postprocessing (for internal use)

The postprocessing workflow

This information is relevant if you want to modify and understand the postprocessor technology.

- When you press one of these buttons on the console following will happen. This will set actual process mode.

- IGEMS2.LSP is loaded. This file contains all important functionality for the postprocessing.

- IGEMS will set initial global variables. These variables are mainly used to support the OPTIONS function. (The different variables are specified at the OPTIONS function).

- The postprocessor will be loaded

- IGEMS start the lisp function $INITIALIZE. This function will first load the variables from the machine and materials then starts the Options dialog.

In the Options dialog you can make a lot of settings. (You can configurate what settings you want to be shown in the Strategy command)

- The $initialize function will start the C# function CAM-INITIALIZE.

- The CAM-INITIALIZE will create the preparation file. The layout of the preparation file is depending on the settings in the machine settings and the settings in the Options dialog

- The CAM-INITIALIZE function starts the lisp function $MAIN.

- If needed the $MAIN function call for filename. It will also do a lot of calculations and set up many $variables that later will be used by the postprocessor. All needed variables and information are not directly presented in the preparation file. The $MAIN function will therefore pre-post the preparation file. This is done when the CHKALL function are called.

- IGEMS will ask for a filename (This will only happen when generate a CNC file)

- The functions in the postprocessor will be executed
The whole job consists of many small events of movements (RAPID, TOOLDOWN, PIERCING, LINE, ARC and other). IGEMS are executing the events in sequential order and call for corresponding LISP function in the postprocessor.

The preparation file

($ARC '(137.721099 116.904246 8) '(87.678185 116.904246 8) T 0 2863.7178843 2863.7178843 0.0 '(0 0 -8) 4.712389 0.0 '(0 0 0) 157.2144527)

($ARC <allpos> <cent> <ccw> <flag> <fstart> <fend> <delta> <vectorallrot> <length>)

| <allpos> | list | A list of three reals that describe the end point of the ARC in X, Y and Z. |
| --- | --- | --- |
| <cent> | list | A list of three reals that describe the center in X, Y and Z. |
| <ccw> | bool | T if the arc is CCW else nil |
| <flag> | flag | Flag is a bit code value with following meaning: 1=Belongs to a lead in movement. 2=Belongs to a lead out movement. 4=Belongs to a marking movement. 8=Belongs to a circular piercing. 16=Belongs to a loop segment. 32=Empty 64=If the Arc is generated by $CCIRCLE function. 128= If the arc belongs to special marking. |
| <fstart> | real | The speed at the beginning of the arc. Note! <fstart> and <fend> can only be different if "Speed interpolation by machine" is activated in the machine settings. Note that not all CNC system has support for speed interpolation. |
| <fend> | real | The speed at the end of the arc. |
| <delta> | real | Tangential changes in direction to next object. The value is in radians between +PI to –PI. If the value is negative than the cutting direction will be changed in CW direction, if its positive then it will be changed in CCW direction. The value will be 0.0 for the object that goes to A or B. The value will be 1.5701 for the object that goes to C, D or E |
| <vector> | list | This is the theoretical jet stream direction. The vector length is the same as the actual material thickness in actual tilt of the jet. |
| <allrot> | list | This is a list of three angles. The first two are the primary and secondary motor angle for the kinematic. The last value is the angle for the rotation axis. |
| <length> | real | This is the length of the object. We need this to calculate the correct speed in 6-axis cutting. |

($CHMODE <flag>)

Change of cutting mode

| <flag> | int | The flag has following meaning. 0=Measuring 1=Drilling 2=Marking 3=Pre-piercing 4=Cutting 5=Inkjet 6=Special marking 7=Measurepoints (for part positions) 8=Refpoints (for toolpath position) 9=Sensoring mode : When taking a height for drilling tools or special equipment. When this happen the $CHMODE are called from other functions. This is yet not handled from the preparation file. |
| --- | --- | --- |

($CUTOFF <type> <3d-point>)

This instruction will do a lot of things internally. Most important is to turn of the jet and move the jet up to rapid height.

| <type> | int | The argument has following meaning: 0=This is a 2D $cutoff that (can) moves the jet vertical. The values in the <3d-point> argument is not used. The height is coming from the strategy. 1=This is a 3D-5X $cutoff with a straight vertical lift (only Z will be used). 2=This is a 3D-5X $cutoff where the move out is not vertical. It will use X, Y and Z of the <3d-point>. |
| --- | --- | --- |
| <3d-point> | list | This is a 3D-point. It's only relevant in case that the <type> argument is 1 or 2 |

($CUTON <allpos> <side> <ptype> <qua><clen> <area> <geolen> <leadname> <nodename> <edge> <piercingpos> <delta> <speed> <vector> <flag> <maxslope><dir> <leadplan> <allrot><length><attrib>)

This instruction has the most argument and it will do a lot of things, like move the jet to correct position in XY, tilt the jet in piercing direction, move down to the material and make the piercing and the first linear part of the lead-in.

| <allpos> | list | This is the first connection point on the geometry, at the end of the lead-in. |
| --- | --- | --- |
| <side> | int | 0= The jet should be on the geometry. (G40) 1= The jet will be on the left side of the geometry. On the most CNC this is G41. 2= The jet will be on the right side of the geometry. On the most CNC this is G42, Note! If the machine settings are set to calculate the kerf compensation internally in the CAD/CAM or if the toolpath is from 3D-5X then G41/G42 will not be used. But the side <argument> is still 0,1 or 2. |
| <ptype> | int | This argument controls the type of piercing: 0= Blind lead. This means that the jet will be turned on at the end of the lead-in. 1= Linear piercing. This means that the lead-in will be used as a piercing movement. This is the fastest method if you want to use a lead-in. 2= Stationary piercing. This is not a good type of piercing but it is supported just because that some users still want to use this. 3= Circular piercing. 4= (Deprecated drilling option) In Revision 1 it was possible to drill each geometry before you cut each geometry. This is not possible any more. All drilling will now be done Partwise or Sheetwise. 5= Air start. This option is when the piercing starts at a position that already are cut. For example, if the position is already pierced in low pressure. 6= User. This option is not specified. |
| <qua> | int | This is the start cutting quality. The cutting quality can be from 1 (X-rough) to 4 (X-fine) |
| <clen> | real | Cutting length. The length of the lead-in and lead out is not included in this value |
| <area> | real | Area in square mm of the internal or external geometry. The value is 0 if not possible |
| <geolen> | real | Perimeter of the actual internal or external geometry. 0 if not possible. |
| <leadname> | string | The name of the lead. |
| <nodename> | string | The name of the node. The name is in many cases generated automatically and cannot be changed. |
| <edge> | int | -1 Means it’s an external contour 0 Means an open contour (slot or common cutline or marking) 1 Means it’s an inside contour |
| <piercingpos> | list | This is the piercing position, at the beginning of the lead-in. |
| <iskerfed> | bool | If it is T then IGEMS has already calculated the kerf compensation. |
| <speed> | real | The speed for the lead in. Note! This can be modified in IGEMS2.LSP |
| <vector> | list | This is the theoretical jet stream direction. The vector length is the same as the actual material thickness in actual tilt of the jet. |
| <flag> | int | - FLAG=A bit value that controls following 1: Tilted piercing 2: Tilted cutting (bevel mode is top, bottom or advanced) 4: Top bevel (Will later probably be removed) 8: Bottom bevel (will later probably be removed) 16: This is from 3D-5X 32: Directional bevel (will later probably be removed) 64: TAC can be used on this geometry. 128: Excepted from speed ramping. If force 3X cutting in 3D-5X then this value is not 1 and not 2. |
| <maxslope> | real | This is the maximum cutting angle for actual toolpath- |
| <direction> | real | The direction of the first movement on the geometry. |
| <leadplan> | list | This is a vector that describe the normal to the lead plan. This is used for making the circular piercing in a non XY-plan. Used by CPIERC-CIRCLE and CPIERC-LINE |
| <allrot> | list | Motor angle of A, B and C at the end of the lead-in (at the geometry) |
| <length> | real | This is the movement length of the lead-in. This is needed to calculate the correct speed when using rotation axis. |
| <attrib> | string | The argument is a text attribute that can be entered in the Edit toolpath. If no attribute is given the argument is an empty string "". |

($HOLE <allpos> <type><radius> <attrib> <quality>)

This instruction has 5 different options.

| <allpos> | list | A list of three reals that describe the position in X, Y and Z. |
| --- | --- | --- |
| <type> | int | This is the type of hole instruction. 1=Drilling is not used, it will directly use the $DRILLING instruction. 2=Marking 3=Circular piercing 4=Cutting (HQCH) 5=Stationary piercing 6=Loading a subroutine (needs special arrangement in the postprocessor) |
| <radius> | real | The radius of the hole circle |
| <attrib> | string | The attribute texts. If no text is given the argument is "" |
| <quality> | int | This is not activated yet it is default set to 3 (medium) |

($FLYCTRL <type>)

This instruction is used for turning on water and off abrasive and water on the fly. Not so many controllers can do this without making a stop in the movement.

| <type> | int | 1=Water On 2=Abrasive On 3=Abrasive Off 4=Water Off |
| --- | --- | --- |

($ITEXT <allpos><text> <angle><xlen> <ylen><font><fonttype>)

This instruction will order a Inkjet printer to print on the part/sheet.

| <allpos> | list | A list of three reals that describe the midpoint in X and Y of the text. |
| --- | --- | --- |
| <text> | list | This is the text that should be printed. The text is in a list to support inkjet printers that supports multilines. |
| <angle> | real | The text printing direction. |
| <xlen> | real | The total length of the text |
| <ylen> | real | Theoretical height of the text ("-" is different and "J"). |
| <font> | string | Font name |
| <font type> | int | 0=Single polyfont 1=Truetype 2=Postscript (not implemented) |

($LINE <allpos> <flag> <fs> <fe> <delta> <vector> < allrot> <length>)

| <allpos> | list | A list of three reals that describe the endpoint of the line |
| --- | --- | --- |
| <flag> | int | This is a bit coded value with following meaning: 1=Belong to a lead-in movement. 2=Belong to a lead-out movement. 4=Belong to a waterjet marking object. 8=Belong to circular piercing. 16=Belong to a loop segment. 32=If the line belongs to approach or retract movement. 64=If the line is generated by the $CLINE function. 128=If the line belongs to Special marking. |
| <fs> | Real | The speed at the beginning of the line |
| <fe> | Real | The speed at the end of the line |
| <delta> | Real | This is the tangent angle to next object (how much the direction will change at the intersection of next movement). Some machines need this to be able to slow down when the movement direction suddenly changes. The values are in radians and can be positive or negative. The value will be 0.0 for the object that goes to A or B. The value will be 1.5701 for the object that goes to C, D or E |
| <vector> | list | This is the theoretical jet stream direction. The vector length is the same as the actual material thickness in actual tilt of the jet. |
| <allrot> | list | This is a list of three angles. The first two are the primary and secondary motor angle for the kinematic. The last value is the angle for the rotation axis. |
| <length> | real | This is the length of the line. This is needed to calculate the speed in time ($ft) when using rotation axis. |

($MARKOFF <type> <allpos>)

$MARKOFF will be used to turn off Waterjet marking and in Special marking.

| <type> | int | The meaning of the type is following: 0=This is a 2D markoff and the <allpos> will not be used. 1=This is a 3D-5X markoff with a straight vertical lift (only Z will be used). 2=This is a 3D-5X markoff where the move out is not vertical (X,Y,Z will be used). |
| --- | --- | --- |
| <allpos> | list | This is the end of the lift up movement. X, Y, Z will be used in <type>2 and only Z in <type> 1. |

($MARKON <allpos> <attribute> <clen> <allrot> <special> <vector> <stardir> <maxslope> <3d-5x>)

The $MARKON instruction will be used to turn on waterjet or special marking.

| <allpos> | list | A list of three reals that describe the start point of the marking |
| --- | --- | --- |
| <attribute> | string | This is the attributes that are entered in the marking command. |
| <clen> | real | This is the total length of the marking geometry. |
| <allrot> | list | This is a list of three angles. The first two are the primary and secondary motor angle for the kinematic. The last value is the angle for the rotation axis. |
| <special> | bool | The value is T if it’s special marking else nil. |
| <vector> | list | This is the theoretical jet stream direction. |
| <startdir> | real | This is the direction of the first movement. The idea was to start with a distance from this <allpos> in the negative direction of <startdir> and open the jet on the fly. |
| <maxslope> | real | This is the maximum tilt of the jet for actual marking. |
| <3d-5x> | bool | The value is T if the information comes from 3D-5X else nil. |

($MEASURE <allpos> <index>)

This instruction will create measuring in advance positions.

| <allpos> | list | This is the X, Y and Z position of the measuring. |
| --- | --- | --- |
| <index> | int | This is a number that increase by 1 for each $measure. It start on 1. |

($MEASUREPOINT <allpos> <vector> <attrib>)

This instruction comes from the command "Measure points" in 3D-5X. This command is for automatic pre-location of parts on the CNC machine before the machine starts to work. Therefor all measurepoint will come in the beginning of the Preparation file.

| <allpos> | list | This is the raw X, Y, Z position near the model. |
| --- | --- | --- |
| <vector> | list | This is a direction vector that starts at the <allpos>. The measuring will start at the end of the vector. |
| <attrib> | string | This is a string of two numbers with a space between. Example: "0 4" The first is the type of measurepoint and the second is an index that increase by 1 for every new measurepoint. |

($PART <name><customer><date> <quantity> <disconnected> <bounding box> <posx> <posy> <rotation> <partnid> <pathid> <sortid> <3d-5x> <contains> <color> <attrib> <mirror>)

This instruction indicates on which part the actual machining is referred to. The same part can occur several times depending on kind of operation. IGEMS cnc can isolate on part and put together all instruction that belongs to that part even if the information is on different places. This can probably not be done on other CNC.

| <name> | string | The name of the part as entered when creating the parts. |
| --- | --- | --- |
| <customer> | string | The name of the customer as it was entered in create parts. |
| <date> | string | In following format "12/12/2023 13:22:53" |
| <quantity> | int | Number of parts as entered in create parts. |
| <disconnected> | bool | The value is T if the part (toolpath) is disconnected else nil. |
| <bounding box> | list | This is the list of 4 reals that describe lower left and upper right corner of a bounding box of the part. It seems that disconnected toolpath refer till World Coordinate system and connected parts to program origin. |
| <posx> | real | Part zero position in X. It seems that disconnected toolpath refer till World Coordinate system and connected parts to program origin. |
| <posy> | real | Part zero position in Y. It seems that disconnected toolpath refer till World Coordinate system and connected parts to program origin. |
| <rotation> | real | Part rotation angle |
| <partnid> | int | This variable is an index that stats on zero and increasing for each unique part definition on the sheet layout. If you have one part and have copy that to 100 copies, they will all have the same $partid. (Rev 2 only) |
| <pathid> | int | This variable is an index that starts on zero and increase by one for each unique toolpath on the drawing. (Rev 2 only) |
| <sortid> | int | This variable is an index starts on zero and increasing for all parts on the drawing. The order of the index is in that order they are selected in the Order command. All parts on the sheet layout a unique $sortid. (Rev 2 only). |
| <3d-5x> | Bool, Real | This variable is nil if it's a 2D part and it contains the thickness if it's a 3D-5X part. |
| <contains> | int | This is a bit coded value that describe the operations on actual part. 1=Cutting 2=Marking 4=Low pressure piercing 8=Drillings 16=Not used 32=Hole cutting 64=Cut with ramping 128=Cut with no ramping 256=Special markings 512=Inkjet marking |
| <color> | int | The color of the part |
| <attrib> |  |  |
| <mirrored> |  |  |

($PATHINFO <box> <flag>)

The box is a list of four reals that describes the size of a bounding box of the toolpath).

The <flag> is 1 if the toolpath (geometry) is disconnected. The flag value may be used for more purposes in the future.

($PRE-PIERCING <allpos> <ptype> <lead> <node> <vector> <flag> <plane><allrot> attrib)

This instruction will generate a pre-piercing.

| <allpos> | list | The pre-piercing position in X, Y and Z |
| --- | --- | --- |
| <ptype> | int | The piercing type can be: 2=Stationary piercing 3=Circular piercing |
| <lead> | string | The name of the lead definition |
| <node> | string | The name of the node. |
| <vector> | list | This is the theoretical jet stream direction. The vector length is the same as the actual material thickness in actual tilt of the jet. |
| <flag> | int | A bit value that means following: 1=Tilted piercing 16=The pre-piercing comes from 3D-5X |
| <plane> | list | This is a direction vector that are normal to the piercing plane. |
| <allrot> | list | This is a list of three angles. The first two are the primary and secondary motor angle for the kinematic. The last value is the angle for the rotation axis. |

($QUALITY <QUA>)

This instruction is used if the quality is change inside the toolpath. The quality information is also as an argument in the $CUTON instruction.

| <qua> | int | The qua argument means following: 1= X-Rough 2= Rough 3= Medium 4= Fine 5= X-Fine |
| --- | --- | --- |

($RAPID <allpos> <attrib> <reason><allrot> <tilted> <flag>)

The $rapid information moves the jet to a position in rapid mode (without cutting).

| <allpos> | list | A list of three reals that describe the target point of the rapid. |
| --- | --- | --- |
| <attrib> |  |  |
| <reason> | int | The value is not bit coded and means following: 0=Normal cut the Z-value should be taken from strategy 1=Extra rapid in 2D 2=Rapids between parts in 2D and 3D-5X mode 3=Rapid to sensoring position, for example if we need to take a new height when drilling 4=Rapids in 3D-5X use the Z-value 5=Similar as $reason 0 but the Z-value are already calculated and should be used. |
| <allrot> | list | This is a list of motor rotations for A B and C axis. |
| <tilted> | bool | Tilted is T if next piercing has a tilted piercing. On some kinematics it's important to know if the major axis should be rotated even if the piercing is straight (Flow XD, Apex 60 from Wardjet) |
| <flag> | int | This is a bit coded value that means following. 1: Tilted piercing 2: Tilted cutting (bevel mode is top, bottom or advanced) 4: Top bevel (Will later probably be removed) 8: Bottom bevel (will later probably be removed) 16: This is from 3D-5X 32: Directional bevel (will later probably be removed) 64: TAC can be used on this geometry. 128: Excepted from speed ramping. If force 3X cutting in 3D-5X then this value is not 1 and not 2. 256: If Custom rapid vector are added. |

($REFPOS <allpos> <allrot> <vector> <flag> <probdir>)

The refpos instruction can be used to find a cutting plan for a cut. The instruction is called from the toolpath command in 3D-5X. The refpoints is belonging to a toolpath but the measurepoints is for locating the whole part.

| <allpos> | list | This is the target point in X, Y, Z |
| --- | --- | --- |
| <allrot> | list | This is a list of motor angles för A, B and C |
| <vector> | list | This is a direction vector that starts at the <allpos>. The measuring will start at the end of the vector. The vector length describe the distance. |
| <flag> | int | The value is 1 on Static or Circular piercing The value is 4 on marking. In all other cases the value is 0 |
| <probedir> | bool | This is T if the option "Probe direction" is activated else nil. |

($REWIND <allrot>)

This instruction are used to rewind the major rotation on kinematics that needs this to avoid that the rotation reach the limit switch.

| <allrot> | list | This is a list of motor rotations for A B and C axis. |
| --- | --- | --- |

($ROTAX <vector> <minutes> <allrot> <speed> <len>)

This instruction is used when there is no movement in X, Y or Z. In this case the speed cannot defined as distance/minute. It must be specified as time.

| <vector> | list | This is the theoretical jet stream direction. The vector length is the same as the actual material thickness in actual tilt of the jet at the new position. |
| --- | --- | --- |
| <minutes> | real | This is the time in minutes for the movement. This means that if the movement takes minute the value will be 1.0. If it will take one second the value will be 0.016666 |
| <allrot> | list | <allrot> is a list of motor rotations for A B and C axis. |
| <speed> | real | This is a stupid value for what the speed should be it it was a XY movement. |
| <len> | real | This value is only relevant if there is a rotation axis involved in the rotax movement. |

($STOP <reason> <condition> <msg>)

The $STOP command can also be generated from other function in the postprocessing.

| <reason> | int | The reason value has following meaning: 0=Generated in case of TOOLSET and other automatic generated stops because of not complete postprocessor. 1=A stop generated in the Edit part command. 2=A stop generated in the Order command. 3=If the option "Stop after first cut" is activated. 4=If the option "Stop after first part" is activated. 5=If the option "Always pre-piercing is activated". This is mainly developed for 5-axis cutting, the STOP event will be used just before the cutting start. 6=If no cleaning or pressure release point is specified. 7=Stop generated by the Stop command in 3D-5X |
| --- | --- | --- |
| <condition> | bool | If the value is T then the variable $cond will be set |
| <msg> | string | A text message that describes the reason. |

($TEXT<allpos><text><angle><xlen><ylen><attrib><font><fonttype><special>)

This instruction can generate new RAPID, CUTON, LINE, ARC, CUTOFF if the postprocessor are using the function GENTEXT.

| <allpos> | list | A list of three reals that describe the midpoint in X and Y of the text. |
| --- | --- | --- |
| <text> | string | This is the text that should be printed |
| <angle> | real | The text printing direction. |
| <xlen> | real | The total length of the text |
| <ylen> | real | Theoretical height of the text ("-" is different and "J"). |
| <attrib> | string | This is a text that are entered as the attribute. |
| <font> | string | Font name |
| <font type> | int | 0=Single polyfont 1=Truetype 2=Postscript (not implemented) |
| <special> | bool | The argument is T if special marking should be used else nil |

($TOOLSET <tools> <distance> <align> <index>)

This instruction has information about what tool should be used and what distance it should be between.

| <tools> | int | The value is the summary of all tools that should be active: T1; T2; T3; T4; T5; T6; T7; T8; T9; T10; T11; T12. 1; 2; 4; 8; 16; 32; 64; 128; 256; 512; 1024; 2048. Examples: Use T1 and T3 the tools value will be (1+4)=5 Use T1 and T12 the tools value will be (1+2048)=2049 |
| --- | --- | --- |
| <distance> | real | The distance between tools. Example: We use T1 and T4 and the distance between these tools are 1500. The distance value will be 500 (between each tool). The value can only be 0 if the distance is not important (when Tool 1 only). |
| <align> | int | This value is for what tool alignment this drawing (job) is done. It is not possible to make a job for one alignment and then change machine to another alignment. If this is the case then we cancel the postprocessing. In that the operator must chose the EXPLODE option in the tool setup command. |
| <index> | int | This starts with 1 and increases by one for each new $toolset. |

($TPTOOLSET <tools> <tool-list> <allpos>)

This instruction is for a toolset inside a toolpath. It has another syntax then $toolset. Probably we will change that $toolset will have the $tptoolset argument in the future.

| <tools> | <int> | The value is the summary of all tools that should be active: T1; T2; T3; T4; T5; T6; T7; T8; T9; T10; T11; T12. 1; 2; 4; 8; 16; 32; 64; 128; 256; 512; 1024; 2048. Examples: Use T1 and T3 the tools value will be (1+4)=5 Use T1 and T12 the tools value will be (1+2048)=2049 |
| --- | --- | --- |
| <tool-list> | list | This is a list of reals with the distance between the tools. Example: '(500.0 500.0 500.0) It is now possible to have complete control of the tool distances. |
| <allpos> | list | A list of three reals that describe the first cutting position. It is a good solution to move to this point at the same time as the heads moves to correct tool distances. |

Important variables for Z-axis and sensoring

*ZAxisOption*

0=Fixed axis selected from the Method command

1=CNC controlled selected from the Method command

*ZAxisType*

0=Up/Down Automatic by Water On/Off.

1=Up/Down handled by M-codes.

2=Z is handled by Z-coordinates.

2=By height sensor selected from the Method command

*ZSensoringType*

0=Automatic handled by Water On/Off.

1=Sensor handled by M-codes.

2=Sensor handled by Z-coordinates.

*ZSensoringMode*

Controls what type of sensoring should be used.

0=Once in the beginning

1=Partwise

2=Every piercing

3=Stamping (for straight cuts)

4=Dragging (for tilted cuts)

5=Zone

6=Automatic choice

*FlatZoneRadius* This is used for mode Stamping and Dragging if the geometry at some places is outside the radius. If distance from previous sensoring is more than this value it will always be a sensoring.

*ZAutoDist* In case of *ZSensoringType* is 0 or 1. Then IGEMS will not write any instructions in the CNC file about lift height. This is controlled in the CNC file. This value is used to calculate a lifting time in reports.

3=By measuring in advance selected from the Method command.

*ZPiercingUse*

If IPD (Increased Piercing Distance) should be used or not. Value is 0 to 3 before Options and after options T or nil.

*ZPiercingDist*

The distance for IPD (Increased Piercing Distance). The value is not changed, matter of *ZPiercingUse*

*ZPiercingCNC*

T if the CNC accept increased piercing height when using CNC controlled Z axis else nil.

*ZPiercingSensor*

Is T if the CNC accept increased piercing height when using height sensor else nil.

$sensor

When using IGEMS cnc the *ZSensoringMode* should always be set to Auto, else will not the Auto setting in IGEMS CNC work. IGEMS cnc can override any setting.

Negative values Make a new Z-height probing. No matter of setting in IGEMS cnc

0=Use previous Z height probing for the Z-level.

1=Make a new Z-height probing.

2=Use Stamping sensoring (for straight cuts)

3=Use Dragging sensoring (for bevel cuts)

CNC files with subroutines

In the past it was very common to use CNC-files with subroutines. The major reason was limited memory in the CNC controller. Here is a simplified sample how to do this in IGEMS.

(SETQ *postinit* 2787)

(PRINC (STRCAT "Loading SUB-Program.IPF (Programming example)\n"))

(SETQ xbase 0 ybase 0)

(DEFUN header ()

(SETQ *Linenumberstart* 999)

(NTXT) ; To set the variable $nnext

(SETQ *ViewOutputFile* nil)

)

(DEFUN part ()

;(SETQ $pathid (STRCAT (ITOA $pathid) "-" (ANGTOS $rotate 3))) ; Use this is the machine not support rotation of parts = longer programs

(SETQ $pathid (ITOA $pathid)) ; Use this if machine supports rotation

(IF sublist ; Finish old part

(PROGN

(WRITE (STRCAT (NTXT) "END SUB"))

(SETQ sublist (REVERSE (CONS $ncount (REVERSE sublist))))

;(WRITE (TOSTR sublist))

(SETQ pathidlist (CONS sublist pathidlist))))

(IF (NOT (SETQ exist (ASSOC $pathid pathidlist)))

(PROGN

(SETQ sublist (LIST $pathid $nnext $basex $basey $rotate $partname))

(SETQ xbase $basex ybase $basey)

(WRITE (STRCAT "\nDEFINE SUB:" $pathid))

(SETQ writepath T))

(PROGN

(SETQ sublist (LIST (NTH 0 exist) (NTH 1 exist) $basex $basey $rotate $partname (NTH 6 exist)))

(SETQ xbase 0 ybase 0)

(SETQ writepath nil)

(SETQ pathidlist (CONS sublist pathidlist))

(SETQ sublist nil))

)

)

(DEFUN footer ()

(IF sublist ; Finish old part. Usefull if the last part is a new part.

(PROGN

(SETQ sublist (REVERSE (CONS $ncount (REVERSE sublist))))

(SETQ pathidlist (CONS sublist pathidlist))))

)

(DEFUN final ()

(SETQ fr (OPEN $filename "r"))

(WHILE (SETQ txt (READ-LINE fr))

(SETQ txtlist (CONS txt txtlist))

)

(CLOSE fr)

(SETQ fw (OPEN $filename "w"))

(PRINC (STRCAT (WRITE-LINE "%" fw) "\n"))

(PRINC (STRCAT (WRITE-LINE (STRCAT "HEADER") fw) "\n"))

(SETQ q 0)

(FOREACH s (REVERSE pathidlist)

(PRINC (STRCAT (WRITE-LINE (STRCAT "N" (ITOA (SETQ q (1+ q)))" G59 X" (RTS (NTH 2 s)) " Y" (RTS (NTH 3 s)) " A" (ANGTOS (NTH 4 s) 3) " CALL:" (CAR s) " FROM:N" (ITOA (CADR s)) " TO:N" (ITOA (NTH 6 s)) " NAME:" (NTH 5 s)) fw) "\n"))

(WRITE-LINE "" fw)

)

(PRINC (STRCAT (WRITE-LINE "M30" fw) "\n"))

(PRINC (STRCAT (WRITE-LINE  "" fw) "\n"))

(FOREACH s (REVERSE txtlist)

(PRINC (STRCAT (WRITE-LINE s fw) "\n"))

)

(CLOSE fw)

)

; Normal functions

(DEFUN tooldown ()

(IF writepath (WRITE (STRCAT (NTXT) "TOOLDOWN")))

)

(DEFUN rapid ()

(IF writepath (WRITE (STRCAT (NTXT) "G0 X" (RTS (- $x xbase)) " Y" (RTS (- $y ybase)))))

)

(DEFUN piercing ()

(IF writepath (WRITE (STRCAT (NTXT) "PIERCING")))

)

(DEFUN line ()

(IF writepath (WRITE (STRCAT (NTXT) "G1 X" (RTS (- $x xbase)) " Y" (RTS (- $y ybase)))))

)

(DEFUN arc ()

(IF writepath (WRITE (STRCAT (NTXT) (IF $ccw "G3" "G2") " X" (RTS (- $x xbase)) " Y" (RTS (- $y ybase)) " I" (RTS $irel) " J" (RTS $jrel))))

)

(DEFUN cutoff ()

(IF writepath (WRITE (STRCAT (NTXT) "JETOFF")))

)

(DEFUN toolup ()

(IF writepath (WRITE (STRCAT (NTXT) "TOOLUP")))

)

Simulation functions

Mainly used in IGEMS2.LSP

Information in the preparation file

All coordinates in the preparation file refer to the to the program origin.

($LINE '(206.5261 0.8473 10) 0 410.4860 410.4860 0.0 '(0 0 -10) 4.0805 0.0 '(0 0 0) 0.7)

($LINE '(206.1126 0.2824 10) 0 301.9687 301.9687 0.3208205 '(0 0 -10) 4.0805 0.0 '(0 0 0) 0.7)

The coordinates are the same no matter of what tools or how many tools are used. If the coordinates need to be modified (for example that tool nr 2 should be used instead of tool number 1) than this is added in ILISP2.lsp

$TOOLSET (<tools> <distance> <align> <index>) function call

- <Tools>=Bit coded value of the heads that should be used:
1= Use cutting head 1
2= Use cutting head 2
4= Use cutting head 3
8= Use cutting head 4
16= Use cutting head 5
32= Use cutting head 6
64= Use cutting head 7
128= Use cutting head 8

- <distance>= The distance between each active tools. The value can only be 0 if the distance is not important. Example when using one tool only.

- <align>=This value describes the tool alignment used by this $Toolset. This value may not be the same as the actual machine tool alignment. If this is different than IGEMS will show a message "Impossible tool alignment".
In this case operator must chose the EXPLODE option in the tool setup command.

- 0=Aligned with X, Left to Right 
1=Aligned with X, Right to Left
2=Aligned with Y, Bottom to Top
3=Aligned with Y, Top to Bottom

- This is a unique index number for each $toolset.

($TOOLSET <tools> <distance> <align> <index>) function

The function sets different variables that will be used by other SIM functions.

$tools that are a bit coded value for active tools.

$tooldist are a value that describe the equal distance between each tool.

$toollist is a list with the different distances between each tool. The length of this list is always the number of tools minus one.

PP_XMOD and PP_YMOD are adding its value to the $x, $y in the CNC file.

PP_XMODSIM and PP_YMODSIM are adding its value to the simulation function SIM-LINE, SIM-ARC, SIM-RAPID.

Here are some examples:

Example 1:

We are cut with tool number 2 only and the distance between each tool is set to 202mm. The Align is set to 0.

In the machine setting the settings are as below.

PP_XMOD=202 PP_XMODSIM=202 This will affect the coordinates in the CNC file and the simulation position.

Example 2:

In the example above the variables will be set as follows:

PP_XMOD=0 PP_XMODSIM=202. This will only affect the simulation.

Example 3:

When the option "Closest tool spacing" is activated then the distance between Tool 1 and Tool 2 will be 100 mm instead of 202 mm.

PP_XMOD=100 PP_XMODSIM=100 This will affect the coordinates in the CNC file and the simulation position.

Example 4:

When the option "Coordinate to first used tool" is activated then PP_XMOD is 0 and PP_XMODSIM is 100.

Example 5:

Some machines have the nice option to park the most external tools that are not used. If we still use Tool number two only then tool 1 can be parked. In that case the variables will be as follows.

PP_XMOD=0 and PP_XMODSIM=0

Since Tool 1 is parked then Tool 2 will be handled as Tool 1.

($SIM-LINE <totalx> <totally> <z> <fs> <fe> <c-rot> <v1> <v2> <v3> <len>)

<totalx> should be the summary of following: X value from the allpos+pp_xmodsim

<totally>should be the summary of following: Y value from the allpos + pp_ymodsim

<z> is z value from the allpos list.

<fs> Speed start value at the beginning of the movement

<fe> Speed end value at the end of the movement

<v1> <v2> <v3> is the incremental jet vector.

<len> is the length of the movement

It is also possible to set the variable *SIM-ADDX* and *SIM-ADDY* values in the postprocessor. (this can sometimes be needed for special application like the application to electrosila)

All allpos coordinates in the preparation file is always the same no matter of the machine settings in the "Cutting tools". Following variables may need to be modified:

(SIM-DRAW <draw>)

The argument <draw> to this function can be T or nil. It will handle if the 2D simulator will draw movement lines or not. This function was introduced in 2020.2

(SIM-DRAWLINE <x> <y> <z> <x2> <y2> <z2> <color> <pen size>)

Draw lines in the 3D-simulation window.

(SIM-DRAWRECTANGLE <x1> <y1> <x2> <y2> <color> <line-weight>)

Draw a rectangle in the 2D-simulation window.

Argument <x1> and <y1> describe the lower left corner and <x2> and <y2> describe the upper right corner of the rectangle. The values refer to the actual program origin. There is also two additional argument that controls the <color> and the <line-weight>.

(SIM-HEADUNIT <"name"> <visible> <dx> <dy> <dz>)

This function is only useful in IGEMS Revision 2. With this function you can make units on the 3D simulation invisible or visible. For example, if you don't have any height sensor then you can set the invisible flag to nil.

It is also possible to move a unit in X Y and Z. This can be used for position the drilling unit on correct position.

(SIM-LAXIS1 ProbeUse 1.5) ; Linear Axis 1

This was mainly developed for the special probing tool for SpaceX

(SIM3D-TILT <primary> <secondary> <rel-primary> <rel-secondary>)

Is used to rotate the kinematic in the simulation mode. And to calculate the time. The rotation will be done separately in the postprocessor TILT function.

(SIM3D-TOOLDIR <primary> <secondary>)

The function is used to rotate the kinematic together with another movement. LINE, ARC and RAPID.

(SIM-SANDWEIGHT)

Returns how much abrasive that will be used for actual job.

(SIM-RAXIS1 0 0.5) ; Rotation axis 1

This was mainly developed to handle a visualization of the TILTER height sensor

(SIM-MOVEMODEL <DX> <DY> <DZ>)

This function will move the simulation 3Dmodel (the part) in the distance given by the arguments.

(SSIM-RAPID pp_$x pp_$y pp_zmaster *SpeedRapid* *SpeedAcceleration* pp_crot)

(SSIM-LINE pp_$x pp_$y $fs $fe pp_crot)

(SSIM-ARC pp_$x pp_$y pp_$i pp_$j pp_ccw $fs $fe pp_crot)

The functions above are used in IGEMS.LSP

The variable *SIM-ADDX* and *SIM-ADDY* can be specified in the postprocessor. This will add a displacement to the motion in 3D-Simulation.

Real movements

It is possible to generate a visual RTCP toolpath in the 3D-simulation by activate "Show RTCP path".

If you want a file with XYZ information you can press "ALTGR+S in the simulation window. This will put the information in a hardcoded filename "c:/ncdata/rtcp.txt".

By making a simple LISP script as you can see below, the information can be inserted in 3D-5X.

Press "ALTGR + S" in simulation, run script below, start 3D-5X and press "CTRL-V".

Example code:

(SETQ outtext "")

(SETQ z 358.46)

(DEFUN wr (txt) (SETQ outtext (STRCAT outtext txt "\n")))

(SETQ fr (OPEN "c:/ncdata/rtcp.txt" "r"))

(SETQ sumlen 0.0)

(WHILE

(SETQ txt (READ-LINE fr))

(SETQ to (READ (STRCAT "(" txt ")")))

(IF (NOT from)

(SETQ from to)

(PROGN

(SETQ enlen (DISTANCE from to))

(WR (STRCAT (RTOS (CAR from) 2) " " (RTOS (CADR from) 2) " " (RTOS (+ (CADDR from) z) 2) 
       " " (RTOS (CAR to) 2) " " (RTOS (CADR to) 2) " " (RTOS (+ (CADDR to) z) 2) " 3.0 5"))

(SETQ from to)

(SETQ sumlen (+ sumlen enlen))

)

)

)

(IF fr (CLOSE fr))

(ALERT (STRCAT "Total movement length: " (TOSTR sumlen) " mm"))

(IF outtext (CLIP-SET outtext))

Variables

Following variables is coming from the existing toolpath. The values are depending how the operator have done the machining.

| *3D-5X* | The variable is T if some of the parts is done in 3D-5X. |
| --- | --- |
| *3D-5XStock* |  |
| *3D-5XZeroPoint* | This value is only precented if you have changed the zero point in *3D-5X*. |
| *3D-MaxZ* | This is the highest position of the 3D-5X part. The value is measured from the zero level (the table). If you move the parts up or down in Z. It will affect this value. |
| *3D-MaxZDiff* | This is the difference between the highest Z position of the model and the lowest Z position of the toolpath. |
| *3D-MaxZDiffCurves* | This is the difference in Z between all used top curves. It will not take care of for example standoffs. |
| *3D-MeasurePoints* |  |
| *3D-OnTopPlane* | This value is T if the complete toolpath is on top plane |
| *Date* | Actual date right now. Example: "2022-08-23" |
| *MaxFeed* | This is the highest speed that will be used on actual cut-order. Rapid movements are not included. |
| *MaxSlope* | This is the maximum needed cutting angle in actual cut-order. The value is in radians. |
| *Metric* | Can be T or nil. It is coming from the actual unit setting |
| *MinFeed* | This is the lowest speed that will be used on actual cut-order. |
| *PartsBox* | This is a list of 4 reals. The first two values describe the lower left corner and the two last values describes the upper right corners of all parts included in this job. Example:'(xll yll xur yur) The coordinates refer to the *ZeroPoint* |
| *SharedFolder* | This is the location of the shared folder. Example: "C:/ProgramData/IGEMS Software/IGEMS/22.0.0.0/Shared/" |
| *SheetBounds* | This is a list of lists of all sheets in the cut-order. Example: '((xll yll xur yur "name") (xll yll xur yur "name")) |
| *SheetName* | This is the name of the first sheet in the cut-order. Note! *SheetBounds* includes more information. |
| *SheetQuantity* | The Automatic nest can be instructed to produce identical sheets. This to let the CNC operator to skip load a new program for every new sheet. This value is the number of identical sheets. |
| *SheetX* | This is the size of the sheet in X. If there is no sheet the value is 0. If there are several sheets the value describes the size of the bounding box of all sheets in X |
| *SheetY* | This is the size of the sheet in Y. If there is no sheet the value is 0. If there are several sheets the value describes the size of the bounding box of all sheets in Y |
| *Time* | Actual time when postprocessing. Example: "13:30" |
| *TubeMode* nil) |  |
| *UseSixAxis* | T if the job is done using a rotation axis. This value can be T or nil. |
| *ZeroPoint* | This is a 3D-point describing from where all coordinates are referring to. Example: '(17.03 103.14 0.0). The value is refereeing to IGEMS world coordinate system. |

; ********MACHINE********

| *3D-6XspeedDecimals* 4) | When using a rotation unit the speed must be defined in time mode. This value controls how many decimals this information should be formatted to in the CNC file. The value is defined in the postprocessor section in the machine setting. |
| --- | --- |
| *3D-6XspeedUnits* 1) |  |
| *AddComments* 3) | If comment should be generated to the CNC file. Using the (MSG "Text") function. |
| *AddExtraMovement* nil) |  |
| *AddRadiusInSharpCorners* nil) |  |
| *AddRadiusOutside* nil) |  |
| *ArcVectors* nil) |  |
| *AWJAbrasiveCost* 1.0) |  |
| *AWJAbrasiveFlow* 350.0) |  |
| *AWJAbrasiveFlowCNC* 2) |  |
| *AWJAbrasiveQuality* "GMA Garnet 80") |  |
| *AWJAdditionalKerf* 0.0) |  |
| *AWJBleedOut* 1) |  |
| *AWJCostPerTool* 10.0) |  |
| *AWJDelayAbrasiveWaterOff* 1.2) |  |
| *AWJDelayAbrasiveWaterOn* 1.2) | This value will be copied to the variable $JetAbrasiveOn. The value controls how long time it takes between the signal that turns on the abrasive to the abrasive reach the material. |
| *AWJDelayAfterCutting* 0.9) | This value will be copied to the variable $JetValveOff. The value controls how long time it takes between the signal that turns off the water and to the water has been turned off. |
| *AWJDelayAfterPiercing* 1.2) |  |
| *AWJDelayExtra* 1.0) |  |
| *AWJDelayOpenValve* 0.1) | This value will be copied to the variable $JetValveOn. The value controls how long time it for the machine to open the water valve (before the water is coming). |
| *AWJDelayPreLoad* | This value will be copied to the variable $JetPreLoad. The value controls how long time the abrasive should be open before the water turns on when using low pressure piercing |
| *AWJDelayPressureDown* 4.0) |  |
| *AWJDelayPressureUp* 4.0) |  |
| *AWJDelayWaterAbrasiveOff* 1.0) |  |
| *AWJDelayWaterAbrasiveOn* 1.2) |  |
| *AWJFlyingAbrasive* T) |  |
| *AWJMachineCost* 100.0) |  |
| *AWJMarkingAbrasiveOff* 0.2) |  |
| *AWJMarkingAbrasiveOn* 0.1) |  |
| *AWJMarkingStartDelay* 0.3) |  |
| *AWJMaxAbrasive* 1000.0) |  |
| *AWJMaxPumpPressure* 4000.0) |  |
| *AWJMinAbrasive* 0.0) |  |
| *AWJMinPumpPressure* 100.0) |  |
| *AWJMixingTube* 0.762) |  |
| *AWJOrifice* 0.254) |  |
| *AWJPressure* 3600.0) |  |
| *AWJPressureLoss* 0.0) |  |
| *AWJPumpOnOfCNC* T) |  |
| *AWJPumpPressureCNC* 2) |  |
| *AWJPumpSize* 73.6) |  |
| *AWJSpeedFactor* 1.0) |  |
| *AWJStartPumpAuto* 3) |  |
| *AWJStopPumpAuto* 3) |  |
| *AWJToolDiameter* 0.762) |  |
| *AWJWaterAbrasiveSeparate* T) |  |
| *AxisDirection* 0) |  |
| *AxisSwapG2G3* nil) |  |
| *BarAligned* 0) |  |
| *BarLength* 1100.0) |  |
| *CentripitalAcceleration* 36000.0) |  |
| *CentripitalAccelerationEnabled* nil) |  |
| *CheckPartDistance* nil) |  |
| *ClampBottom* 0.0) |  |
| *ClampLeft* 0.0) |  |
| *ClampRight* 0.0) |  |
| *ClampTAC* nil) |  |
| *ClampTop* 0.0) |  |
| *ControllerAngleInterpolation* T) |  |
| *CoordinateToFirstTool* nil) |  |
| *CutPar* "[10-30-3600]") |  |
| *CutPar*:The selected cutting parameter |  |
| *CuttingLiftHeight* 50.0) |  |
| *CuttingLiftHeightParts* 50.0) |  |
| *CuttingLiftZ* 3) |  |
| *CuttingLiftZParts* 3) |  |
| *CuttingPneumatic* 0) |  |
| *CuttingSpeedToolCenter* T) |  |
| *DangerJetLength* 500.0) |  |
| *DangerZoneUse* nil) |  |
| *DecimalsAngle* 3) |  |
| *DecimalsDelay* 1) |  |
| *DecimalsMovement* 3) |  |
| *DecimalsSpeed* 2) |  |
| *DefaultTAC* T) |  |
| *DefaultZeroCorner* 0) |  |
| *DivideLongObjects* T) |  |
| *DrillAjustableOffsets* nil) |  |
| *DrillByType* 2) |  |
| *DrillDiameter* 4.0) |  |
| *DrillingExtraDelay* 2.0) |  |
| *DrillingOffsetByController* T) |  |
| *DrillingOnFirst* nil) |  |
| *DrillingOnTool* 1) |  |
| *DrillingSortPos* T) |  |
| *DrillingTools* T) |  |
| *DrillingXOffset* 100.0) |  |
| *DrillingYOffset* 0.0) |  |
| *DrillingZOffset* 0.0) |  |
| *DrillLiftHeight* 10.0) |  |
| *DrillLiftZ* 3) |  |
| *DrillPneumatic* 1) |  |
| *DrillPriority* 1) |  |
| *Encoding* 0) |  |
| *Expert* nil) |  |
| *FlattenAllArcs* nil) |  |
| *GoToParkPos* 0) |  |
| *GridDist* 30.0) |  |
| *GridHor* nil) |  |
| *Has5XKinematics* T) |  |
| *HasSixAxis* T) |  |
| *HeadDefaultTool* 1) |  |
| *HeadDefIniFile* "C:/ProgramData/IGEMS Software/IGEMS/22.0.0.0/Shared/Sim3d/TILTERDRILL/def.ini") |  |
| *HeadDir* 1) |  |
| *HeadKinematic* 0) |  |
| *HeadKinematicName* "TILTERDRILL") |  |
| *HeadMaxTilt* 45.0) |  |
| *HeadMinTilt* 0.05) |  |
| *HeadMinTiltSixAxis* 1.234) |  |
| *HeadPos* 1) |  |
| *HeadPrimaryAxisLimitations* T) |  |
| *HeadPrimaryMax* 720.0) |  |
| *HeadPrimaryMaxSpeed* 900.0) |  |
| *HeadPrimaryMin* -721.0) |  |
| *HeadPrimaryParkingAngle* 0.0) |  |
| *HeadPrimaryReversed* nil) |  |
| *HeadSecondaryMaxSpeed* 900.0) |  |
| *HeadSecondaryPrefer* 2) |  |
| *HeadSecondaryReversed* nil) |  |
| *HeadTiltAngle* 65.0) |  |
| *HeadType* 4) |  |
| *IgemsCNCSend* 1) |  |
| *IgnoreSheets* nil) |  |
| *IncrementalCircleCenter* nil) |  |
| *IncrementalPositions* nil) |  |
| *Inkjet* nil) |  |
| *InkjetOffsetByController* nil) |  |
| *InkjetOnTool* 1) |  |
| *InkjetSpeed* 4000.0) |  |
| *InkjetXOffset* 100.0) |  |
| *InkjetYOffset* 50.0) |  |
| *InkjetZOffset* 25.0) |  |
| *InternalToolCompensation* 2) |  |
| *LabelDirection* 0) |  |
| *LabelFont* "Lucida Console") |  |
| *LabelHeight* 10.0) |  |
| *LAG* 1) |  |
| *LargestSegmentAngle* 270.0) |  |
| *LaserMachineCost* 100.0) |  |
| *LaserSpeedOverride* 100) |  |
| *LaserToolDiameter* 0.2) |  |
| *LineNumberIncrement* 1) |  |
| *LineNumberStart* 10) |  |
| *LongestLine* 50.0) |  |
| *MachineCost* 100.0) |  |
| *MachineExtraScript* ";Script that is run in preparation file\n(SETQ simulate nil)") |  |
| *MachineName* "IGEMS CNC V4") |  |
| *MachineName*:The name of the machine |  |
| *MachineType* 0) |  |
| *MarkingByType* 2) |  |
| *MarkingFlyingStartStop* 0) |  |
| *MarkingInkjet* 0) |  |
| *MarkingLiftHeight* 5.0) |  |
| *MarkingLiftZ* 3) |  |
| *MarkingOffsetByController* nil) |  |
| *MarkingOnTool* 1) |  |
| *MarkingPneumatic* 0) |  |
| *MarkingPriority* 1) |  |
| *MarkingType* nil) |  |
| *MarkingXOffset* 0.0) |  |
| *MarkingYOffset* 0.0) |  |
| *MarkingZOffset* 0.0) |  |
| *MaxForbiddenTilt* 0.0) |  |
| *MaxRadius* 30000.0) |  |
| *MaxToolDiameter* 0.9) |  |
| *MeasureOffsetbyController* nil) |  |
| *MinChordLength* 0.05) |  |
| *MinLineLength* 0.01) |  |
| *MinRadius* 0.05) |  |
| *OutputBackupFolder* "E:/") |  |
| *OutputFolder* "C:/ToDo/") |  |
| *OutputMakeBackup* 1) |  |
| *OutputUnit* 0) |  |
| *OutputUseFixedFolder* T) |  |
| *OxyfuelHighSpeed* 100) |  |
| *OxyfuelLowSpeed* 100) |  |
| *OxyfuelMachineCost* 100.0) |  |
| *OxyfuelToolDiameter* 1.5) |  |
| *ParkPosX* 500.0) |  |
| *ParkPosY* 760.0) |  |
| *PiercingAlways* 0) |  |
| *PiercingByType* 2) |  |
| *PiercingLiftHeight* 20.0) |  |
| *PiercingLiftZ* 3) |  |
| *PiercingPneumatic* 0) |  |
| *PiercingPriority* 2) |  |
| *PlasmaHighSpeed* 100) |  |
| *PlasmaLowSpeed* 100) |  |
| *PlasmaMachineCost* 100.0) |  |
| *PlasmaToolDiameter* 1.5) |  |
| *PPFileByDrawing* 0) |  |
| *PPFileExtension* ".INC") |  |
| *PPFileLength* 1) |  |
| *PPFilePrefix* "") |  |
| *PPName* "IGEMS CNC V4") |  |
| *ProductionTimePerYear* 1800.0) |  |
| *ReportBackupFolder* "") |  |
| *ReportBackupFolderCost* "") |  |
| *ReportCostTemplate* "CostEstimate.tx") |  |
| *ReportCreateAutomatically* nil) |  |
| *ReportCreateAutomaticallyCost* nil) |  |
| *ReportCreatePDF* T) |  |
| *ReportCreatePDFCost* nil) |  |
| *ReportFolder* "C:/REPORT/") |  |
| *ReportFolderCost* "C:/REPORT_COST/") |  |
| *ReportIncludePartDetails* nil) |  |
| *ReportMakeBackup* nil) |  |
| *ReportMakeBackupCost* nil) |  |
| *ReportPostTemplate* "Postprocessor.tx") |  |
| *ReportPrint* 0) |  |
| *ReportPrintCost* 0) |  |
| *ReportSameAsNC* nil) |  |
| *ReportSameAsNCCost* nil) |  |
| *Revision* 2) |  |
| *RTCPExpectX* 0.0) |  |
| *RTCPExpectY* 0.0) |  |
| *RTCPMeasureX* 0.0) |  |
| *RTCPMeasureY* 0.0) |  |
| *RTCPUse* nil) |  |
| *SerialProduction* 0) |  |
| *ServicePosX* 0.0) |  |
| *ServicePosY* 0.0) |  |
| *ShowOptions* T) |  |
| *ShowSensoringInSimulation* T) |  |
| *SixAxisDirection* 0) |  |
| *SixAxisManualRotation* nil) |  |
| *SixAxisNoSpeedOpt* T) |  |
| *SixAxisOrigo* '(390.2 174.55 138)) |  |
| *SixAxisReverseRotation* nil) |  |
| *SizeX* 2000.0) |  |
| *SizeY* 1200.0) |  |
| *SkipAbrasiveFlowChanges* 0) |  |
| *SkipPressureChanges* 0) |  |
| *SkipPressureInit* 0) |  |
| *SkipToolInit* 0) |  |
| *SpeedAcceleration* 300.0) |  |
| *SpeedMaxCutting* 4000.0) |  |
| *SpeedMinCutting* 1.0) |  |
| *SpeedMode* 2); 0=No speed, 1=Fixed speed, 2=By CNC, 3=By machine |  |
| *SpeedRapid* 20000.0) |  |
| *StandoffDistance* 0.0) |  |
| *StandoffStartAngle* 0.0) |  |
| *StopAfterFirstCut* 0) |  |
| *StopAfterFirstPart* 0) |  |
| *SupressTrailingDots* T) |  |
| *SupressTrailingZeroes* T) |  |
| *TabX* T) |  |
| *TabXSize* 50.0) |  |
| *TabY* nil) |  |
| *TabYSize* 50.0) |  |
| *TAC* 1) |  |
| *TiltForwardMin* nil) |  |
| *ToolActiveByController* T) |  |
| *ToolClosestDistance* nil) |  |
| *ToolDiameter* 0.762) |  |
| *ToolDistanceByController* T) |  |
| *ToolFixDistance* 500.0) |  |
| *ToolMaxDistance* 1500.0) |  |
| *ToolMinDistance* 500.0) |  |
| *ToolMovingSpeed* 30000.0) |  |
| *ToolOutsideDiameter* 7.4) |  |
| *ToolPark* T) |  |
| *ToolQuantity* 1) |  |
| *Tube6AxisPrefere* 0) |  |
| *TubeAsLinear* nil) |  |
| *TubeDirection* 0) |  |
| *TubeLap* 360.0) |  |
| *TubeReverseRotation* nil) |  |
| *TubeRotateMode* 0) |  |
| *TubeVHeight* 60.0) |  |
| *UseClamps* nil) |  |
| *UseLineNumbers* nil) |  |
| *VacuumAssist* nil) |  |
| *VacuumOnDelay* 0.0) |  |
| *VectorizingAcc* 0.02) |  |
| *ViewOutputFile* 2) |  |
| *VOC* 1) |  |
| *WaterDelayAfterCutting* 0.0) |  |
| *WaterDelayAfterPiercing* 0.0) |  |
| *WaterDelayExtra* 0.0) |  |
| *WaterHighSpeed* 100) |  |
| *WaterLowSpeed* 100) |  |
| *WaterMachineCost* 100.0) |  |
| *WaterMaxPumpPressure* 6000.0) |  |
| *WaterPressure* 4100.0) |  |
| *WaterPumpOnOffCNC* nil) |  |
| *WaterToolDiameter* 0.2) |  |
| *ZAcceleration* 250.0) |  |
| *ZAutoDist* 20.0) |  |
| *ZAxisOption* 2) |  |
| *ZAxisType* 2) |  |
| *Zbase* 0) |  |
| *ZFixed* nil) |  |
| *ZMaxMovement* 300.0) |  |
| *ZMaxMovement5x* 100.0) |  |
| *ZMeasure* nil) |  |
| *ZMeasureDist* 0.0) |  |
| *ZMeasureDistBevel* 100.0) |  |
| *ZMeasureMaxPoints* 3) |  |
| *ZMeasureType* 0) |  |
| *ZMeasureXOffset* 0.0) |  |
| *ZMeasureYOffset* 0.0) |  |
| *ZMinMovement* 0.0) |  |
| *ZMovable* T) |  |
| *ZMovableQuantity* 1) |  |
| *ZPiercingCNC* T) |  |
| *ZPiercingDist* 4.321) |  |
| *ZPiercingSensor* T) |  |
| *ZPiercingUse* 1) |  |
| *ZPneumaticStroke* T) |  |
| *ZSensor* T) |  |
| *ZSensoringDistance* 50.0) |  |
| *ZSensoringMode* 6) |  |
| *ZSensoringTime* 1.0) |  |
| *ZSensorToZero* T) |  |
| *ZSensorType* 2) |  |
| *ZSpeed* 4000.0) |  |
| *ZStandOff* 10.0) |  |

MATERIAL

| *AWJMarkingAbrasiveFlow* 50.0) |  |
| --- | --- |
| *AWJMarkingAbrasiveFlowByMachine* T) |  |
| *AWJMarkingPressure* 500.0) |  |
| *AWJMarkingPressureByMachine* nil) |  |
| *AWJMarkingUseAbrasive* nil) |  |
| *AWJPointMarkingDelay* 1.0) |  |
| *Brittle* nil) |  |
| *CollarWidth* 5.0) |  |
| *CostDist* 0.0) |  |
| *CostPiercing* 0.0) |  |
| *Density* 2700.0) |  |
| *DisplayName* "Aluminium") |  |
| *DrillingRotationSpeed* 1999.0) |  |
| *DrillingSpeed* 500.0) |  |
| *DrillingStep* 2.0) |  |
| *LeadGap* 0.25) |  |
| *LinearPiercing* 0.75) |  |
| *Machinability* 220.0) |  |
| *MarkingSpeed* 1000.0) |  |
| *MaxLeadLength* 5.0) |  |
| *MinLeadLength* 3.0) |  |
| *Name* "Aluminium") |  |
| *Options* "") |  |
| *OvercutDistance* 3.0) |  |
| *PartDistance* 4.0) |  |
| *PiercingDiameter* 1.4) |  |
| *Sandwiched* nil) |  |
| *SpecialMarkingSpeed* 1000.0) |  |
| *Technology* "") |  |
| *Thickness* 5.0) |  |
| *User* "") |  |

; ********THICKNESS********

| *ArcFac* 1.0) |  |
| --- | --- |
| *AvailableAWJ* T) |  |
| *AvailableLaser* nil) |  |
| *AvailableOxyfuel* nil) |  |
| *AvailablePlasma* nil) |  |
| *AvailableWater* nil) |  |
| *AWJAutomatic* nil) |  |
| *AWJAutomaticPiercing* T) |  |
| *AWJCircularPiercing* 1.0) |  |
| *AWJCircularPiercingDiameter* 1.4) |  |
| *AWJCircularPiercingSpeed* 273.0) |  |
| *AWJCuttingAbrasiveFlow* 350.0) |  |
| *AWJCuttingAbrasiveFlowByMachine* T) |  |
| *AWJCuttingPressure* 3800.0) |  |
| *AWJCuttingPressureByMachine* T) |  |
| *AWJFixedSpeed* 1000.0) |  |
| *AWJFixedTAC* nil) |  |
| *AWJFixedTACAngle* 0.0) |  |
| *AWJLinearPiercingDistance* 0.75) |  |
| *AWJOvercutDistance* 3.0) |  |
| *AWJPiercingAbrasiveFlow* 300.0) |  |
| *AWJPiercingAbrasiveFlowByMachine* T) |  |
| *AWJPiercingPressure* 2000.0) |  |
| *AWJPiercingPressureByMachine* T) |  |
| *AWJStationaryPiercing* 3.0) |  |
| *AWJTACRough* -2.8624052) |  |
| *AWJTACRough1* 0.0) |  |
| *AWJTACRough2* -2.8624052) |  |
| *AWJTACXFINE* 2.8624052) |  |
| *AWJTACXFINE1* -1.0) |  |
| *AWJTACXFINE2* 2.8624052) |  |
| *FlatZoneRadius* 300.0) |  |
| *LaserCuttingSpeedFast* 1500.0) |  |
| *LaserCuttingSpeedMedium* 1000.0) |  |
| *LaserCuttingSpeedSlow* 800.0) |  |
| *LaserCuttingSpeedXFast* 2000.0) |  |
| *LaserCuttingSpeedXSlow* 500.0) |  |
| *LaserLeadinSpeed* 1000.0) |  |
| *LaserLinearPiercingDistance* 1.0) |  |
| *LaserPiercingDiameter* 2.0) |  |
| *LaserPiercingTime* 1.0) |  |
| *OxyfuelCuttingSpeed* 1000.0) |  |
| *OxyfuelLargeArea* 10.0) |  |
| *OxyfuelLeadinSpeed* 1000.0) |  |
| *OxyfuelPiercingDiameter* 5.0) |  |
| *OxyfuelPiercingTime* 1.0) |  |
| *OxyfuelReduceSpeedOnSmallInternals* nil) |  |
| *OxyfuelSmallArea* 3.0) |  |
| *OxyfuelSpeedReduction* 50) |  |
| *PlasmaAccDist* 0.0) |  |
| *PlasmaAmpere* 30.0) |  |
| *PlasmaCuttingSpeed* 1000.0) |  |
| *PlasmaDecDist* 0.0) |  |
| *PlasmaHighRadius* 30.0) |  |
| *PlasmaLeadinSpeed* 1000.0) |  |
| *PlasmaLoopRadius* 0.0) |  |
| *PlasmaLowRadius* 5.0) |  |
| *PlasmaMaxToolDiameter* 1.1) |  |
| *PlasmaPiercingDiameter* 5.0) |  |
| *PlasmaPiercingTime* 1.0) |  |
| *PlasmaSlowDownInCorners* nil) |  |
| *PlasmaSpeedReduction* 50) |  |
| *PlasmaSteps* 0) |  |
| *PlasmaTAC* 0.0) |  |
| *QualityCost* 6.0) |  |
| *QualityName* "Standard") |  |
| *SheetFileName* "") |  |
| *TACAdjustment* 0.0) |  |
| *WaterAccDist* 0.0) |  |
| *WaterCuttingSpeed* 2000.0) |  |
| *WaterDecDist* 0.0) |  |
| *WaterHighSpeed* 3000.0) |  |
| *WaterLowSpeed* 1000.0) |  |
| *WaterPiercingDiameter* 1.0) |  |
| *WaterPiercingOvercutDistance* 3.0) |  |
| *WaterPiercingTime* 1.0) |  |
| *WaterSlowDownInCorners* nil) |  |
| *WaterSteps* 0) |  |